67-57.040 Property Standards.

(1) All Units must meet the more stringent of the State or local building code requirements as applicable. All Units must contain at a minimum:

(a) Range and oven and Energy Star rated refrigerator;

(b) Washer and dryer hookups;

(c) Telephone hook-ups (minimum of 2);

(d) Cable or satellite TV hookups (minimum of 2);

(e) Hallways should be no less than 36'' wide;

(f) Thermostat and light switches should be no more than 48'' above floor level;

(g) Electrical outlets should be located no less than 18'' above floor level;

(h) Reinforced walls for future installation of horizontal grab bars in toilet, bathtub, and shower walls;

(i) Lever-action handles on all doors and faucets in Units and public areas;

(j) Toggle-type switches for lights and fans;

(k) Minimum of 32'' clear openings in all interior and bathroom doorways; and

(l) At least one accessible means of egress/ingress, which may be waived for manufactured housing units.

(2) All Units must include at a minimum the following exterior features:

(a) Minimal landscaping;

(b) Paved/surfaced driveway and walkway to one entry door; and

(c) Off-street parking.

(3) Site standards must include:

(a) Slope and terrain must be suitable for development. There shall be no problems with drainage, steep slopes or waterways on the site;

(b) Access to site must be compatible with existing traffic patterns and street capacity. Site shall not enter or exit onto a major high-volume traffic artery that would create problems for resident access or hazards to children;

(c) Site must be free from excessive traffic and noise, including that from cars, trains and airplanes. Members must submit a mitigation plan to the Corporation for Loan Servicing, detailing the proposed means and methods of risk reduction, if a Unit is (i) located within 3,000 feet of an active railroad line and/or subject to high railroad noise levels, (ii) located within 1,000 feet of a major high-volume traffic artery, freeway, or other highway would carry a daily volume of 25,000 motor vehicles or more, or (iii) within 500 feet if the highway that would carry more than 10,000 vehicles but fewer than 25,000. The site must not be located in an airport clear zone;

(d) The site must be free from significant industrial or agricultural hazards, including hazardous substances; toxic chemicals; gas, oil and chemical storage tanks and facilities; runoff; spills; odors; noise; and airborne particulates; and

(e) The neighborhood shall be primarily residential and adjacent areas should be compatible with residential development.

(4) Manufactured homes must meet or exceed the following standards, which are adopted and incorporated herein by reference and are available at http://www.floridahousing.org/Home/Developers/HomeownershipPrograms/HOP, unless superseded by state or local building codes:

(a) Meet the Manufactured Home Construction and Safety Standards (MHCSS) pursuant to 24 CFR XX Part 3280;

(b) Be sited on a permanent foundation in accordance with the Permanent Foundation Guide for Manufactured Housing (HUD-007487);

(c) Be built to the increased standards in the HOP Manufactured Housing Standards:

(d) Be considered real property by the local property appraiser and the land and home must be taxed together;

(e) Be connected to permanent utility hook-ups; and

(f) All purchase contracts must be turn-key.

Specific Authority 420.507(12), (23) FS. Law Implemented 420.507(18), 420.5089 FS. History–New 6-26-06.