62-45.180 Mixing Zones.

(1) The Department may propose zones of mixing or approve zones of mixing proposed by the applicant for maintenance dredging and maintenance dredged material disposal sites permitted pursuant to this chapter.

(2) The Department may allow the water quality adjacent to a maintenance dredging operation or a return discharge point from a maintenance dredged material disposal site to be temporarily degraded to the extent that only the minimum conditions described in subsection 62-3.051(1), F.A.C., apply within a limited, defined region known as a zone of mixing. However, no zone of mixing or combination of zones of mixing shall be allowed to impair any of the designated uses of the receiving body of water.

(3) A zone of mixing shall be determined based on consideration of the following:

(a) The presence of grass beds, live reefs, oyster and clam beds, or other productive marine and estuarine habitats;

(b) Physical and chemical characteristics of the materials to be dredged;

(c) The anticipated frequency of maintenance dredging or discharge from the disposal site(s);

(d) The expected duration of the dredging operation or the return flow from the disposal site; and

(e) The ambient water quality including any possible synergistic effects of resuspension of the material being dredged or return flow from the disposal site(s).

(4) A zone of mixing shall be time limited based on the following:

(a) The anticipated settling time of material resuspended during the dredging operation in the channels, turning basins, or harbor berths; or

(b) The anticipated mixing or settling time of material or pollutants discharged from the disposal site(s).

(5) Zones of mixing shall be proposed by the applicant for dredging and disposal operations according to the following:

(a) Zones of mixing for the dredging operation shall be located at a specified distance from the cutter head, suction head, clam bucket, or other dredging tool based on hydrographic characteristics of the channel, turning basin or harbor berth, the presence of natural resources listed in paragraph (3)(a) above, and the length of settling time necessary for resuspended material. The Department recognizes that this zone of mixing will move with the dredge as it moves.

(b) A zone of mixing for the disposal operation shall extend a specified distance from the weirs or other points of discharge based on the hydrographic characteristics of the receiving waters, the presence of natural resources listed in paragraph (3)(a) above, the estimated rates of effluent discharge, and estimated settling time for removing suspended material.

Specific Authority 403.061(7), 403.805(1) FS. Law Implemented 403.061(11), 403.816 FS. History–New 3-28-84, Formerly 17-45.18, 17-45.180.