62-242.800 Low Emissions Adjustment.

(1) A low emissions adjustment is required pursuant to the waiver provisions of Section 325.209, F.S., and Department of Highway Safety and Motor Vehicles Rule 15C-6.002, F.A.C.

(2) The low emissions adjustment for a gasoline powered motor vehicle is designed to adjust and repair or replace common tune-up components of the engine in an effort to restore the vehicle to the manufacturer’s specifications and minimize the amount of exhaust emissions. The low emissions adjustment shall consist of performing the following procedures, when applicable.

(a) Inspect for dirty or plugged air cleaner or anything that may restrict the air intake system. Replace filter element. Clean and repair as required.

(b) Check vacuum hoses for cracks, leaks or blockage and repair or replace as required.

(c) Inspect, adjust or replace common ignition components such as points, condenser, rotor, distributor cap, and electronic ignition components. Inspect and clean or replace spark plugs, and repair or replace spark plug wires as required.

(d) Check the dwell angle and timing according to manufacturer’s specifications, +- 2 degrees maximum.

(e) Inspect and adjust choke fast-idle speed setting. Inspect choke linkage and equivalent components, such as vacuum “pull-offs,” as applicable, for proper operation per manufacturer’s procedures and specifications.

(f) With the air filter installed, check the idle speed and fuel-to-air mixture according to the manufacturer’s specifications. Idle should be set to the manufacturer’s specification, +- 50 rpm.

(g) Check for manufacturer’s recommended positive crankcase ventilation valve and its correct operation, verifying free-flow through the positive crankcase ventilation system passages and hoses. Repair or replace system parts as required.

(h) Check EGR valve for proper installation and functional operation.

(i) If applicable, check the onboard diagnostics system for diagnostic codes and repair according to the manufacturer’s specifications.

(3) The low emissions adjustment for a diesel-powered motor vehicle shall consist of performing the following procedures:

(a) Inspect for dirty or plugged air cleaner or anything that may restrict the air intake system. Replace filter element. Clean and repair as required.

(b) Check fuel injection system timing according to manufacturer’s specification and adjust as required.

(c) Check for fuel injector fouling or mismatch and repair or replace as required.

(d) If tailpipe exhaust smoke continues to be visually detectable, check fuel pump and air-fuel ratio control according to manufacturer’s specifications and adjust or replace as required.

Specific Authority 325.206, 325.209, 403.061 FS. Law Implemented 325.206, 325.209, 403.021, 403.061 FS. History–New 2-20-89, Amended 3-21-91, 2-2-93, Formerly 17-242.800.