- 62-296.700 Reasonably Available Control Technology (RACT) Particulate Matter.

(1) Applicability.

(a) Any existing emissions unit that emits particulate matter and is located in a particulate matter air quality maintenance area or in the area of influence of such an air quality maintenance area, except an emissions unit which has received a determination of Best Available Control Technology pursuant to Rule 17-2.630 (repealed) or 62-296.330 (repealed), F.A.C., or received a permit in connection with Rule 17-2.500 (transferred), 17-2.510 (transferred), 62-212.400 or 62-212.500, F.A.C., shall limit the emission of particulate matter through the application of Reasonably Available Control Technology (RACT) as specified in Rules 62-296.701 through 62-296.712, F.A.C., or Rules 62-296.401 through 62-296.415, F.A.C.

(b) [Reserved].

(2) Exemptions. The following facilities and emissions units which are located within a particulate matter air quality maintenance area or area of influence are exempt from the provisions of this rule:

(a) Any facility with total maximum allowable emissions of particulate matter of less than 15 tons per year and 5 pounds per hour.

(b) Any facility whose owner or operator demonstrates to the Department that the impact within the designated air quality maintenance area of the total maximum allowable particulate matter emissions from such facility will not exceed 1 ug/m3 , annual average, and 5 ug/m3 , 24-hour average.

(c) Any emissions unit which has total allowable emissions of particulate matter of less than one ton per year.

(d) Any emissions unit of unconfined particulate matter which is located more than five kilometers outside the boundary of a particulate matter air quality maintenance area.

(e) Any emissions unit of unconfined particulate matter from open stockpiling of materials, vehicular traffic and other emissions from roads and plant grounds, or construction activities.

(f) Any moveable drop transfer point where the discharge point and receiving point of the materials being handled must be moved in relationship to each other, either continuously or intermittently, such that enclosure of the drop transfer point with a device to control emissions of particulate matter is not practicable.

(3) Specific RACT Emission Limiting Standards for Stationary Emissions Units. The specific particulate matter emission limiting standards set forth in Rules 62-296.401 through 62-296.414, F.A.C., have been found to represent the application of RACT for each emissions unit category listed in those rules, except for those emissions unit categories listed in Rules 62-296.701 through 62-296.712, F.A.C. For those emissions unit categories the particulate matter emission standards in Rules 62-296.701 through 62-296.712, F.A.C., have been found to represent the application of RACT.

(4) Maximum Allowable Emission Rates.

(a) Emissions Unit Data. The new or revised operating permit for each emissions unit subject to the provisions of this rule shall specify:

1. The maximum heat input rate, charging rate, production rate, through-put rate, and/or materials handling rate, as appropriate. The maximum heat input rate, charging rate, production rate, through-put rate, or materials handling rate shall be the maximum rate at which the emissions unit is capable of being operated on a continuous basis.

2. The maximum dry standard volumetric flow rate for each emission point, when applicable:

The maximum dry standard volumetric flow rate for each emissions unit or component emissions unit operation shall be the minimum dry standard volumetric flow rate that is necessary to safely and properly vent or operate the emissions unit when it is operated at its maximum continuous operating rate.

3. The control device through which each gas stream is vented and the emission point from which each gas stream is discharged to the open air,

4. The height above ground, exit diameter, UTM coordinates, and nature of each emission point through which particulate is or may be vented,

5. The exit gas temperature, actual volumetric flow rate and moisture content of each particulate bearing gas stream that is or may be vented to the open air,

6. Pertinent operating or control equipment parameters, such as pH of scrubber solution, pressure drop in scrubber, pressure on spray nozzle, etc., when such information is needed to confirm the control device is operating normally,

7. The permitted operating schedule (hrs./day, days/wk., wk./yr.).

(b) Maximum Emission Rates. The new or revised operating permit for each emissions unit shall specify the maximum allowable emission rate for each emissions unit or group of commonly vented emissions units sources in accordance with the following provisions:

1. The maximum allowable emission rate expressed in lbs/hr, lbs/day and tons/yr (or other equivalent units) shall be determined for each emissions unit (for example, each drop transfer point, screening operation, kiln, or dryer) by applying the appropriate emission limitation contained in Rules 62-296.401 through 62-296.414, F.A.C., or Rules 62-296.701 through 62-296.712, F.A.C., to the maximum applicable emissions unit operation rate or dry standard volumetric flow rate and the permitted operating schedule as specified in the operating permit pursuant to the provisions of paragraph 62-296.700(4)(a), F.A.C.

2. If several emissions units are vented through a common control device or emission point, the maximum allowable emission rate for the common emission point shall be the sum of the individual maximum allowable emission rates for each emissions unit vented by the emission point.

3. The owner or operator of an emissions unit or a group of emissions units that is subject to an emission limitation set forth in Rules 62-296.701 through 62-296.712, F.A.C., and that is vented through more than one emission point, shall, subject to the approval of the Department, prorate the total allowable emission for such emissions unit among all emission points that vent the affected emissions unit such that a specific maximum allowable emission rate is assigned to each emission point. The operating permits for emissions units shall be revised in accordance with subsection 62-296.700(4), F.A.C., to reflect the maximum allowable emission rates for each emission point.

4. The operating permit shall specify whether compliance shall be determined by measuring the emissions vented from each individual emissions unit or by measuring the emissions from the common emission point. In determining whether compliance shall be determined for each emissions unit individually or for a group of commonly vented emissions units at the common emission point, the department shall consider the following factors:

a. If all emissions units that are vented through a common emission point are subject to the same type of emission limiting standard (i.e., grains per dry standard cubic foot (gr/dscf)) and are all part of the same system of unit operations such that when one emissions unit is in operation the other emissions units will also normally be in operation, the Department may specify that compliance be determined at the common point of emission.

b. If the various emissions units that are vented through a common emission point are parts of different operating systems or are subject to different types of emission limiting standards (i.e., gr/dscf, lbs/ton of feed, lbs/MMBTU, percent opacity, etc.). The Department may specify that compliance with the various emission standards be determined separately for each emissions unit operation.

(5) Circumvention. No owner or operator of an emissions unit subject to the requirements of Rules 62-296.401 through 62-296.414 or Rules 62-296.701 through 62-296.712, F.A.C., establishing maximum concentrations of emissions of particulate matter in the exhaust gas from the emissions unit shall circumvent the provisions of an applicable emission limitation by increasing the volume of gas in any exhaust or group of exhausts for the purpose of reducing the stack gas concentration. This includes allowing dilution air to enter the system through leaks, open vents, or similar means.

(6) Operation and Maintenance Plan. The new or revised operating permit for each emissions unit subject to the provisions of this rule shall specify an operation and maintenance plan for the particulate control devices, the collection systems and the processing systems.

(a) Air Pollution Control Devices and Collection Systems. The plan shall include a schedule for the maintenance and inspection of each control device and collection system and a schedule for recording performance parameters of the control devices, collection systems and auxiliary equipment. Records of inspections, maintenance and performance data of control devices and auxiliary equipment shall be retained by the emissions unit for a minimum of two years and shall be made available to the Department upon request. The performance parameters shall include such physical, chemical or electrical characteristics as are applicable to the particular emissions unit and which are indicators of the condition, operating rates and efficiencies. Such parameters may include, but shall not be limited to, the following indicators for:

1. Scrubbers

Pressure drop, total

Pressure drop, scrubber

Pressure drop, mist eliminator

Liquor feed rate

Liquor feed composition and pH

Liquor feed solids and undissolved solids contents

Water makeup rate

Fan(s) current at rated voltage

Pump(s) current at rated voltage

Gas flowrate

Gas temperatures, inlet and outlet (minimum)

2. Baghouses

Bag pressure drop

Gas flowrate: direct method preferred; indirect method acceptable

Air to cloth ratio

Bag Weave

Bag material

Gas temperature, inlet and outlet

Bag cleaning conditions:

Pulse: Air pressure

Shake: shaker motor current

Reverse: reverse air fan current

Bag cleaning cycle:

Shake: duration, frequency, and delay periods

Reverse: duration, frequency, and delay periods

3. Electrostatic Precipitators

The following information shall be recorded unless otherwise agreed to by the Department:

Primary voltage

Primary current

Secondary current

Spark rate

Additional information, including but not limited to the following, may be required to be included as descriptive information in the operation and maintenance plan, but shall not be required to be recorded routinely unless the Department determines that a precipitator’s ability to achieve compliance with applicable emission limiting standards is questionable:

Secondary voltage

Rapper frequency, plate

Rapper Vibrator frequency, wire

Rapper duration, plate

Rapper Vibrator duration, wire

Gas temperature, inlet and outlet

Estimated gas flowrate

Static pressure

(b) Control Equipment Data. The Operation and Maintenance plan shall include identification of control device(s) for each emissions unit subject to provisions of this rule including but not limited to the following appropriate design specifications and other descriptive data:

1. Manufacturer,

2. Model name and number,

3. Type: scrubber, baghouse, electrostatic precipitator, dry scrubber, etc.,

4. Design flow rate (liquid and/or gas),

5. For EPS's: primary and secondary voltage and current,

6. Efficiency rating at design capacity,

7. Pressure drop,

8. Liquid to gas ratio,

9. Scrubbing liquor composition.

(c) Processing or Materials Handling Systems.

1. Appropriate parameters of processing or materials handling systems provide a measure of the rate of operations. The operation and maintenance plan shall include performance parameters which indicate the rate of operation, process weight through-put, the fuel or other energy source, the materials being processed or other physical or chemical characteristics, as applicable. Such parameters may include, but shall not be limited to the following:

a. Weight per unit time of raw materials input,

b. Process temperature or pressure,

c. Fuel or fuel mixture,

d. Chemical or physical data on product or raw materials,

e. Air to fuel ratio or percent excess oxygen,

f. Electrical power use rate by auxiliary equipment.

2. The plan shall contain inspection and maintenance schedules including periodic assessments of the condition of manholes, ducting, breaching, hoods, conveyor and elevator housing, loading sheds and other equipment, and a schedule for recording of performance parameter data.

(d) Fossil Fuel Steam Generators. The operation and maintenance plan for fossil fuel steam generators may include, but shall not be limited to, the following:

Steam flow

Fuel type (e.g., gas, oil, coal, or mixtures thereof)

Consumption rate for type(s) of fuel(s) burned

Fuel oil temperature (if applicable)

(e) Records of inspection, maintenance and performance parameter data shall be retained for a minimum of two years and shall be made available to the Department upon request.

Rulemaking Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. Formerly 17-2.650(2)(a)-(g), 17-296.700, Amended 11-23-94, 1-1-96.