62-296.704 Asphalt Concrete Plants.

(1) Applicability. The emission limitations set forth in Rule 62-296.704, F.A.C., shall apply to any facility used to manufacture asphalt concrete by heating and drying aggregate and mixing with asphalt cements, excluding unloading and storage of raw materials.

(2) Emission Limitations. No owner or operator of an asphalt concrete plant shall cause, permit, or allow the emission of particulate matter in excess of 0.06 gr/dscf, or visible emissions the density of which is greater than 20 percent opacity.

(3) Test Methods and Procedures. All emissions tests performed pursuant to the requirements of this rule shall comply with the following requirements.

(a) The test method for visible emissions shall be EPA Method 9, incorporated and adopted by reference in Chapter 62-297, F.A.C.

(b) The test method for particulate matter emissions shall be EPA Method 5 or EPA Method 5A, incorporated and adopted by reference in Chapter 62-297, F.A.C. The minimum sample volume shall be 30 dry standard cubic feet.

(c) Test procedures shall meet all applicable requirements of Chapter 62-297, F.A.C.

Specific Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061, 403.087 FS. History–Formerly 17-2.650(2)(c)4., 17-296.704, Amended 11-23-94, 1-1-96.