62-303.320 Exceedances of Aquatic Life-Based Water Quality Criteria.

(1) Water segments shall be placed on the planning list if, using objective and credible data, as defined by the requirements specified in this section, the number of samples that do not meet an applicable water quality criterion due to pollutant discharges is greater than or equal to the number listed in Table 1 for the given sample size. For sample sizes up to 500, waters are placed on the planning list when 10% or more of the samples do not meet the applicable criteria with a minimum of an 80% confidence level using a binomial distribution. For sample sizes greater than 500, the Department shall calculate the number of samples not meeting the criterion that are needed to list the waterbody with an 80% confidence level for the given sample size using the binomial distribution.

| Table 1: Planning List |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Minimum number of samples not meeting an applicable water quality criterion needed to put a water on the planning list with at least 80% confidence. |  |  |  |  |  |  |  |
| Sample sizes |  | Are listed if they have at least this # of samples that do not meet a criterion |  |  | Sample sizes |  | Are listed if they have at least this # of samples that do not meet a criterion |
| From | To |  |  |  | From | To |  |
| 10 | 15 | 3 |  |  | 256 | 264 | 31 |
| 16 | 23 | 4 |  |  | 265 | 273 | 32 |
| 24 | 31 | 5 |  |  | 274 | 282 | 33 |
| 32 | 39 | 6 |  |  | 283 | 292 | 34 |
| 40 | 47 | 7 |  |  | 293 | 301 | 35 |
| 48 | 56 | 8 |  |  | 302 | 310 | 36 |
| 57 | 65 | 9 |  |  | 311 | 320 | 37 |
| 66 | 73 | 10 |  |  | 321 | 329 | 38 |
| 74 | 82 | 11 |  |  | 330 | 338 | 39 |
| 83 | 91 | 12 |  |  | 339 | 348 | 40 |
| 92 | 100 | 13 |  |  | 349 | 357 | 41 |
| 101 | 109 | 14 |  |  | 358 | 367 | 42 |
| 110 | 118 | 15 |  |  | 368 | 376 | 43 |
| 119 | 126 | 16 |  |  | 377 | 385 | 44 |
| 127 | 136 | 17 |  |  | 386 | 395 | 45 |
| 137 | 145 | 18 |  |  | 396 | 404 | 46 |
| 146 | 154 | 19 |  |  | 405 | 414 | 47 |
| 155 | 163 | 20 |  |  | 415 | 423 | 48 |
| 164 | 172 | 21 |  |  | 424 | 432 | 49 |
| 173 | 181 | 22 |  |  | 433 | 442 | 50 |
| 182 | 190 | 23 |  |  | 443 | 451 | 51 |
| 191 | 199 | 24 |  |  | 452 | 461 | 52 |
| 200 | 208 | 25 |  |  | 462 | 470 | 53 |
| 209 | 218 | 26 |  |  | 471 | 480 | 54 |
| 219 | 227 | 27 |  |  | 481 | 489 | 55 |
| 228 | 236 | 28 |  |  | 490 | 499 | 56 |
| 237 | 245 | 29 |  |  | 500 | 500 | 57 |
| 246 | 255 | 30 |  |  |  |  |  |

(2) The Department’s Florida Storage and Retrieval (FLASTORET) database, or its successors, shall be the primary source of data used for determining whether samples do not meet water quality criteria. As required by subsection 62-40.540(3), F.A.C., the Department, other state agencies, the Water Management Districts, and local governments collecting surface water quality data in Florida shall enter the data into FLASTORET within one year of collection. Other sampling entities that want to ensure their data will be considered for evaluation should ensure their data are entered into FLASTORET. The Department shall consider data submitted to the Department from other sources and databases if the data meet the sufficiency and data quality requirements of this section.

(3) Unless information presented to the Department demonstrates otherwise, data older than ten years at the time the water segment is proposed for listing on the planning list are not representative of current conditions and shall not be used to develop planning lists, except to evaluate historical trends in chlorophyll a or TSIs. Any determinations by the Department to use data older than 10 years shall be documented, and the documentation shall include the basis for the decision that the data are representative of current conditions. Further, more recent data shall take precedence over older data if:

(a) The newer data indicate a change in water quality and this change is related to changes in pollutant loading to the watershed or improved pollution control mechanisms in the watershed contributing to the assessed area, or

(b) The Department determines that the older data do not meet the data quality requirements of this section or are no longer representative of the water quality of the segment. The Department shall note for the record that the older data were excluded and provide details about why the older data were excluded.

(4) To place a water segment on the planning list using Table 1, a water segment shall have a minimum of ten samples for the ten-year period, with at least five temporally independent samples. To be treated as a temporally independent sample, samples shall be at least one week apart, regardless whether the samples are collected at different locations within the segment.

(a) Samples collected at the same location less than four days apart shall be considered as one sample, with the median value used to represent the sampling period. However, if any of the individual dissolved oxygen (DO) values are less than 1.5 mg/l or, for other parameters, individual values exceed acutely toxic levels as listed in Table 2, then the worst-case value shall be used to represent the sampling period. The worst-case value is the minimum value for DO, both the minimum and maximum for pH, or the maximum value for other parameters. However, when DO data are available from diel or depth profile studies, the lower tenth percentile value shall be used to represent worst-case conditions for comparison against the minimum criteria.

(b) Samples collected within 200 meters of each other will be considered the same station or location, unless there is a tributary, an outfall, or significant change in the hydrography of the water.

(c) Samples collected from different stations within a water segment shall be assessed as separate samples even if collected at the same time.

(d) In making the determination to list water segments, the Department shall consider ambient background conditions, including seasonal and other natural variations.

| Table 2. Acutely Toxic Levels for Parameters with Aquatic Life-Based Criteria |  |  |  |
| --- | --- | --- | --- |
| Parameter | Units | Freshwater Value | Marine Value |
| Aldrin | ug/L | 3 | 1.3 |
| Aluminum | ug/L | 750 | N/A |
| Arsenic | ug/L | 340 | 69 |
| Cadmium | ug/L | exp((1.0166*(lnH))-3.924) | 40 |
| Chlordane | ug/L | 2.4 | 0.09 |
| Chlorine | ug/L | 19 | 13 |
| Chromium III | ug/L | exp((0.8190(lnH))+3.7256) | N/A |
| Chromium VI | ug/L | 16 | 1100 |
| Copper | ug/L | exp((0.9422*(lnH))-1.700) | 5.8 |
| Cyanide | ug/L | 22 | 1 |
| DDT | ug/L | 1.1 | 0.13 |
| Dieldrin | ug/L | 0.24 | 0.71 |
| Endosulfan | ug/L | 0.22 | 0.034 |
| Endrin | ug/L | 0.086 | 0.037 |
| Heptachlor | ug/L | 0.52 | 0.053 |
| Lead | ug/L | exp((1.273(lnH))-1.460) | 221 |
| Lindane | ug/L | 0.95 | 0.16 |
| Nickel | ug/L | exp((0.8460(lnH))+2.255) | 75 |
| Parathion | ug/L | 0.065 | N/A |
| Pentachlorophenol | ug/L | exp(1.005(pH)-4.869) | 13 |
| Selenium | ug/L | N/A | 290 |
| Silver | ug/L | exp((1.72(lnH))-6.59) | 2.2 |
| Toxaphene | ug/L | 0.73 | 0.21 |
| Zinc | ug/L | exp((0.8473(lnH))+0.884) | 95 |

(5) For predominantly marine waters, the Department shall evaluate both the minimum allowable DO of 4.0 mg/l and the daily average DO criterion of 5.0 mg/l using Table 1. At least four temporally independent samples are required to calculate the daily average for any given day.  For DO, temporally independent shall be defined as at least 4 hours apart. If there are sufficient data to determine daily averages for more than one day within a four-day period, the Department shall use the median value of the daily averages to represent the sampling period.

(6) Notwithstanding the requirements of subsection (4), water segments shall be included on the planning list if:

(a) There are less than ten samples for the segment, but there are three or more temporally independent samples that do not meet an applicable water quality criterion, or

(b) More than one samples do not meet an acute toxicity-based water quality criterion listed in Rule 62-302.530, F.A.C., or a water quality criterion for a synthetic organic compound or synthetic pesticide in any three year period.

(7) Values that exceed possible physical or chemical measurement constraints (pH greater than 14, for example) or that represent data transcription errors shall be excluded from the assessment. Outliers identified through statistical procedures shall be evaluated to determine whether they represent valid measures of water quality. If the Department determines that they are not valid, they shall be excluded from the assessment. However, the Department shall note for the record that the data were excluded and explain why they were excluded.

(8) The Department shall consider all readily available water quality data collected and analyzed in accordance with Chapter 62-160, F.A.C. If requested, the sampling agency must provide to the Department all of the data quality assessment elements listed in Table 2 of the Department’s Guidance Document “Data Quality Assessment Elements for Identification of Impaired Surface Waters” (DEP EAS 01-01, April 2001), which is incorporated by reference.

(9) For the assessment of metals criteria,

(a) Surface water data for mercury shall be collected and analyzed using clean sampling and analytical techniques, and

(b) The corresponding hardness value shall be required for freshwater metals criteria that are hardness dependent. If the ambient hardness value is less than 25 mg/L as CaCO3, then a hardness value of 25 will be used to calculate the criteria. If data are not used due to sampling or analytical techniques or because hardness data were not available, the Department shall note for the record that data were excluded and explain why they were excluded.

(10) Surface water data with values below the applicable practical quantification limit (PQL) or method detection limit (MDL) shall be assessed in accordance with paragraphs 62-4.246(6)(b)-(d) and subsection (8), F.A.C.

(a) If sampling entities want to ensure that their data will be considered for evaluation, they should review the Department’s list of approved MDLs and PQLs developed pursuant to Rule 62-4.246, F.A.C., and, if available, use approved analytical methods with MDLs below the applicable water quality criteria. If there are no approved methods with MDLs below a criterion, then the method with the lowest MDL should be used. Analytical results listed as below detection or below the MDL shall not be used for developing planning lists if the MDL was above the criteria and there were, at the time of sample collection, approved analytical methods with MDLs below the criteria on the Department’s list of approved MDLs and PQLs.

(b) If appropriate analytical methods were used, then data with values below the applicable MDL will be deemed to meet the applicable water quality criterion and data with values between the MDL and PQL will be deemed to be equal to the MDL.

(11) It should be noted that the data requirements of this rule constitute the minimum data set needed to assess a water segment for impairment. Agencies or groups designing monitoring networks are encouraged to consult with the Department to determine the sample design appropriate for their specific monitoring goals.

Specific Authority 403.061, 403.067 FS. Law Implemented 403.062, 403.067 FS. History–New 6-10-02, Amended 12-11-06.