62-304.415 Lower St. Johns River Basin TMDLS.

Lower St. Johns River.

(1) The Total Maximum Daily Load for the freshwater segments of the Lower St. Johns River, which is that portion of the river from Buffalo Bluff to Black Creek, is 500,325 kilograms per year (kg/y) of Total Phosphorus (TP) and 8,570,260 kg/y of Total Nitrogen (TN), and is allocated as follows:

(a) The Wasteload Allocation for point sources discharging wastewater to the freshwater portion of the river is 41,097 kg/y of TP and 207,347 kg/y of TN, and the Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is a 34.5 percent reduction in current TP loading and a 15.3 percent reduction in current TN loading,

(b) The Load Allocation for nonpoint sources is 459,228 kg/y of TP and 8,362,913 kg/y of TN, and

(c) The Margin of Safety is implicit.

(2) The Total Maximum Daily Load for the marine segments of the Lower St. Johns River, which is that portion of the river from Black Creek to the mouth, is 1,472,984 kilograms per year (kg/y) of Total Nitrogen (TN), and is allocated as follows:

(a) The Wasteload Allocation for point sources is 1,112,480 kg/y of TN,

(b) The Load Allocation for nonpoint sources is 360,504 kg/y of TN, and

(c) The Margin of Safety is implicit.

(3) Durbin Creek. The Total Maximum Daily Load for Durbin Creek is 400 counts/100 mL for fecal coliform, and is allocated as follows:

(a) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is a 63 percent reduction of current anthropogenic fecal coliform loading,

(b) The Load Allocation for nonpoint sources is a 63 percent reduction of current fecal coliform loading, and

(c) The Margin of Safety is implicit.

(4) Goodbys Creek. The Total Maximum Daily Load for Goodbys Creek is 400 counts/100 mL for fecal coliform, and is allocated as follows:

(a) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is an 87 percent reduction of current anthropogenic fecal coliform loading,

(b) The Load Allocation for nonpoint sources is an 87 percent reduction of current fecal coliform loading, and

(c) The Margin of Safety is implicit.

(5) Hogan Creek. The Total Maximum Daily Load for Hogan Creek is 400 counts/100 mL for fecal coliform, and is allocated as follows:

(a) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is a 92 percent reduction of current anthropogenic fecal coliform loading,

(b) The Load Allocation for nonpoint sources is a 92 percent reduction of current fecal coliform loading, and

(c) The Margin of Safety is implicit.

(6) Miramar Creek. The Total Maximum Daily Load for Miramar Creek is 400 counts/100 mL for fecal coliform, and is allocated as follows:

(a) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is a 92 percent reduction of current anthropogenic fecal coliform loading,

(b) The Load Allocation for nonpoint sources is a 92 percent reduction of current fecal coliform loading, and

(c) The Margin of Safety is implicit.

(7) Butcher Pen Creek. The Total Maximum Daily Load for Butcher Pen Creek is 400 counts/100 mL for fecal coliform, and is allocated as follows:

(a) The Wasteload Allocation for wastewater discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is to meet the applicable water quality criteria for fecal coliforms,

(b) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is an 83 percent reduction of current anthropogenic fecal coliform loading,

(c) The Load Allocation for nonpoint sources is an 83 percent reduction of current fecal coliform loading, and

(d) The Margin of Safety is implicit.

(8) Cedar River. The Total Maximum Daily Load for the Cedar River is 400 counts/100 mL for fecal coliform and 2,400 counts/100 mL for total coliform, and is allocated as follows:

(a) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is an 83 percent reduction of current anthropogenic fecal coliform loading and an 81 percent reduction of current anthropogenic total coliform loading,

(b) The Load Allocation for nonpoint sources is an 83 percent reduction of current fecal coliform loading and an 81 percent reduction of total coliform loading, and

(c) The Margin of Safety is implicit.

(9) Williamson Creek. The Total Maximum Daily Load for Williamson Creek is 400 counts/100 mL for fecal coliform and 2,400 counts/100 mL for total coliform, and is allocated as follows:

(a) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is an 83 percent reduction of current anthropogenic fecal coliform loading and a 66 percent reduction of current anthropogenic total coliform loading,

(b) The Load Allocation for nonpoint sources is an 83 percent reduction of current fecal coliform loading and a 66 percent reduction of total coliform loading, and

(c) The Margin of Safety is implicit.

(10) Wills Branch. The Total Maximum Daily Load for Wills Branch is 400 counts/100 mL for fecal coliform and 2,400 counts/100 mL for total coliform, and is allocated as follows:

(a) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is an 80 percent reduction of current anthropogenic fecal coliform loading and an 81 percent reduction of current anthropogenic total coliform loading,

(b) The Load Allocation for nonpoint sources is an 80 percent reduction of current fecal coliform loading and an 81 percent reduction of total coliform loading, and

(c) The Margin of Safety is implicit.

(11) Moncrief Creek. The Total Maximum Daily Load for Moncrief Creek is 400 counts/100 mL for fecal coliform and 2,400 counts/100 mL for total coliform, and is allocated as follows:

(a) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is an 83 percent reduction of current anthropogenic fecal coliform loading and a 98 percent reduction of current anthropogenic total coliform loading,

(b) The Load Allocation for nonpoint sources is an 83 percent reduction of current fecal coliform loading and a 98 percent reduction of total coliform loading, and

(c) The Margin of Safety is implicit.

(12) Ribault River. The Total Maximum Daily Load for the Ribault River is 400 counts/100 mL for fecal coliform, and is allocated as follows:

(a) The Wasteload Allocation for wastewater discharges subject to the Department’s National Pollutant Discharge Elimination System Permitting Program is to meet the applicable water quality criteria for fecal coliforms,

(b) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is a 50 percent reduction of current anthropogenic fecal coliform loading,

(c) The Load Allocation for nonpoint sources is a 50 percent reduction of current fecal coliform loading, and

(d) The Margin of Safety is implicit.

Specific Authority 403.061, 403.067 FS. Law Implemented 403.061, 403.062, 403.067 FS. History–New 12-3-03, Amended 5-15-06.