62-312.803 General Permit for Installation or Maintenance of Boat Ramps.

(1) A general permit is hereby granted to any person installing or maintaining a boat ramp; provided

(a) This general permit shall apply to only those boat ramps not exempt from permitting pursuant to Section 403.813(1)(c), F.S.; and

(b) No ramp shall be installed under this general permit unless navigational access to the proposed ramp exists; and

(c) There shall be no filling other than the actual boat ramp surface and incidental filling associated with recontouring the land under the ramp to create a level grade; and

(d) There shall be no more than 25 cubic yards of material removed for installation of the boat ramp surface; and

(e) The installation or use of the boat ramp shall not interfere with navigation in the water body; and

(f) The above-water, level portion of the ramp shall be landward of the mean or ordinary high water line; and

(g) All spoil material shall be deposited on self-contained upland spoil site which shall be located so as to prevent the escape of spoil material into waters of the state; and

(h) Maintenance dredging shall be limited to that amount of material necessary to restore the ramp to its original configuration and dimension; and

(i) The boat ramp shall not infringe upon the riparian rights of adjacent property owners; and

(j) The permittee shall remit to the Board of Trustees of the Internal Improvement Trust Fund payment for material removed from sovereignty or state-owned lands in accordance with the schedule established by the Board.

(2) This general permit shall be subject to the general conditions in Rule 62-4.540, F.A.C., and the following specific conditions:

(a) Turbidity curtains or similar devices shall be utilized to prevent violation of state water quality standards.

(b) The installation of a boat ramp does not obligate the Department to approve any subsequent request to dredge for navigational access.

Specific Authority 373.414(11)-(16), 373.4145, 403.805(1) FS. Law Implemented 253.123, 253.124, 373.414(11)-(16), 474.4145, 403.061, 403.087, 403.088, 403.813, 403.814, 403.817 FS. History–New 7-8-82, Formerly 17-4.57, 17-4.570, Amended 3-26-89, Formerly 17-12.803, 17-312.803.