62-312.804 General Permit for Riprap.

(1) A general permit is hereby granted to any person installing riprap at the toe of an existing vertical seawall, provided

(a) The riprap consists only of natural boulders or clean concrete rubble six (6) inches to three (3) feet in diameter or in average dimensions; and

(b) The slope of the riprap is no steeper than 2H:1V and the horizontal distance is no more than eight (8) feet; and

(c) There are no reinforcing rods or other similar protrusions in concrete rubble and all rubble or boulders are free of attached sediments; and

(d) Neither the distance nor the use of the riprap interferes with navigation or infringes upon the riparian rights of the adjacent property owners; and

(e) There is no filling or dredging associated with the placement of riprap other than the riprap material itself.

(2) This general permit shall be subject to the general conditions of Rule 62-4.540, F.A.C., and specific conditions as follows:

(a) Turbidity curtains or similar devices where appropriate shall be utilized to prevent violation of state water quality standards; and

(b) Installation of the structure does not obligate the Department to approve any subsequent request to dredge for navigational access; and

(c) There shall be no backfilling to obtain useable upland or to straighten an otherwise sinuous shoreline; and

(d) There shall be no filling or backfilling to reclaim land lost by avulsion or erosion.

Specific Authority 373.414(11)-(16), 373.4145, 403.805(1) FS. Law Implemented 253.123, 253.124, 373.414(11)-(16), 373.4145, 403.061, 403.087, 403.088, 403.813, 403.814, 403.817 FS. History–New 7-8-82, Formerly 17-4.58, 17-4.580, Amended 3-26-89, Formerly 17-12.804, 17-312.804.