62-312.805 General Permit for Headwalls and Culverts Associated with a Stormwater Discharge Facility.

(1) A general permit is hereby granted to any person to construct headwalls and culverts associated with a stormwater discharge facility; provided,

(a) A stormwater permit has been granted to the permittee by the Department or an entity on behalf of the Department; and

(b) The discharge from the stormwater system is to an artificial waterway; and

(c) The culvert shall not extend waterward of the headwall; and

(d) The culvert and the headwall shall not extend more than one (1) foot waterward of the existing shoreline.

(2) This general permit shall be subject to the general conditions in Rule 62-4.540, F.A.C., and the specific conditions as follows:

(a) Turbidity curtains or similar devices shall be utilized where appropriate to prevent violation of state water quality standards; and

(b) There shall be no backfilling to obtain useable upland or to straighten an otherwise sinuous shoreline.

Specific Authority 373.414(11)-(16), 373.4145, 403.805(1) FS. Law Implemented 253.123, 253.124, 373.414(11)-(16), 373.4145, 403.061, 403.087, 403.088, 403.813, 403.814, 403.817 FS. History–New 7-8-82, Formerly 17-4.59, 17-4.590, 17-12.805, 17-312.805.