62-312.807 General Permit for the Construction of Artificial Reefs.

(1) A general permit is hereby granted to any person to construct an artificial reef, provided:

(a) The material to be used shall be clean concrete or rock, or clean steel boat hulls; and

(b) The material shall be free of soils, oil and greases, debris, litter, putrescible substances or other pollutants; and

(c) The material shall be firmly anchored to the bottom and shall not be indiscriminately dumped; and

(d) The material shall be placed so that the top of the reef does not exceed 1/2 the distance from the bottom to the surface of the water unless a greater distance is required for safe navigation. At no time shall the distance between the top of the reef and the surface of the water be less than 6 feet.

(2) This general permit shall be subject to the general conditions of Rule 62-4.540, F.A.C., and the following specific conditions:

(a) The permittee shall conduct a survey of the bottomland on which the reef is to be built and shall submit the survey to the Department with notice pursuant to Rule 62-4.530, F.A.C., demonstrating that the bottom does not have grassbeds, or hardbottom or other corals; and

(b) There shall be no reefs constructed in shallow bay or estuarine bottoms; and

(c) There shall be no “white goods”, asphalt material, tires or other pollutant materials used in construction of the reef; and

(d) The site shall be marked with buoys to ensure that no material is deposited outside of the site; and

(e) The permittee shall notify the National Ocean Service, National Oceanographic and Atmospheric Association, U.S. Department of Commerce, Rockville, Maryland, of the precise location of the reef.

Specific Authority 373.414(11)-(16), 373.4145, 403.805(1) FS. Law Implemented 253.123, 253.124, 373.414(11)-(16), 373.4145, 403.061, 403.087, 403.088, 403.813, 403.814, 403.817 FS. History–New 4-26-84, Formerly 17-4.68, 17-4.680, 17-12.807, 17-312.807.