62-312.808 General Permit for Certain Piers and Associated Structures.

(1) A general permit is hereby granted to any person to construct minor works as described below:

(a) Construction of private, single-family piers along with associated boat lifts where the pier:

1. Does not accommodate the mooring of more than two water craft; and

2. Does not, together with its walkways and end “T” or “L”, exceed 6 feet in width; and

3. Is constructed in waters or to waters that are sufficiently deep to avoid bottom scour by vessel operation or by prop dredging.

(b) Construction of boat shelters associated with a pier where the shelter has no walls or doors.

(c) Construction of an extension to an existing private, single-family pier where the extension is no more than 6 feet in width and is designed to enhance safety of boat use, to reach navigable water depths, or to accommodate a larger boat.

(2) This general permit shall be subject to the general conditions of Rule 62-4.540, F.A.C., and specific conditions as follows:

(a) Construction shall not occur over submerged grassbeds.

(b) There shall be no bait houses, storage shelters, sun decks, gazebos, screen porches, wet bars, living quarters, or other non-water-dependent structures over waters of the state or on the pier.

(c) The pier and its use shall not interfere with navigation in the water body.

(d) There shall be no dredging associated with pier construction other than that required for installation of the pilings for the pier and mooring of the water craft.

(e) There shall be no fish cleaning facilities, boat repair facilities or equipment and fueling facilities on the pier.

(f) The pier and its use shall not infringe on the riparian rights of adjacent property owners and shall be in accordance with the Rules of the Board of Trustees of the Internal Improvement Trust Fund, when construction occurs on state-owned bottomland.

Specific Authority 373.414(11)-(16), 373.4145, 403.805(1) FS. Law Implemented 253.123, 253.124, 373.414(11)-(16), 373.4145, 403.061, 403.087, 403.088, 403.813, 403.814, 403.817 FS. History–New 4-26-84, Formerly 17-4.69, 17-4.690, Amended 3-26-89, Formerly 17-12.808, 17-312.808.