62-312.809 General Permit to the Florida Department of Transportation for Minor Bridge Construction.

(1) A general permit is hereby granted to the Florida Department of Transportation to conduct the work described below:

(a) The construction of bridges and approaches parallel and adjacent to existing bridges where the combined total of dredging and filling, both temporary and permanent, in waters of the State does not exceed 0.50 acre.

(b) The replacement or modification of bridges and approaches where the combined total of dredging and filling, both temporary and permanent, in waters of the State does not exceed 0.50 acre.

(c) The construction of bridges over waters of the State where no permanent dredging or filling is required other than placement of bridge support structures, and the combined total of temporary dredging and filling does not exceed 0.25 acre.

(d) Channel clearing and shaping, not to exceed a combined total of 0.5 acres of dredging and/or filling in waters of the State, to facilitate maximum hydraulic efficiency of the structures detailed in paragraphs (a) and (b) above, where the spoil material is used on an upland portion of the project or is deposited on a self-contained, upland spoil site. Escape of spoil material and/or return water from the spoil deposition area into waters of the State is prohibited.

(2) This general permit shall be subject to the general conditions of Rule 62-4.540, F.A.C., and the following specific conditions:

(a) In addition to compliance with the notice provisions of Rule 62-4.530, F.A.C., at least 90 days prior to commencement of construction the permittee will notify the appropriate Department district office of the date the permitted construction activities are planned to begin, and within 90 days following completion of construction, the permittee will notify by letter the appropriate Department district office of the date construction activities were completed.

(b) No access or work channels are authorized by this general permit.

(c) Temporary fill roads may not be constructed waterward of mean high water or ordinary high water.

(d) All temporary fill areas shall be regarded to original elevations and revegetated with native wetland species.

(e) This general permit shall not apply to bridge construction in Class I or Class II surface waters, or waters designated as Outstanding Florida Waters.

(f) Hydraulic openings of new bridges constructed under paragraph (1)(c) above shall be sufficient to prevent downstream scour, increased downstream water velocities, and significant backwater conditions upstream of the bridge structure.

(g) Minimum horizontal and vertical navigational clearances on new or replacement bridges over navigable waters of the United States shall be established in accordance with procedures outlined in the U.S. Coast Guard Bridge Administration Manual, COMDTINST M16590.5, May 7, 1982.

(h) Horizontal and vertical clearances for replacement bridge structures shall meet or exceed those of the bridge being replaced.

(i) Horizontal and vertical clearances on parallel bridges adjacent to existing bridges shall meet or exceed those of the existing bridge.

(j) The permittee shall limit crossings of any particular stream to an average of one every 0.5 mile.

(k) Temporary erosion controls for all exposed soils within jurisdictional areas shall be completed within seven calendar days of the most recent construction activity.

(l) The fill areas and the banks of the water body shall be stabilized with vegetation and/or riprap as soon as possible following completion of slope construction. This stabilization is considered a construction priority, and completed fill slopes in jurisdictional areas shall not remain unstabilized while other construction continues.

(m) This general permit does not authorize the use of dredged material for roadway construction.

(n) The permittee shall use best management practices, including turbidity curtains or similar devices, in strict adherence to the Florida Department of Transportation “Standard Specifications for Road and Bridge Construction, Edition of 1986” to prevent violations of State water quality standards.

(o) This general permit authorizes dredging and filling activities associated with minor bridge construction only and does not relieve the permittee from compliance with any other applicable Department sections.

(3) This general permit shall apply only where all required permits for stormwater discharge have been received prior to beginning of construction.

Specific Authority 373.414(11)-(16), 373.4145, 403.805(1) FS. Law Implemented 373.414(11)-(16), 373.4145, 403.061, 403.087, 403.088, 403.813, 403.814, 403.817, 403.912, 403.913, 403.918, 403.931 FS. History–New 7-11-90, Amended 1-29-91, Formerly 17-312.809.