62-312.813 General Permit for the Installation of Fences.

(1) A general permit is hereby granted to any person installing a fence in waters of the state, except in Outstanding Florida Waters, Aquatic Preserves, Class II waters, or waters approved for shellfish harvesting; provided:

(a) The structure shall not block navigation, create a navigational hazard, impede the natural flow of water by itself or through the accumulation of debris, and

(b) No fence shall be installed into open waters (areas of water bodies not supporting emergent vegetation) of any navigable river, stream, canal, or tributary thereto, a distance of more than twenty-five (25) feet, or more than twenty percent (20%) of the width of the open water, whichever is less, and in any lake shall not extend more than fifteen (15) feet waterward of the landward extent of waters of the state, and

(c) The fence will be constructed of wire attached to wood or mental posts which project at least two (2) feet above the minimum flood or ordinary high water elevation of the waterway, and

(d) In navigable waters and all lakes, the structure shall be adequately marked with reflectors visible from both directions paralleling the shoreline, and

(e) Dredging or filling performed shall be limited to that necessary to install individual fence posts, and

(f) The installation shall not infringe on the riparian rights of adjacent property owners and shall be in accordance with the sections promulgated under the authority of the Board of Trustees of the Internal Improvement Trust Fund when installation occurs on state-owned bottomland.

(2)(a) This general permit shall be subject to the general conditions of Rule 62-4.540, F.A.C., and the following specific condition:

(b) Turbidity curtains or similar devices shall be utilized, as necessary, to prevent violation of state water quality standards.

(3) This general permit shall not be used to regulate agricultural installation of fences pursuant to Section 403.913, F.S.

Specific Authority 373.414(11)-(16), 373.4145, 403.805(1) FS. Law Implemented 373.414(11)-(16), 373.4145, 403.061, 403.087, 403.088, 403.813, 403.814, 403.912, 403.913, 403.918, 403.919 FS. History– New 9-3-86, Formerly 17-4.780, 17-12.813, 17-312.813.