62-312.814 General Permit to Florida Game and Fresh Water Fish Commission, U.S. Forest Service, and County and Municipal Governments for the Construction of Freshwater Fish Attractors.

(1) A general permit is hereby granted to the Florida Game and Fresh Water Fish Commission, the U.S. Forest Service, and County and Municipal Governments to construct a fish attractor in fresh water lakes; provided:

(a) That any fish attractor to be constructed by County or Municipal Government must have the written approval of the Florida Game and Fresh Water Fish Commission, and

(b) The material to be used shall be clean concrete or rock, or brush, or logs, or trees, and

(c) The material shall be free of soils, preservatives, oil and greases, debris, litter, putrescible substances, “white goods”, asphalt material, tires, or other pollutants, and

(d) The material shall be firmly anchored to the bottom and shall not be indiscriminately dumped, and

(e) The size of an individual fish attractor shall not exceed one quarter of an acre in area, and

(f) The material shall be placed so that the top of the fish attractor is at least three (3) feet below the surface of the water at ordinary low water and shall be outside any posted navigational channels.

(2) This general permit shall be subject to the general conditions of Rule 62-4.540, F.A.C., and the following specific conditions:

(a) No fish attractor material shall be placed on or in areas vegetated by aquatic herbs or algae (except nuisance exotics), and

(b) The site shall be marked with a buoy or buoys to ensure that no material is deposited outside of the site.

Specific Authority 373.414(11)-(16), 373.4145, 403.805(1) FS. Law Implemented 373.414(11)-(16), 373.4145, 403.061, 403.087, 403.088, 403.813, 403.814, 403.912, 403.913, 403.918, 403.919 FS. History–New 9-3-86, Formerly 17-4.790, 17-12.814, 17-312.814.