- 62-304.625 Peace River Basin TMDLs.

(1) Lake Cannon. The nutrient TMDL for Lake Cannon is an annual in-lake geometric mean concentration of 1.07 mg/L Total Nitrogen (TN) and 0.03 mg/L Total Phosphorus (TP) which are intended to achieve the applicable annual geometric mean (AGM) chlorophyll a criterion for low color and high alkalinity lakes, and is allocated as follows:

(a) The WLA for wastewater sources is not applicable;

(b) The WLA for discharges subject to the Department’s National Pollutant Discharge Elimination System (NPDES) Municipal Separate Stormwater Sewer System (MS4) Permitting Program is a 20% reduction of TN (calculated from 1.33 mg/L) and 0% reduction of TP, which is based on the highest AGM concentrations from the 2008 ‒ 2016 period;

(c) The Load Allocation (LA) for nonpoint sources is 20% reduction of TN (calculated from 1.33 mg/L) and 0% reduction of TP, which is based on the highest AGM concentration from the 2008 ‒ 2016 period, and

(d) The Margin of Safety is implicit.

(e) While the LA for TN and TP has been expressed as the percent reduction needed to attain the applicable Class III nutrient criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the restoration of nutrient conditions in the impaired waterbody. However, it is not the intent of this TMDL to abate natural background conditions.

(2) Lake Howard. The nutrient TMDL for Lake Howard is an AGM concentration of 1.07 mg/L TN and 0.03 mg/L TP, which are intended to achieve the applicable AGM chlorophyll a for low color and high alkalinity lakes, and is allocated as follows:

(a) The WLA for wastewater sources is not applicable;

(b) The WLA for discharges subject to the Department’s NPDES MS4 Permitting Program is a 39% reduction of TN (calculated from 1.76 mg/L) and 0% reduction of TP, which is based on the highest AGM concentrations from the 2008 – 2016 period;

(c) The LA for nonpoint sources is 39% reduction of TN (calculated from 1.76 mg/L) and 0% reduction of TP, which is based on the highest AGM concentration from the 2008 ‒ 2016 period, and

(d) The Margin of Safety is implicit.

(e) While the LA for TN and TP has been expressed as the percent reduction needed to attain the applicable Class III nutrient criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the restoration of nutrient conditions in the impaired waterbody. However, it is not the intent of this TMDL to abate natural background conditions.

(3) Lake Idylwild. The nutrient TMDL for Lake Idylwild is an AGM concentration of 1.07 mg/L TN and 0.03 mg/L TP, which are intended to achieve the applicable AGM chlorophyll a for low color and high alkalinity lakes, and is allocated as follows:

(a) The WLA for wastewater sources is not applicable;

(b) The WLA for discharges subject to the Department’s NPDES MS4 Permitting Program is a 31% reduction of the TN (calculated from 1.54 mg/L) and 0% reduction of TP, which is based on the highest AGM concentrations from the 2008 ‒ 2016 period;

(c) The LA for nonpoint sources is 31% reduction of TN (calculated from 1.54 mg/L) and 0% reduction of TP, which is based on the highest AGM concentration from the 2008 ‒ 2016 period, and

(d) The Margin of Safety is implicit.

(e) While the LA for TN and TP has been expressed as the percent reduction needed to attain the applicable Class III nutrient criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the restoration of nutrient conditions in the impaired waterbody. However, it is not the intent of this TMDL to abate natural background conditions.

(4) Lake Jessie. The nutrient TMDL for Lake Jessie is an AGM concentration of 1.07 mg/L TN and 0.03 mg/L TP, which are intended to achieve the applicable AGM chlorophyll a for low color and high alkalinity lakes, and is allocated as follows:

(a) The WLA for wastewater sources is not applicable;

(b) The WLA for discharges subject to the Department’s NPDES MS4 Permitting Program is a 12% reduction of TN (calculated from 1.21 mg/L) and 0% reduction of TP, which is based on the highest AGM concentrations from the 2008 – 2016 period;

(c) The LA for nonpoint sources is 12% reduction of TN (calculated from 1.21 mg/L) and 0% reduction of TP, which is based on the highest AGM concentration from the 2008 ‒ 2016 period, and

(d) The Margin of Safety is implicit.

(e) While the LA for TN and TP has been expressed as the percent reduction needed to attain the applicable Class III nutrient criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the restoration of nutrient conditions in the impaired waterbody. However, it is not the intent of this TMDL to abate natural background conditions.

(5) Lake Lulu. The nutrient TMDL for Lake Lulu is an AGM concentration of 1.07 mg/L TN and 0.03 mg/L TP, which are intended to achieve the applicable AGM chlorophyll a for low color and high alkalinity lakes, and is allocated as follows:

(a) The WLA for wastewater sources is not applicable;

(b) The WLA for discharges subject to the Department’s NPDES MS4 Permitting Program is a 35% reduction of TN (calculated from 1.64 mg/L) and 0% reduction of TP, which is based on the highest AGM concentrations from the 2008 – 2016 period;

(c) The LA for nonpoint sources is 35% reduction of TN (calculated from 1.64 mg/L) and 0% reduction of TP, which is based on the highest AGM concentration from the 2008 ‒ 2016 period, and

(d) The Margin of Safety is implicit.

(e) While the LA for TN and TP has been expressed as the percent reduction needed to attain the applicable Class III nutrient criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the restoration of nutrient conditions in the impaired waterbody. However, it is not the intent of this TMDL to abate natural background conditions.

(6) Lake May. The nutrient TMDL for Lake May is an AGM concentration of 1.07 mg/L TN and 0.03 mg/L TP, which are intended to achieve the applicable AGM chlorophyll a for low color and high alkalinity lakes, and is allocated as follows:

(a) The WLA for wastewater sources is not applicable;

(b) The WLA for discharges subject to the Department’s NPDES MS4 Permitting Program is a 44% reduction of TN (calculated from 1.90 mg/L) and a 57% reduction of TP (calculated from 0.07 mg/L), which is based on the highest AGM concentrations from the 2008 – 2016 period;

(c) The LA for nonpoint sources is 44% reduction of TN (calculated from 1.90 mg/L) and 57% reduction of TP (calculated from 0.07 mg/L), which is based on the highest AGM concentration from the 2008 ‒ 2016 period, and

(d) The Margin of Safety is implicit.

(e) While the LA for TN and TP has been expressed as the percent reduction needed to attain the applicable Class III nutrient criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the restoration of nutrient conditions in the impaired waterbody. However, it is not the intent of this TMDL to abate natural background conditions.

(7) Lake Mirror. The Total Maximum Daily Load for Lake Mirror for Total Phosphorus (TP) is 55 kilograms per year (kg/year), and is allocated as follows:

(a) The Wasteload Allocation for wastewater sources is not applicable because there are no permitted point sources authorized to discharge wastewater to the lake;

(b) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is a 28 percent reduction of the average TP loading for the 1990 to 1999 period;

(c) The Load Allocation for nonpoint sources is 51 kg/year of TP; and,

(d) The Margin of Safety is 4 kg/year of TP.

(8) Lake Shipp. The nutrient TMDL for Lake Shipp is an AGM concentration of 1.07 mg/L TN and 0.03 mg/L TP, which are intended to achieve the applicable AGM chlorophyll a for low color and high alkalinity lakes, and is allocated as follows:

(a) The WLA for wastewater sources is not applicable;

(b) The WLA for discharges subject to the Department’s NPDES MS4 Permitting Program is a 49% reduction of TN (calculated from 2.08 mg/L) and 0% reduction of TP, which is based on the highest AGM concentrations from the 2008 – 2016 period;

(c) The LA for nonpoint sources is 49% reduction of TN (calculated from 2.08 mg/L) and 0% reduction of TP, which is based on the highest AGM concentration from the 2008 ‒ 2016 period, and

(d) The Margin of Safety is implicit.

(e) While the LA for TN and TP has been expressed as the percent reduction needed to attain the applicable Class III nutrient criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the restoration of nutrient conditions in the impaired waterbody. However, it is not the intent of this TMDL to abate natural background conditions.

(9) Wahneta Farms Drainage Canal. The fecal coliform Total Maximum Daily Load for Wahneta Farms Drainage Canal Creek is 400 counts/100 mL, and is allocated as follows:

(a) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1999 to 2004 period, will require a 39% reduction at sources contributing to exceedances of the criteria;

(b) The Load Allocation for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1999 to 2004 period, will require a 39% reduction at sources contributing to exceedances of the criteria; and,

(c) The Margin of Safety is implicit.

(10) Peace River Above Bowlegs Creek. The fecal coliform Total Maximum Daily Load for the Peace River Above Bowlegs Creek is a median of 2.29 x 1012 colonies/day under “moist conditions,” which are defined as flows ranging from 99 to 665 cubic feet per second (cfs), and a median of 1.66 x 1011 colonies/day under “dry conditions,” which are defined as flows ranging from 6.4 to 35 cfs, and is allocated as follows:

(a) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1993 to 2003 period, will require a 23% reduction at sources contributing to exceedances of the criteria during moist conditions and a 52% reduction at sources contributing to exceedances of the criteria during dry conditions;

(b) The Load Allocations for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1993 to 2003 period, will require a 23% reduction at sources contributing to exceedances of the criteria during moist conditions and a 52% reduction at sources contributing to exceedances of the criteria during dry conditions; and,

(c) The Margin of Safety is implicit.

(11) Peace Creek Drainage Canal. The fecal coliform Total Maximum Daily Load for Peace Creek Drainage Canal is a median of 3.62 x 1011 colonies/day, and is allocated as follows:

(a) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Wastewater Permitting Program is to meet applicable water quality criteria for fecal coliform;

(b) The Wasteload Allocation for discharges subject to the Department’s National Pollutant Discharge Elimination System Municipal Stormwater Permitting Program is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1992 to 2004 period, will require a 62% reduction at sources contributing to exceedances of the criteria;

(c) The Load Allocation for nonpoint sources is to address anthropogenic sources in the basin such that in-stream concentrations meet the fecal coliform criteria which, based on the measured concentrations from the 1992 to 2004 period, will require a 62% reduction at sources contributing to exceedances of the criteria; and,

(d) The Margin of Safety is implicit.

(12) While the Load Allocation and Wasteload Allocation for fecal coliform as set forth in subsections (9), (10) and (11), above, have been expressed as the percent reductions needed to attain the applicable Class III criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the required reduction of in-stream fecal coliform concentrations. However, it is not the intent of these Total Maximum Daily Loads to abate natural background conditions.

(13) Lake Bonny: The nutrient Total Maximum Daily Load (TMDL) for Lake Bonny is an annual in-lake geometric mean concentration of 0.89 mg/L Total Nitrogen (TN) and 0.04 mg/L TP, and is allocated as follows:

(a) The wasteload allocation (WLA) for wastewater point sources is not applicable;

(b) The WLA for discharges subject to the Department’s NPDES Municipal Separate Storm Sewer System (MS4) Permitting Program is a 64% reduction of TN and a 60% reduction of TP based on mean concentrations from the 2002-2012 period;

(c) The Load Allocation (LA) for nonpoint sources is a 64% reduction of TN and a 60% reduction of TP based on mean concentrations from the 2002-2012 period; and,

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for TN and TP has been expressed as the percent reduction needed to attain the applicable Class III nutrient criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the restoration of nutrient conditions in the impaired waterbody. However, it is not the intent of this TMDL to abate natural background conditions.

(14) Lake Hollingsworth: The nutrient TMDL for Lake Hollingsworth is an annual in-lake geometric mean concentration of 0.86 mg/L TN and 0.03 mg/L TP, which is intented to achieve the applicable annual geometric mean chlorophyll a criterion for low color, high alkalinity lakes, and is allocated as follows:

(a) The WLA for wastewater point sources is not applicable;

(b) The WLA for discharges subject to the Department’s NPDES MS4 Permitting Program is a 68% reduction of TN (calculated from 2.66 mg/L) and a 75% reduction of TP (calculated from 0.12 mg/L), which are the highest annual geometric based on mean concentrations from the 2002-2012 period;

(c) The LA for nonpoint sources is a 68% reduction of TN (calculated from 2.66 mg/L) and a 75% reduction of TP (calculated from 0.12 mg/L), which are the highest annual geometric based on mean concentrations from the 2002-2012 period; and,

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for TN and TP has been expressed as the percent reduction needed to attain the applicable Class III nutrient criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the restoration of nutrient conditions in the impaired waterbody. However, it is not the intent of this TMDL to abate natural background conditions.

(15) Lake Lena: The nutrient TMDL for Lake Lena is an annual in-lake geometric mean concentration of 1.14 mg/L TN, and is allocated as follows:

(a) The WLA for wastewater point sources is not applicable;

(b) The WLA for discharges subject to the Department’s NPDES MS4 Permitting Program is a 42% reduction of TN based on mean concentrations from the 2003-2012 period;

(c) The LA for nonpoint sources is a 42% reduction of TN based on mean concentrations from the 2003-2012 period; and,

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for TN has been expressed as the percent reduction needed to attain the applicable Class III nutrient criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the restoration of nutrient conditions in the impaired waterbody. However, it is not the intent of this TMDL to abate natural background conditions.

(16) Deer Lake: The nutrient TMDL for Deer Lake is an annual in-lake geometric mean concentration of 1.42 mg/L TN, and is allocated as follows:

(a) The WLA for wastewater point sources is not applicable;

(b) The WLA for discharges subject to the Department’s NPDES MS4 Permitting Program is a 12% reduction of TN based on mean concentrations from the 2005-2012 period;

(c) The LA for nonpoint sources is a 12% reduction of TN based on mean concentrations from the 2005-2012 period; and,

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for TN has been expressed as the percent reduction needed to attain the applicable Class III nutrient criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the restoration of nutrient conditions in the impaired waterbody. However, it is not the intent of this TMDL to abate natural background conditions.

(17) Lake Haines. The nutrient TMDL for Lake Haines is an annual geometric mean concentration of 1.05 mg/L TN and 0.03 mg/L TP, which is intended to achieve the applicable annual geometric mean chlorophyll a criterion for high color lakes, and is allocated as follows:

(a) The WLA for wastewater point sources is not applicable,

(b) The WLA for discharges subject to the Department’s NPDES MS4 Permitting Program is a 33% reduction of TN (calculated from 1.56 mg/L) and a 0% reduction of TP (calculated from 0.03 mg/L), which are the highest annual geometric mean concentrations from the 2003-2016 period,

(c) The LA for nonpoint sources is a 33% reduction of TN (calculated from 1.56 mg/L) and a 0% reduction of TP (calculated from 0.03 mg/L), which are the highest annual geometric mean concentration from the 2003-2016 period, and

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for TN and TP has been expressed as the percent reduction needed to attain the applicable Class III nutrient criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the restoration of nutrient conditions in the impaired waterbody. However, it is not the intent of this TMDL to abate natural background conditions.

(18) Lake Rochelle. The nutrient TMDL for Lake Rochelle is an annual geometric mean concentration of 1.05 mg/L TN and 0.03 mg/L TP, which is intended to achieve the applicable annual geometric mean chlorophyll a criterion for low color, high alkalinity lakes, and is allocated as follows:

(a) The WLA for wastewater point sources is not applicable,

(b) The WLA for discharges subject to the Department’s NPDES MS4 Permitting Program is a 32% reduction of TN (calculated from 1.54 mg/L) and a 0% reduction of TP (calculated from 0.03 mg/L), which are the highest annual geometric mean concentrations from the 2003-2016 period,

(c) The LA for nonpoint sources is a 32% reduction of TN (calculated from 1.54 mg/L) and a 0% reduction of TP (calculated from 0.03 mg/L), which are the highest annual geometric mean concentrations from the 2003-2016 period, and

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for TN and TP has been expressed as the percent reduction needed to attain the applicable Class III nutrient criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the restoration of nutrient conditions in the impaired waterbody. However, it is not the intent of this TMDL to abate natural background conditions.

(19) Lake Conine. The nutrient TMDL for Lake Conine is an annual geometric mean concentration of 1.05 mg/L TN and 0.03 mg/L TP, which is intended to achieve the applicable annual geometric mean chlorophyll a criterion for low color, high alkalinity lakes, and is allocated as follows:

(a) The WLA for wastewater point sources is not applicable,

(b) The WLA for discharges subject to the Department’s NPDES MS4 Permitting Program is a 36% reduction of TN (calculated from 1.65 mg/L) and a 57% reduction of TP (calculated from 0.07 mg/L), which are the highest annual geometric mean concentrations from the 2003-2016 period,

(c) The LA for nonpoint sources is a 36% reduction of TN (calculated from 1.65 mg/L) and a 57% reduction of TP (calculated from 0.07 mg/L), which are the highest annual geometric mean concentrations from the 2003-2016 period, and

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for TN and TP has been expressed as the percent reduction needed to attain the applicable Class III nutrient criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the restoration of nutrient conditions in the impaired waterbody. However, it is not the intent of this TMDL to abate natural background conditions.

(20) Lake Alfred. The nutrient TMDL for Lake Alfred is an annual geometric mean concentration of 1.69 mg/L TN and 0.03 mg/L TP, which is intended to achieve the applicable annual geometric mean chlorophyll a criterion for low color, high alkalinity lakes, and is allocated as follows:

(a) The WLA for wastewater point sources is not applicable,

(b) The WLA for discharges subject to the Department’s NPDES MS4 Permitting Program is a 16% reduction of TN (calculated from 2.00 mg/L) and a 0% reduction of TP (calculated from 0.03 mg/L), which are the highest annual geometric mean concentrations from the 2003-2016 period,

(c) The LA for nonpoint sources is a 16% reduction of TN (calculated from 2.00 mg/L) and a 0% reduction of TP (calculated from 0.03 mg/L), which are the highest annual geometric mean concentrations from the 2003-2016 period, and

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for TN and TP has been expressed as the percent reduction needed to attain the applicable Class III nutrient criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the restoration of nutrient conditions in the impaired waterbody. However, it is not the intent of this TMDL to abate natural background conditions.

(21) Lake Blue. The nutrient TMDL for Lake Blue is an annual geometric mean concentration of 1.16 mg/L TN and 0.03 mg/L TP, which is intended to achieve the applicable annual geometric mean chlorophyll a criterion for low color, high alkalinity lakes, and is allocated as follows:

(a) The WLA for wastewater point sources is not applicable,

(b) The WLA for discharges subject to the Department’s NPDES MS4 Permitting Program is a 66% reduction of TN (calculated from 3.45 mg/L) and a 67% reduction of TP (calculated from 0.09 mg/L), which are the highest annual geometric mean concentrations from the 2003-2016 period,

(c) The LA for nonpoint sources is a 66% reduction of TN (calculated from 3.45 mg/L) and a 67% reduction of TP (calculated from 0.09 mg/L), which are the highest annual geometric mean concentrations from the 2003-2016 period, and

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for TN and TP has been expressed as the percent reduction needed to attain the applicable Class III nutrient criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the restoration of nutrient conditions in the impaired waterbody. However, it is not the intent of this TMDL to abate natural background conditions.

(22) Lake Marianna. The nutrient TMDL for Lake Marianna is an annual geometric mean concentration of 1.00 mg/L TN and 0.03 mg/L TP, which is intended to achieve the applicable annual geometric mean chlorophyll a criterion for low color, high alkalinity lakes, and is allocated as follows:

(a) The WLA for wastewater point sources is not applicable,

(b) The WLA for discharges subject to the Department’s NPDES MS4 Permitting Program is a 44% reduction of TN (calculated from 1.79 mg/L) and a 0% reduction of TP (calculated from 0.03 mg/L), which are the highest annual geometric mean concentrations from the 2003-2016 period,

(c) The LA for nonpoint sources is a 44% reduction of TN (calculated from 1.79 mg/L) and a 0% reduction of TP (calculated from 0.03 mg/L), which are the highest annual geometric mean concentrations from the 2003-2016 period, and

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for TN and TP has been expressed as the percent reduction needed to attain the applicable Class III nutrient criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the restoration of nutrient conditions in the impaired waterbody. However, it is not the intent of this TMDL to abate natural background conditions.

(23) Lake Ariana. The nutrient TMDL for Lake Ariana is an annual geometric mean concentration of 0.97 mg/L TN and 0.03 mg/L TP, which is intended to achieve the applicable annual geometric mean chlorophyll a criterion for low color, high alkalinity lakes, and is allocated as follows:

(a) The WLA for wastewater point sources is not applicable,

(b) The WLA for discharges subject to the Department’s NPDES MS4 Permitting Program is a 36% reduction of TN (calculated from 1.51 mg/L) and a 0% reduction of TP (calculated from 0.03 mg/L), which are the highest annual geometric mean concentrations from the 2003-2016 period,

(c) The LA for nonpoint sources is a 36% reduction of TN (calculated from 1.51 mg/L) and a 0% reduction of TP (calculated from 0.03 mg/L), which are the highest annual geometric mean concentrations from the 2003-2016 period, and

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for TN and TP has been expressed as the percent reduction needed to attain the applicable Class III nutrient criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the restoration of nutrient conditions in the impaired waterbody. However, it is not the intent of this TMDL to abate natural background conditions.

(24) Eagle Lake. The nutrient TMDL for Eagle Lake is an annual geometric mean concentration of 0.63 mg/L TN and 0.01 mg/L TP, which is intended to achieve the applicable annual geometric mean chlorophyll a criterion for low color, low alkalinity lakes, and is allocated as follows:

(a) The WLA for wastewater point sources is not applicable,

(b) The WLA for discharges subject to the Department’s NPDES MS4 Permitting Program is a 38% reduction of TN (calculated from 1.01 mg/L) and a 50% reduction of TP (calculated from 0.02 mg/L), which are the highest annual geometric mean concentrations from the 2003-2016 period,

(c) The LA for nonpoint sources is a 38% reduction of TN (calculated from 1.01 mg/L) and a 50% reduction of TP (calculated from 0.02 mg/L), which are the highest annual geometric mean concentrations from the 2003-2016 period, and

(d) The Margin of Safety is implicit.

(e) While the LA and WLA for TN and TP has been expressed as the percent reduction needed to attain the applicable Class III nutrient criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the restoration of nutrient conditions in the impaired waterbody. However, it is not the intent of this TMDL to abate natural background conditions.

(25) Lake Eloise. The nutrient TMDL for Lake Eloise is an AGM concentration of 1.07 mg/L TN and 0.03 mg/L TP, which are intended to achieve the applicable AGM chlorophyll a for low color and high alkalinity lakes, and is allocated as follows:

(a) The WLA for wastewater sources is not applicable;

(b) The WLA for discharges subject to the Department’s NPDES MS4 Permitting Program is a 33% reduction of TN (calculated from 1.59 mg/L) and 0% reduction of TP, which is based on the highest AGM concentrations from the 2008 – 2016 period;

(c) The LA for nonpoint sources is 33% reduction of TN (calculated from 1.59 mg/L) and 0% reduction of TP, which is based on the highest AGM concentration from the 2008 ‒ 2016 period, and

(d) The Margin of Safety is implicit.

(e) While the LA for TN and TP has been expressed as the percent reduction needed to attain the applicable Class III nutrient criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the restoration of nutrient conditions in the impaired waterbody. However, it is not the intent of this TMDL to abate natural background conditions.

(26) Lake Hartridge. The nutrient TMDL for Lake Hartridge is an AGM concentration of 1.07 mg/L TN and 0.03 mg/L TP, which are intended to achieve the applicable AGM chlorophyll a for low color and high alkalinity lakes, and is allocated as follows:

(a) The WLA for wastewater sources is not applicable;

(b) The WLA for discharges subject to the Department’s NPDES MS4 Permitting Program is a 36% reduction of TN (calculated from 1.66 mg/L) and 0% reduction of TP, which is based on the highest AGM concentrations from the 2008 – 2016 period;

(c) The LA for nonpoint sources is 36% reduction of TN (calculated from 1.66 mg/L) and 0 % reduction of TP, which is based on the highest AGM concentration from the 2008 ‒ 2016 period, and

(d) The Margin of Safety is implicit.

(e) While the LA for TN and TP has been expressed as the percent reduction needed to attain the applicable Class III nutrient criteria, it is the combined reductions from both anthropogenic point and nonpoint sources that will result in the restoration of nutrient conditions in the impaired waterbody. However, it is not the intent of this TMDL to abate natural background conditions.

Rulemaking Authority 403.061, 403.067 FS. Law Implemented 403.061, 403.062, 403.067 FS. History–New 5-1-07, Amended 2-19-15, 8-19-18, 10-29-19.