62-505.650 Priority Determination.

The project priority score is determined based upon public health as well as surface and ground water pollution considerations. The quantity of existing flow that will be collected, treated, or which otherwise generates the need for the project shall be used as a further determinant of priority. Special consideration shall be given to a project which will assist in the restoration or protection of a priority water body identified in an adopted Surface Water Improvement and Management (SWIM) Plan. The project priority score shall be rounded to the nearest one ten-thousandth (0.0001) of a point.

(1) Each project shall be categorized by the highest single base score justified by any one of its components or facilities, and it shall be assigned to one of the categories indicated below.

Project Category		 Base Score

Projects to eliminate public health hazards	140

Projects to achieve compliance with laws requiring elimination of	120

discharges to specific water bodies

Projects to achieve compliance with other pollution control requirements:

Toxics control	100

Residuals management	80

Reclaimed water (excluding discharge to surface water)	80

Treatment to reduce nutrients before discharge to surface water	60

Other facilities	40

Projects not required to achieve compliance with pollution control

requirements:

Reuse of reclaimed water or residuals	30

Rehabilitation of existing wastewater transmission systems necessary for	30

the hydraulic or structural integrity of the sewer system

Elimination of infiltration/inflow	30

Projects not otherwise categorized	20

(2) The project’s base score shall be adjusted by multiplying it by the flow factor for the highest existing average daily flow for a one-month period generating the need for the associated facility. However, a minimum flow factor of 5.0000, shall be used. Flow factors for any flow not listed below may be determined by interpolating between the data tabulated below or by using the formula:

Flow factor = 5.0000 + [(flow in gallons per day)/1,000,000]

Flow (gallons per day)		Flow Factor

100	5.0001

1,000	5.0010

10,000	5.0100

100,000	5.1000

1,000,000	6.0000

(3) The adjusted base score shall be further increased by multiplying it by 1.2 if the associated facility will assist in the restoration or protection of a water body with an adopted SWIM Plan.

Specific Authority 403.1835(7), 403.1838(3)(a), (b) FS. Law Implemented 403.1835, 403.1838 FS. History–New 11-30-98, Amended 7-22-99.