62-520.200 Definitions for Ground Water.

(1) “Acute Toxicity” shall have the same meaning as provided in subsection 62-302.200(1), F.A.C.

(2) “Aquifer” means a geologic formation, group of formations, or part of a formation capable of yielding a significant amount of ground water to wells, springs or surface water.

(3) “Background” means the condition of waters in the absence of the activity or discharge under consideration, based on the best scientific information available to the Department.

(4) “Commission” means the Environmental Regulation Commission.

(5) “Confined Aquifer” means an aquifer bounded above and below by impermeable beds or by beds of distinctly lower permeability than that of the aquifer itself.

(6) “Department” means the Department of Environmental Protection.

(7) “Designated Use” means the present and future most beneficial use of a body of water as designated by the Environmental Regulation Commission by means of the classification system contained in this chapter.

(8) “Effluent” means either domestic effluent or industrial effluent and has the same definition as effluent in Rule 62-600.200 or 62-660.200, F.A.C.

(9) “Effluent Limitation” means any restriction established by the Department on quantities, rates or concentrations of chemical, physical, biological or other constituents which are discharged from sources into waters of the State.

(10) “Ground Water” means water beneath the surface of the ground within a zone of saturation, whether or not flowing through known and definite channels.

(11) “Installation” means any structure, equipment, facility, or appurtenances thereto, operation or activity which may be a source of pollution.

(12) “Natural Background” means the condition of waters in the absence of man-induced alterations based on the best scientific information available to the Department. The establishment of natural background may be based on historical pre-alteration data.

(13) “Pollution” means the presence in the outdoor atmosphere or waters of the state of any substances, contaminants, noise, or man-made or man-induced alteration of the chemical, physical, biological or radiological integrity of air or water in quantities or levels which are or may be potentially harmful or injurious to human health or welfare, animal or plant life, or property, including outdoor recreation.

(14) “Reclaimed water” means the same as defined in Rule 62-600.200, F.A.C.

(15) “Secretary” means the Secretary of the Department.

(16) “Single Source Aquifer” means an aquifer or a portion of an aquifer which, pursuant to subsections 62-520.410(5) and (6), F.A.C., is determined by the Commission to be the only reasonably available source of potable water to a significant segment of the population.

(17) “Site” means the area within an installation’s property boundary where wastes or reclaimed water are released or applied to the ground water.

(18) “Surface Water” means water upon the surface of the earth, whether contained in bounds created naturally or artificially or diffused. Water from natural springs shall be classified as surface water when it exits from the spring onto the earth’s surface.

(19) “Unconfined Aquifer” means an aquifer which has a water table.

(20) “Wastes” means sewage, industrial wastes, and all other liquid, gaseous, solid, radioactive, or other substances which may pollute or tend to pollute any waters of the State.

(21) “Waters” include, but are not limited to, rivers, lakes, streams, springs, impoundments, and all other waters or bodies of water, including fresh, brackish, saline, tidal, surface or underground waters. Waters owned entirely by one person other than the state are included only in regard to possible discharge on other property or water. Underground waters include, but are not limited to, all underground waters passing through pores of rock or soils or flowing through in channels, whether manmade or natural.

(22) “Water Table” means the upper surface of a zone of saturation, where the body of ground water is not confined by an overlying impermeable zone.

(23) “Zone of Discharge” means a volume underlying or surrounding the site and extending to the base of a specifically designated aquifer or aquifers, within which an opportunity for the treatment, mixture or dispersion of wastes into receiving ground water is afforded.

(24) “Zone of Saturation” means a subsurface zone in which all of the interstices are filled with water.

Specific Authority 403.061 FS. Law Implemented 403.021, 403.031, 403.061 FS. History–New 9-8-92, Amended 4-14-94, Formerly 17-520.200.