62-560.610 Best Available Technology for Achieving Compliance with a Maximum Contaminant Level or Maximum Residual Disinfectant Level.

(1) The technologies listed in Table 1 are Best Available Technology for achieving compliance with the maximum contaminant levels for inorganic contaminants listed in subsection 62-550.310(1), F.A.C.

(2) Best Available Technology for achieving compliance with the maximum residual disinfectant levels listed in subsection 62-550.310(2), F.A.C., is control of treatment processes to reduce disinfectant demand or control of disinfection treatment processes to reduce disinfectant levels.

(3) The technologies listed in Table 2 are Best Available Technology for achieving compliance with the maximum contaminant levels for disinfection byproducts listed in subsection 62-550.310(3), F.A.C.

(4) The technologies listed in Table 3 are Best Available Technology for achieving compliance with the maximum contaminant levels for organic contaminants listed in subsection 62-550.310(4), F.A.C.

(5) Best Available Technology for achieving compliance with the maximum contaminant level for microbiological contaminants listed in subsection 62-550.310(5), F.A.C., includes the following:

(a) Protection of wells from contamination by coliforms by appropriate placement and construction;

(b) Maintenance of a disinfectant residual throughout the distribution system;

(c) Proper maintenance of the distribution system including appropriate pipe replacement and repair procedures, main flushing programs, proper operation and maintenance of storage tanks and reservoirs and continual maintenance of positive water pressure in all parts of the distribution system;

(d) Filtration or disinfection of surface water or ground water under the direct influence of surface water, as described in Part VI of Chapter 62-555, F.A.C., or disinfection of ground water not under the direct influence of surface water using strong oxidants such as chlorine, chlorine dioxide, or ozone; and

(e) The development and implementation of an Environmental Protection Agency-approved State Wellhead Protection Program under Section 1428 of the Federal Safe Drinking Water Act.

(6) The technologies listed in Table 4 are Best Available Technology for achieving compliance with the maximum contaminant levels for radionuclides listed in subsection 62-550.310(6), F.A.C.

Specific Authority 403.861(9) FS. Law Implemented 403.0877, 403.853 FS. History–New 11-19-87, Formerly 17-22.510, Amended 1-18-89, 1-3-91, 1-1-93, 1-26-93, Formerly 17-560.610, Amended 11-27-01, 4-14-03.