62-625.880 Tables.

(1) Selected industrial subcategories considered dilute for purposes of the combined waste stream formula.

(a) Auto and Other Laundries:

Carpet and Upholstery Cleaning,

Coin-Operated Laundries and Dry Cleaning,

Diaper Services,

Dry Cleaning Plants except Rug Cleaning,

Industrial Laundries,

Laundry and Garment Services, Not Elsewhere Classified,

Linen Supply,

Power Laundries, Family and Commercial.

(b) Electrical and Electronic Components:

Capacitors (Fluid Fill),

Carbon and Graphite Products,

Dry Transformers,

Ferrite Electronic Devices,

Fixed Capacitors,

Fluorescent Lamps,

Fuel Cells,

Incandescent Lamps,

Magnetic Coatings,

Mica Paper Dielectric,

Motors, Generators, Alternators,

Receiving and Transmitting Tubes,

Resistance Heaters,

Resistors,

Switchgear,

Transformer (Fluid Fill),

(c) Metal Molding and Casting:

Nickel Casting,

Tin Casting,

Titanium Casting.

(d) Gum and Wood Chemicals:

Char and Charcoal Briquettes.

(e) Inorganic Chemicals Manufacturing:

Ammonium Chloride,

Ammonium Hydroxide,

Barium Carbonate,

Calcium Carbonate,

Carbon Dioxide,

Carbon Monoxide and By-product Hydrogen,

Hydrochloric Acid,

Hydrogen Peroxide (Organic Process),

Nitric Acid,

Oxygen and Nitrogen,

Potassium Iodide,

Sodium Chloride (Brine Mining Process),

Sodium Hydrosulfide,

Sodium Hydrosulfite,

Sodium Metal,

Sodium Silicate,

Sodium Thiosulfate,

Sulfur Dioxide,

Sulfuric Acid,

(f) Leather:

Gloves,

Luggage,

(g) Paving and Roofing:

Asphalt Concrete,

Asphalt Emulsion,

Linoleum,

Printed Asphalt Felt,

Roofing.

(h) Pulp, Paper, and Paperboard, and Builders’ Paper and Board Mills:

Groundwood-Chemi-Mechanical.

(i) Rubber Manufacturing:

Tire and Inner Tube Plants,

Emulsion Crumb Rubber,

Solution Crumb Rubber,

Latex Rubber,

Small-sized General Molded, Extruded and Fabricated Rubber Plants,

Medium-sized General Molded, Extruded and Fabricated Rubber Plants,

Large-sized General Molded, Extruded and Fabricated Rubber Plants,

Wet Digestion Reclaimed Rubber,

Pan, Dry Digestion, and Mechanical Reclaimed Rubber,

Latex Dipped, Latex-Extruded, and Latex-Molded Rubber,

Latex Foam.

(j) Soap and Detergent Manufacturing:

Soap Manufacture by Batch Kettle,

Fatty Acid Manufacture by Fat Splitting,

Soap Manufacture by Fatty Acid,

Neutralization,

Glycerin Concentration,

Glycerin Distillation,

Manufacture of Soap Flakes and Powders,

Manufacture of Bar Soaps,

Manufacture of Liquid Soaps,

Manufacture of Spray Dried Detergents,

Manufacture of Liquid Detergents,

Manufacture of Dry Blended Detergents,

Manufacture of Drum Dried Detergents,

Manufacture of Detergent Bars and Cakes.

(k) Textile Mills:

Apparel Manufacturing,

Cordage and Twine,

Padding and Upholstery Filling.

(l) Timber Products Processing:

Barking Process,

Finishing Processes,

Hardboard-Dry Process.

(m) Footnotes.

The exemption for the manufacture of products in the Electrical and Electronic Components Category is for operations not covered by Electroplating/Metal Finishing pretreatment regulations.

Except for production attributed to lead-sheathed hose manufacturing operations.

Except for production attributed to chromic acid form-cleaning operations.

Except for production that generates zinc as a pollutant in discharge.

(2) Regulated pollutants eligible for a removal credit.

|  |  | Residuals Use or Disposal Practice |
| --- | --- | --- |
| Pollutant | Land Application (mg/Kg) | Surface Disposal (mg/Kg) |
|  | ____________________________________________ |  |  |
| Arsenic | 75 | 73 |
| Cadmium | – | 85 |
| Chromium | – | 600 |
| Copper | 4300 | – |
| Lead | 840 | – |
| Mercury | 57 | – |
| Molybdenum | 75 | – |
| Nickel | 420 | 420 |
| Selenium | 100 | – |
| Zinc | 7500 | – |

(3) The following organic pollutants are eligible for a removal credit if the requirements for total hydrocarbons in Chapter 62-296, F.A.C., are met when sewage sludge is fired in a sewage sludge incinerator: Acrylonitrile, Aldrin/Dieldrin (total), Benzene, Benzidine, Benzo(a)pyrene, Bis(2-chloroethyl)ether, Bis(2-ethylhexyl)phthalate, Bromodichloromethane, Bromoethane, Bromoform, Carbon tetrachloride, Chlordane, Chloroform, Chloromethane, DDD, DDE, DDT, Dibromochloromethane, Dibutyl phthalate, 1,2-dichloroethane, 1,1-dichloroethylene, 2,4-dichlorophenol, 1,3-dichloropropene, Diethyl phthalate, 2,4-dinitrophenol, 1,2-diphenylhydrazine, Di-n-butyl phthalate, Endosulfan, Endrin, Ethylbenzene, Heptachlor, Heptachlor epoxide, Hexachlorobutadiene, Alphahexachlorocyclohexane, Betahexachlorocyclohexane, Hexachlorocyclopentadiene, Hexachloroethane, Hydrogen cyanide, Isophorone, Lindane, Methylene chloride, Nitrobenzene, N-Nitrosodimethylamine, N-Nitrosodi-n-propylamine, Pentachlorophenol, Phenol, Polychlorinated biphenyls, 2,3,7,8-tetrachlorodibenzo-p-dioxin, 1,1,2,2-tetrachloroethane, Tetrachloroethylene,

Toluene, Toxaphene, Trichloroethylene, 1,2, 4-Trichlorobenzene, 1,1,1-Trichloroethane, 1,1,2-Trichloroethane, and 2,4,6-Trichlorophenol.

(4) Additional pollutants eligible for a removal credit.

|  |  |  | Residuals Use or Disposal Practice Surface Disposal |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Pollutant | LA | UL | L | I |  |
|  | ________________________________________________________________________ |  |  |  |  |  |  |  |  |
| Arsenic | – | – | 100(1) | – |  |
| Aldrin/Dieldrin (Total) | 2.7 | – | – | – |  |
| Benzene | 16(1) | 140 | 3400 | – |  |
| Benzo(a)pyrene | 15 | 100(1) | 100(1) | – |  |
| Bis(2-ethylhexyl)phthalate | – | 100(1) | 100(1) |  | – |
| Cadmium | – | 100(1) | 100(1) | – |  |
| Chlordane | 86 | 100(1) | 100(1) | – |  |
| Chromium | (2) | – | 100(1) | – |  |
| Copper | – | 46(1) | 100(1) | 140 |  |
|  |  |  |  | 0 |  |
| DDD, DDE, DDT (Total) | 1.2 | 2000 | 2000 |  | – |
| 2,4-Dichlorophenoxy-aceti | – | 7 | 7 |  | – |
| Fluoride | 730 | – | – | – |  |
| Heptachlor | 7.4 | – | – | – |  |
| Hexachlorobenzene | 29 | – | – | – |  |
| Hexachlorobutadiene | 600 | – | – | – |  |
| Iron | 78(1) | – | – | – |  |
| Lead | – | 100(1) | 100(1) | – |  |
| Lindane | 84 | 28(1) | 28(1) | – |  |
| Malathion | – | 0.63 | 0.63 | – |  |
| Mercury | – | 100(1) | 100(1) | – |  |
| Molybdenum | – | 40 | 40 | – |  |
| Nickel | – | – | 100(1) | – |  |
| N-Nitrosodimethylamine | 2.1 | 0.088 | 0.088 |  | – |
| Pentachlorophenol | 30 | – | – | – |  |
| Phenol | – | 82 | 82 | – |  |
| Polychlorinated biphenyls | 4.6 | <50 | <50 |  | – |
| Selenium | – | 4.8 | 4.8 | 4.8 |  |
| Toxaphene | 10 | 26(1) | 26(1) | – |  |
| Trichloroethene | 10(1) | 9500 | 10(1) | – |  |
| Zinc | – | 4500 | 4500 | 450 |  |
|  |  |  |  | 0 |  |

(a) Table abbreviations.

1. “LA” means land application.

2. “SD” means surface disposal.

3. “UL” means a disposal site without a liner and leachate collection system.

4. “L” means a disposal site with a liner and leachate collection system.

5. “I” means incineration.

(b) Footnotes.

1. The (1) in the above table indicates that the units are grams per kilograms – dry weight basis. All other values have the units milligrams per kilogram – dry weight basis.

2. The (2) in the above table indicates the value is to be established on a case-by-case basis.

Specific Authority 403.061(7), (31), 403.0885 FS. Law Implemented 403.0885, 403.08851 FS. History–New 11-29-94, Amended 1-8-97.