62-640.860 Other Solids.

(1) General Criteria. The disposal or use of other solids as defined in subsection 62-640.200(24), F.A.C., shall be authorized in a Department permit and addressed in the Agricultural Use Plan for the application site.

(a) The beneficial use of other solids which meets the criteria of subsection 62-640.860(2), F.A.C., shall be authorized in a wastewater permit.

(b) The disposal of other solids which does not meet the criteria of subsection 62-640.860(2), F.A.C., shall be in accordance with the design and operational criteria of Chapter 62-701, F.A.C.

(2) Beneficial Use of Other Solids. Other solids may be used in a way which is beneficial to the land if all of the following conditions are met.

(a) The permittee shall demonstrate how use of the other solids will be beneficial to the land, such as use as a soil amendment.

(b) The other solids shall be monitored and analyzed in accordance with Rule 62-640.650, F.A.C.

(c) The other solids shall meet the pathogen and vector attraction reduction requirements, and the site use restrictions of Rule 62-640.600, F.A.C. The Department shall review and approve the design and operational parameters of the treatment method used to reduce pathogens and vector attraction during application for a wastewater permit.

(d) The land application of other solids shall meet all of the criteria provided in Rule 62-640.700, F.A.C., for land application of residuals.

(e) The application rate of other solids to land shall be justified by the permittee in the permit application. The permittee shall demonstrate that the application rate is beneficial to the land. The application rate shall not be determined exclusively by the nitrogen content of the other solids, except that the application rate shall not exceed the agronomic needs of the site vegetation.

(3) Other solids which are combined with residuals prior to final treatment of the residuals are subject to all of the requirements of this chapter that apply to residuals.

Specific Authority 403.051, 403.061, 403.062, 403.087, 403.088, 403.704, 403.707 FS. Law Implemented 403.021, 403.051, 403.061, 403.087, 403.088, 403.0881, 403.702, 403.704, 403.707, 403.708 FS. History–New 3-30-98.