62-701.200 Definitions.

The following words, phrases or terms as used in Chapters 62-701 through 62-722, F.A.C., unless the context indicates otherwise, shall have the following meaning:

(1) “Agricultural wastes” means the solid wastes resulting from normal farming operations, the raising and slaughtering of animals, and the processing of animal products, orchard, and field crops, which are stored, transported, or disposed of as an unwanted waste material.

(2) “Airport” means any area of land or water, or any manmade object or facility located thereon, which is used, or intended for use, for the landing and takeoff of aircraft, and any appurtenant areas which are used, or intended for use, for airport buildings or other airport facilities or rights-of-way, together with all airport buildings and facilities located thereon.

(3) “Air quality standards” means, unless otherwise specified, those standards set forth in Chapter 62-204, F.A.C.

(4) “ASTM” means the American Society for Testing and Materials.

(5) “Aquifer” means a geologic formation, group of formations, or part of a formation capable of yielding a significant amount of ground water to wells, springs, or surface water.

(6) “Ash residue” means all the solid residue and any entrained liquids resulting from the combustion of solid waste in a solid waste combustor, including bottom ash, fly ash and combined bottom and fly ash, but excluding recovered metals, glass, and other recovered materials separated from the ash residue.

(a) “Bottom ash” means the solid material remaining after combustion of solid waste, which is discharged from the grates or stoker of a solid waste combustor.

(b) “Fly ash” means the residue from the combustion of solid waste, which is entrained in the gas stream of a solid waste combustor. Fly ash includes particulates, cinders, soot, and solid waste from air pollution control equipment.

(7) “Backyard composting” means the composting of organic solid waste, such as grass clippings, leaves or food waste, generated by a homeowner or tenant of a single or multi-family residential unit or an apartment complex unit, where composting occurs at that dwelling unit.

(8) “Biomedical waste” has the meaning given it in Chapter 64E-16, F.A.C.

(9) “Biological waste” means solid waste that causes or has the capability of causing disease or infection and includes biomedical waste, animals which died from disease, and other wastes capable of transmitting pathogens to humans or animals. The term does not include human remains that are disposed of by persons licensed under Chapter 470, F.S.

(10) “Bird hazard” means an increase in the likelihood of bird/aircraft collisions that may cause damage to the aircraft or injury to its occupants.

(11) “Bulky wastes” means items whose large size or weight precludes or complicates their handling by normal collection, processing, or disposal methods.

(12) “Cell” means a volume of solid waste received since the last previous application of initial cover. The compacted waste and subsequent initial cover constitute a cell which usually contains wastes deposited in one day.

(13) “Class I waste” means solid waste which is not hazardous waste, and which is not prohibited from disposal in a lined landfill under Rule 62-701.300, F.A.C.

(14) “Class III waste” means yard trash, construction and demolition debris, processed tires, asbestos, carpet, cardboard, paper, glass, plastic, furniture other than appliances, or other materials approved by the Department that are not expected to produce leachate which poses a threat to public health or the environment.

(15) “Clean debris” means any solid waste which is virtually inert, which is not a pollution threat to ground water or surface waters, is not a fire hazard, and is likely to retain its physical and chemical structure under expected conditions of disposal or use. The term includes brick, glass, ceramics, and uncontaminated concrete including embedded pipe or steel.

(16) “Clean wood” means wood, including lumber, tree and shrub trunks, branches, and limbs, which is free of paint, glue, filler, penthachlorophenol, creosote, tar, asphalt, other wood preservatives or treatments.

(17) “Closing” means the time at which a solid waste management facility ceases to accept wastes, and includes those actions taken by the owner or operator of the facility to prepare the facility for any necessary monitoring and maintenance after closing.

(18) “Closure” means the cessation of operation of a solid waste management facility and the act of securing such a facility so that it will pose no significant threat to human health or the environment. This includes closing, long term monitoring, maintenance, and financial responsibility.

(19) “Cm/sec” means centimeters per second.

(20) “Co-disposal” means the disposal of two or more different types of waste in the same solid waste disposal unit.

(21) “Combustion” means the treatment of solid waste in a device that uses heat as the primary means to change the chemical, physical, or biological character or composition of the waste. Combustion processes include incineration and pyrolysis.

(22) “Commercial solid waste” means all types of solid waste generated by stores, offices, restaurants, warehouses, and other nonmanufacturing activities, excluding residential and industrial wastes.

(23) “Compost” means solid waste which has undergone biological decomposition of organic matter, and has been disinfected using composting or similar technologies, and has been stabilized to a degree which is potentially beneficial to plant growth and which is used or sold for use as a soil amendment, artificial top soil, growing medium amendment or other similar uses.

(24) “Composting” means the process by which biological decomposition of organic solid waste is carried out under controlled aerobic conditions, and which stabilizes the organic fraction into a material which can easily and safely be stored, handled and used in an environmentally acceptable manner. The presence of anaerobic zones within the composting material will not cause the process to be classified as other than composting.

(25) “Composting facility” means a solid waste management facility where solid waste is processed using composting technology. Processing may include physical turning, windrowing, aeration or other mechanical handling of organic matter.

(26) “Composite liner” means a liner comprised of a geomembrane, which is underlain and in direct contact with a soil component.

(27) “Construction and demolition debris” means discarded materials generally considered to be not water soluble and non-hazardous in nature, including but not limited to steel, glass, brick, concrete, asphalt material, pipe, gypsum wallboard, and lumber, from the construction or destruction of a structure as part of a construction or demolition project or from the renovation of a structure, including such debris from construction of structures at a site remote from the construction or demolition project site. The term includes rocks, soils, tree remains, trees, and other vegetative matter which normally results from land clearing or land development operations for a construction project; clean cardboard, paper, plastic, wood, and metal scraps from a construction project; effective January 1, 1997, except as provided in Section 403.707(12)(j), F.S., unpainted, non-treated wood scraps from facilities manufacturing materials used for construction of structures or their components and unpainted, non-treated wood pallets provided the wood scraps and pallets are separated from other solid waste where generated and the generator of such wood scraps or pallets implements reasonable practices of the generating industry to minimize the commingling of wood scraps or pallets with other solid waste; and de minimis amounts of other non-hazardous wastes that are generated at construction or demolition projects, provided such amounts are consistent with best management practices of the construction and demolition industries. Mixing of construction and demolition debris with other types of solid waste will cause it to be classified as other than construction and demolition debris.

(28) “Contaminated soil” has the meaning given it in subsection 62-713.200(3), F.A.C.

(29) “Curing area” means an area where organic material that has undergone the rapid initial stage of composting is further stabilized into a humus-like material.

(30) “Degradable waste” means waste that decomposes through chemical breakdown or microbiological activity. It includes materials such as food and vegetative wastes, but does not include materials like concrete, ash residue from the combustion of solid wastes and metals.

(31) “Department” means the State of Florida Department of Environmental Protection.

(32) “Design period” means the operating life of the solid waste management facility plus any long-term care period after closing.

(33) “Disease vectors” means any rodents, flies, mosquitoes, or other animals, including insects, capable of transmitting disease to humans.

(34) “Disinfection” means, as relates to composting, the selective destruction of pathogens indicated by a reduction in indicator organisms to less than or equal to 100 fecal coliform most probable number per gram of volatile suspended solids where the organic solid waste was maintained at or above 55 degrees Celsius for three consecutive days in a mechanical composter or in an aerated, insulated static pile, or for 15 consecutive days in an aerated windrow with at least one turning or a non-aerated windrow with at least four turnings of the windrow.

(35) “Disposal” means the discharge, deposit, injection, dumping, spilling, leaking, or placing of any solid waste into or upon any land or water so that such solid waste or any constituent thereof may enter other lands or be emitted into the air or discharged into any waters, including ground waters, or otherwise enter the environment.

(36) “Engineer of record” means a professional engineer registered in the State of Florida in accordance with provisions of Chapter 471, F.S., who is appointed by the owner or operator of the solid waste management facility.

(37) “Facility” means all contiguous land and structures, other appurtenances, and improvements on the land used for solid waste management.

(38) “Fill” means man-made deposits of earth or waste materials used to fill excavations, to increase the vertical or horizontal extent of land or solid waste disposal units, or to build embankments.

(39) “Final cover” means the materials used to cover the top and sides of a landfill when fill operations cease.

(40) “Foreign matter” means the inorganic and organic constituents in a solid waste stream that are not readily decomposed and that may be present in the compost. For purposes of Chapter 62-709, F.A.C., foreign matter is metals, glass, plastics, rubber, bones, and leather, but does not include sand, grit, rocks or other similar materials.

(41) “Garbage” means all kitchen and table food waste, and animal or vegetative waste that is attendant with or results from the storage, preparation, cooking, or handling of food materials.

(42) “Gas condensate” means the liquid generated as a result of gas recovery processes at a landfill.

(43) “Gas recovery facility” means a system of wells, trenches, pipes, and other related ancillary structures such as manholes, compressors, and monitoring installations that collect and transport the gas produced in a waste disposal unit to one or more gas processing points or flares. The flow of gas through such a system may be produced by naturally occurring gas pressure gradients or may be aided by an induced draft generated by mechanical means.

(44) “Generation” means the act or process of producing solid waste.

(45) “Geocomposite” means a product composed of two or more materials, at least one of which is a geosynthetic.

(46) “Geomembrane” means a low-permeability synthetic membrane used as an integral part of a system designed to limit the movement of liquid or gas in the system.

(47) “Geogrid” means a geosynthetic formed by a regular network of integrally connected elements with apertures greater than 6.35 mm (1/4 inch) to allow interlocking with surrounding soil, rock, earth and other surrounding materials to function primarily as reinforcement.

(48) “Geonet” means a geosynthetic consisting of integrally connected parallel sets of ribs overlying similar sets at various angles for planar drainage of liquids or gases.

(49) “Geosynthetic” means a planar product manufactured from polymeric material used with soil, rock, earth, or other geotechnical engineering-related material as an integral part of a man-made project, structure or system.

(50) “Geosynthetic clay liner” means a low-permeability manufactured material consisting of a layer of low permeability clay placed between two geotextiles.

(51) “Geotextile” means a permeable textile used as a part of a system designed to act as a filter to prevent the flow of fine particles into drainage systems, to provide planar flow for drainage, to serve as a cushion to protect geomembranes, or to provide structural support.

(52) “GRI” means Geosynthetic Research Institute.

(53) “Ground water” means water beneath the surface of the ground within a zone of saturation, whether or not it is flowing through known and definite channels.

(54) “Hazardous waste” means a solid waste regulated by the Department as a hazardous waste pursuant to Chapter 62-730, F.A.C.

(55) “Household waste” means any solid waste, including garbage, trash, and sanitary waste in septic tanks, derived from households, including single and multiple residences, hotels and motels, bunkhouses, ranger stations, crew quarters, campgrounds, picnic grounds, and day-use recreation areas.

(56) “Indoor” means within a structure which excludes rain and public access and would control air flows in the event of a fire.

(57) “Industrial byproducts” means those materials which have a demonstrated recycling potential, can be feasibly recycled, and have been diverted or removed from the solid waste stream for sale, use, or reuse. The term does not include any materials which are defined as recovered materials. Industrial byproducts are regulated as solid waste unless otherwise exempted under paragraph 62-701.220(2)(d), F.A.C.

(58) “Industrial solid waste” means solid waste generated by manufacturing or industrial processes that is not a hazardous waste. Such waste may include, but is not limited to, waste resulting from the following manufacturing processes: electric power generation; fertilizer/agricultural chemicals; food and related products or byproducts; inorganic chemicals; iron and steel manufacturing; leather and leather products; nonferrous metals manufacturing or foundries; organic chemicals; plastics and resins manufacturing; pulp and paper industry; rubber and miscellaneous plastic products; stone, glass, clay, and concrete products; textile manufacturing; transportation equipment; and water treatment. This term does not include mining waste or oil and gas waste.

(59) “Initial cover” means a 6-inch layer of compacted earth, used to cover an area of solid waste before placement of additional waste, intermediate cover, or final cover. The term also includes other material or thickness, approved by the Department, that minimizes disease vector breeding, animal attraction, and moisture infiltration, minimizes fire potential, prevents blowing litter, controls odors, and improves landfill appearance.

(60) “Integral to” means, as regards the examination and certification of resource recovery equipment, that the machinery or equipment provides a significant function in the resource recovery or recycling process, such that the resource recovery or recycling process could not proceed without that piece of machinery or equipment.

(61) “Intermediate cover” means a layer of compacted earth at least one foot in depth applied to a solid waste disposal unit. The term also includes other material or thickness, approved by the Department, that minimizes disease vectors, odors, and fire, and is consistent with the leachate control design of the landfill.

(62) “Land clearing debris” means rocks, soils, tree remains, trees, and other vegetative matter which normally results from land clearing or land development operations for a construction project. Land clearing debris does not include vegetative matter from lawn maintenance, commercial or residential landscape maintenance, right-of-way or easement maintenance, farming operations, nursery operations, or any other sources not related directly to a construction project.

(63) “Land reclamation” means the restoration of productivity to lands made barren through processes such as erosion, mining or land clearing.

(64) “Landfill” means a solid waste disposal facility, which is an area of land or an excavation where wastes are or have been placed for disposal, for which a permit, other than a general permit, is required by Section 403.707, F.S. This term shall not include:

(a) A land spreading site;

(b) A surface impoundment;

(c) An injection well defined under and subject to the provisions of Chapter 62-528, F.A.C.; or

(d) A construction and demolition debris disposal site regulated by Rule 62-701.730, F.A.C.

(65) “Lateral expansion” means any horizontal increase in the dimensions of the waste boundary of an existing solid waste disposal unit.

(66) “Leachate” means liquid that has passed through or emerged from solid waste and may contain soluble, suspended or miscible materials.

(67) “Lead-acid battery” means those lead-acid batteries designed for use in motor vehicles, vessels, and aircraft, and includes such batteries when sold as a component part of a motor vehicle, vessel, or aircraft, but not when sold to recycle components.

(68) “Lift” means a completed horizontal series of cells.

(69) “Lined landfill” means a landfill constructed with a liner made of synthetic materials, low-permeability soils, or a combination of these materials, which has been permitted by the Department, and which met the Department’s landfill design criteria specified in this chapter or previous versions of this chapter at the time of permitting.

(70) “Liner” means a continuous layer of low-permeability natural or synthetic materials, under the bottom and sides of a landfill, solid waste disposal unit, or leachate surface impoundment, which controls the downward or lateral escape of waste constituents, or leachate.

(71) “Liner system” means a system of leachate collection and liner layers comprised of natural or synthetic materials installed between the subgrade and the waste for the purpose of containing the waste and collecting and removing leachate.

(72) “Liquid waste” means any waste material that is determined to contain free liquids as defined by Method 9095 (Paint Filter Liquids Test), as described in “Test Methods for Evaluating Solid Wastes, Physical/Chemical Methods” (EPA Pub. No. SW-846).

(73) “Local government” means any municipality, county, district, or authority, or any agency thereof, or a combination of two or more of the foregoing when acting jointly in connection with a project, which has jurisdiction over the collection, recycling, disposal or treatment of solid waste.

(74) “Lower explosive limit” means the lowest percent by volume of a mixture of explosive gases which will propagate a flame in air at a temperature of 25 degrees Celsius and atmospheric pressure.

(75) “Manure” means a solid waste composed of excreta of animals, and residual materials that have been used for bedding, sanitary or feeding purposes for such animals. For purposes of Chapter 62-709, F.A.C., manure does not mean such material generated and managed by normal farming operations.

(76) “Materials recovery” means any process by which one or more of the various components in solid waste is separated and concentrated for reuse.

(77) “Materials recovery facility” means a solid waste management facility that provides for the extraction from solid waste of recyclable materials, materials suitable for use as a fuel or soil amendment, or any combination of such materials.

(78) “Mesophilic stage” means a biological stage in the composting process characterized by active bacteria which favor a moderate temperature range of 20-45 degrees Celsius. It occurs later in a composting process after the thermophilic stage and is associated with a moderate rate of decomposition.

(79) “Method detection limit” means the smallest concentration of an analyte of interest that can be measured and reported with 99 percent confidence that the concentration is greater than zero. The method detection limit shall be determined pursuant to procedures outlined in Chapter 62-160, F.A.C., which is hereby incorporated by reference.

(80) “Monitoring wells” means strategically located wells from which water samples are drawn for water quality analysis.

(81) “Monofill” means a waste pile, landfill or solid waste disposal unit into which only one type of solid waste is placed.

(82) “Motor vehicle” means an automobile, motorcycle, truck, trailer, semitrailer, truck tractor and semitrailer combination, or any other vehicle operated in this state, used to transport persons or property, and propelled by power other than muscular power, but the term does not include traction engines, road rollers, such vehicles as run only upon a track, bicycles, moped, or farm tractors and trailers.

(83) “Normal farming operations” means the customary and generally accepted activities, practices, and procedures that farmers adopt, use, or engage in during the production and preparation for market of poultry, livestock, and associated farm products; and in the production, harvesting, or packaging of agricultural crops which include agronomic, horticultural, and silvicultural crops. Included are the management, collection, storage, composting, transportation, and utilization of organic agricultural waste, manure, and materials solely deprived from agricultural crops.

(84) “Objectionable odors” has the meaning given that term in Rule 62-210.200, F.A.C.

(85) “Oily wastes” means those materials which are mixed with used oil and have become separated from that used oil. Oily wastes also means materials, including wastewaters, centrifuge solids, filter residues or sludges, bottom sediments, tank bottoms, and sorbents which have come into contact with, and have been contaminated by, used oil.

(86) “100-year floodplain” means the lowland and relatively flat areas adjoining inland and coastal waters, including flood-prone areas of offshore islands, that are inundated by the 100-year flood.

(87) “On-site” means on the same or geographically contiguous property, which may be divided by a public or private right-of-way. It does not include two or more parcels of land more than a mile apart which are connected only by a public or private right-of-way.

(88) “Open burning” means the burning of any material under such conditions that the products of combustion are emitted directly into the atmosphere.

(89) “Operator” means any person, including the owner, who is principally engaged in, and is in charge of, the actual operation, supervision, and maintenance of a solid waste management facility.

(90) “Person” means any and all persons, natural or artificial, including any individual, firm, or association; any municipal or private corporation organized or existing under the laws of Florida or any other state; any county of this state; and any governmental agency of this state or the Federal Government.

(91) “PGI” means PVC Geomembrane Institute.

(92) “Population” means the most recent population census determination under Section 186.901, F.S.

(93) “Potable water well” means any excavation that is drilled or bored, or converted from non-potable water use, when the intended use of such excavation is for the location and acquisition of ground water which supplies water for human consumption.

(94) “Processed tire” means a tire that has been treated mechanically, chemically, or thermally so that the resulting material is a marketable product or is suitable for proper disposal.

(95) “Processing” means any technique designed to change the physical, chemical, or biological character or composition of any solid waste so as to render it safe for transport, amenable to recovery, storage or recycling; safe for disposal; or reduced in volume or concentration. As regards used oil, the term means chemical or physical operations designed to produce from used oil, or to make used oil more amenable for production of, fuel oils, lubricants, or other used oil-derived products. Processing includes blending used oil with virgin petroleum products, blending used oils to meet the fuel specifications, filtration, simple distillation, chemical or physical separation and rerefining.

(96) “Professional engineer” means an engineer registered in the State of Florida in accordance with Chapter 471, F.S.

(97) “Professional geologist” means a geologist registered in the State of Florida in accordance with Chapter 492, F.S.

(98) “Public used oil collection center” means:

(a) An automotive service facility or government-sponsored collection facility which accepts for disposal small quantities of used oil from households; or

(b) A facility which stores used oil in aboveground tanks, and which accepts small quantities of used oil from households.

(99) “Putrescible waste” means solid waste which contains organic matter capable of being decomposed by microorganisms and of such a character and proportion as to be capable of attracting or providing food for birds. The term does not include uncontaminated yard trash or clean wood.

(100) “PVC” means polyvinyl chloride.

(101) “Quantity of tires” means either weight, volume, or actual number of tires. For purposes of Chapter 62-711, F.A.C., assume that, for passenger tires, there are 100 tires per ton and 10 tires per cubic yard and that, for truck tires, there are 20 tires per ton.

(102) “Recovered materials” means metal, paper, glass, plastic, textile, or rubber materials that have known recycling potential, can be feasibly recycled, and have been diverted and source separated or have been removed from the solid waste stream for sale, use, or reuse as raw materials, whether or not the materials require subsequent processing or separation from each other, but does not include materials destined for any use that constitutes disposal. Recovered materials as described above are not solid waste.

(103) “Recovered materials processing facility” means a facility engaged solely in the storage, processing, resale, or reuse of recovered materials. Such a facility is not a solid waste management facility if it meets the conditions of paragraph 62-701.220(2)(c), F.A.C.

(104) “Recyclable material” means those materials which are capable of being recycled and which would otherwise be processed or disposed of as solid waste.

(105) “Recycling” means any process by which solid waste, or materials which would otherwise become solid waste, are collected, separated, or processed and reused or returned to use in the form of raw materials or products.

(106) “Recycling equipment” means machinery or equipment exclusively and integrally used in the actual process by which solid waste or materials which would otherwise become solid waste are separated or processed and reused or returned to use in the form of raw materials or products.

(107) “Resource recovery” means the process of recovering materials or energy from solid waste, excluding those materials or solid waste under control of the Nuclear Regulatory Commission.

(108) “Resource recovery equipment” means equipment or machinery exclusively and integrally used in the actual process of recovering material or energy resources from solid waste. This definition specifically includes recycling equipment.

(109) “Sanitary nuisance” means a condition created by any person, or the keeping, maintaining, propagation, existence, or permitting of anything by a person by which the health or lives of individuals may be threatened or impaired, or by which disease may be caused or transmitted.

(110) “Shredding” means a process of reducing the particle size of solid waste through the use of grinding, shredding, milling, or rasping machines.

(111) “Site” means the area of land or water within the property boundaries of a solid waste management facility where one or more solid waste processing, resource recovery, recycling, storage, or disposal areas are located.

(112) “Sludge” means a solid waste pollution control residual which is generated by any industrial or domestic wastewater treatment plant, water supply treatment plant, air pollution control facility, septic tank, grease trap, portable toilet or related operation, or any other such waste having similar characteristics. Sludge may be a solid, liquid, or semisolid waste but does not include the treated effluent from a wastewater treatment plant.

(113) “Solid waste” means: sludge unregulated under the federal Clean Water Act or Clean Air Act; sludge from a waste treatment works, water supply treatment plant, or air pollution control facility; or garbage, rubbish, refuse, special waste, or other discarded material, including solid, liquid, semi-solid, or contained gaseous material resulting from domestic, industrial, commercial, mining, agricultural, or governmental operations. Materials not regulated as solid waste pursuant to this chapter are: recovered materials; nuclear source or byproduct materials regulated under Chapter 404, F.S., or under the Federal Atomic Energy Act of 1954 as amended; suspended or dissolved materials in domestic sewage effluent or irrigation return flows, or other regulated point source discharges; regulated air emissions; and fluids or wastes associated with natural gas or crude oil exploration or production.

(114) “Solid waste combustor” means an enclosed device that uses controlled combustion, the primary purpose of which is to thermally break down solid, liquid, or gaseous combustible solid wastes to an ash residue that contains little or no combustible material.

(115) “Solid waste disposal facility” means any solid waste management facility which is the final resting place for solid waste, including landfills and incineration facilities that produce ash from the process of incinerating municipal solid waste.

(116) “Solid waste disposal unit” means a discrete area of land used for the disposal of solid waste.

(117) “Solid waste management” means the process by which solid waste is collected, transported, stored, separated, processed, or disposed of in any other way, according to an orderly, purposeful, and planned program which includes closure and long-term maintenance.

(118) “Solid waste management facility” means any solid waste disposal area, volume reduction plant, transfer station, materials recovery facility, or other facility, the purpose of which is resource recovery or the disposal, recycling, processing, or storage of solid waste. The term does not include recovered materials processing facilities which meet the requirements of paragraph 62-701.220(2)(c), F.A.C., except the portion of such facilities, if any, that is used for the management of solid waste.

(119) “Special wastes” means solid wastes that can require special handling and management, including but not limited to, white goods, waste tires, used oil, lead-acid batteries, construction and demolition debris, ash residue, yard trash, biological wastes, and mercury-containing devices and lamps.

(120) “Stabilized” means that biological and chemical decomposition of the wastes has ceased or diminished to a level so that such decomposition no longer poses a pollution, health, or safety hazard. As regards composting, the term means that the compost has at least passed through the thermophilic stage, and that biological decomposition of the solid waste has occurred to a sufficient degree that will allow beneficial use.

(121) “Subgrade” means soils native to or imported to a site, or other materials authorized by a Department permit or this chapter, which may be graded and compacted before a landfill liner system is constructed over them.

(122) “Thermophilic stage” means a biological stage in the composting process characterized by active bacteria which favor a high temperature range of 45-75 degrees Celsius. It occurs early in a composting process before the mesophilic stage and is associated with a high rate of decomposition.

(123) “Tire” means a continuous solid or pneumatic rubber covering encircling the wheel of a motor vehicle.

(124) “Tire disposal” means to deposit, dump, spill or place any waste tire or residuals into or upon any structure, land or water. Tire disposal also includes the burning of any waste tire or residuals in a waste-to-energy facility, incinerator, or other facility used solely for the disposal of solid waste.

(125) “Tire recycling” means any process by which waste tires or residuals are reused or returned to use in the form of products or raw materials.

(126) “Ton” means a short ton, 2000 pounds (.9078 metric tons).

(127) “Transfer station” means a facility the primary purpose of which is to store or hold solid waste for transport to a processing or disposal facility.

(128) “Unlined landfill” means a landfill which does not have a bottom liner system which was approved by the Department as part of a construction permit, or which has a bottom liner system which does not or did not meet the Department’s landfill design criteria specified in this chapter or in previous versions of this chapter at the time of construction. This term includes landfills underlain by only in-situ soils unless these soils have been tested and approved by the Department as part of a construction permit.

(129) “Used oil” means any oil which has been refined from crude oil or synthetic oil and, as a result of use, storage, or handling, has become contaminated and unsuitable for its original purpose due to the presence of physical or chemical impurities or loss of original properties.

(130) “Used tire” means a whole waste tire which has a minimum tread depth of 3/32 inch or greater and is suitable for use on a motor vehicle.

(131) “Uppermost aquifer” means the geologic formation nearest the natural ground surface that is an aquifer, as well as lower aquifers that are hydraulically interconnected with this aquifer within a facility’s property boundary.

(132) “Vector” means, as regards compost, a carrier organism that is capable of transmitting a pathogen from one organism to another.

(133) “Volume reduction plant” means an incinerator, pulverizer, compactor, shredding and baling plant, composting plant, or other plant which accepts and processes solid waste for recycling or disposal.

(134) “Waste tire” means a tire that has been removed from a motor vehicle and has not been retreaded or regrooved. “Waste tire” includes used tires and processed tires.

(135) “Waste tire collection center” means a site where waste tires are collected from the public before being offered for recycling or disposal and where fewer than 1,500 tires are kept on-site on any given day.

(136) “Waste tire collector” means a person who transports more than 25 waste tires over public highways at any one time.

(137) “Waste tire processing facility” means a site where equipment is used to recapture reusable byproducts from waste tires or to cut, burn, or otherwise alter waste tires so that they are no longer whole. The term includes mobile waste tire processing equipment.

(138) “Waste tire residuals” means any liquids, sludges, metals, fabric or byproducts resulting from the processing or storage of tires. Residuals do not include processed tires held for recycling or disposal, provided the conditions of Rule 62-711.530, F.A.C., are met.

(139) “Waste tire site” means a site at which 1,500 or more waste tires are accumulated. For purposes of this term a site means a piece of property owned, rented, or otherwise controlled by a person, including all contiguous or adjacent properties owned, rented, or otherwise controlled by that person.

(140) “Water quality standards and criteria” means, unless otherwise specified, those standards and criteria set forth in Chapters 62-302 and 62-520, F.A.C.

(141) “White goods” means inoperative and discarded refrigerators, ranges, washers, water heaters, freezers, and other similar domestic and commercial large appliances.

(142) “Working face” means that portion of a landfill where waste is deposited, spread, and compacted before placement of initial cover.

(143) “Yard trash” means vegetative matter resulting from landscaping maintenance or land clearing operations and includes materials such as tree and shrub trimmings, grass clippings, palm fronds, trees and tree stumps.

(144) “Zone of discharge” has the meaning given it in Rule 62-520.200, F.A.C.

All other definitions found in Chapter 403, F.S., and Chapters 62-702 through 62-722, F.A.C., to the extent that they are consistent with the definitions of this chapter, are applicable to the terms used in this chapter.

Specific Authority 403.704 FS. Law Implemented 403.702-.717, 403.75-.769 FS. History–Formerly 10D-12.02, 10-1-74, Revised 7-20-76, Amended 5-24-79, 6-13-84, 4-25-85, 7-1-85, 12-10-85, Formerly 17-7.02, 17-7.020, Amended 8-2-89, 6-25-90, Formerly 17-701.020, Amended 1-6-93, 1-2-94, 5-19-94, Formerly 17-701.200, Amended 12-23-96, 5-27-01.