62-761.300 Applicability.

(1) General Requirements:

(a) Underground storage tank systems: The requirements of this Chapter, unless specified otherwise, apply to owners and operators of facilities, or owners and operators of UST systems with individual storage tank capacities greater than 110 gallons, that contain or contained:

1. Vehicular fuel, subject to Chapter 17-61, F.A.C., after May 21, 1984;

2. Pollutants or hazardous substances after December 10, 1990; or

3. Regulated substances in unmaintained storage tank systems.

(b) This rule is applicable to non-residential facilities. Under 40 C.F.R. 280, residential tanks greater than 1100 gallons containing motor fuels are subject to federal UST rules (advisory information only-not required by this Chapter).

(2) Exemptions: The following underground systems are exempt from the requirements of this Chapter:

(a) Any storage tank system storing any hazardous waste listed or identified under Subtitle C of the Resource Conservation and Recovery Act, or a mixture of such hazardous waste and other regulated substances;

(b) Any storage tank system regulated under the Toxic Substances Control Act (15 U.S.C. 2065);

(c) Any pesticide waste degradation system regulated under Chapter 62-660, F.A.C.;

(d) Storage tank systems used solely for temporary storage of mixtures of pesticides and dilutent for reapplication as pesticides;

(e) Any storage tank system with a storage capacity of less than 30,000 gallons used for the sole purpose of storing heating oil for consumptive use on the premises where stored;

(f) Any tank that contains asphalt or asphalt products not containing other regulated substances;

(g) Any storage tank system storing regulated substances that are solid or gaseous at standard temperature and pressure;

(h) Any storage tank containing LP gas;

(i) Any storage tank system that contains small quantities (de minimus, as per 40 C.F.R. Section 280.10(b)(5)) of regulated substances;

(j) Any wastewater treatment tank system that is part of a wastewater treatment facility regulated under Section 402 or 307(b) of the Clean Water Act;

(k) Any septic tank system;

(l) Any stormwater or wastewater collection system;

(m) Any surface impoundment, pit, pond, or lagoon;

(n) Any agricultural storage tank system of 550 gallons capacity or less;

(o) Any residential storage tank system;

(p) Any emergency spill or emergency overflow containment storage tank system that is emptied as soon as possible after use, and that routinely remains empty;

(q) Any flow-through process tank system. For industrial and manufacturing facilities, integral piping is considered to terminate at the forwarding pump or valve used to transfer regulated substances to process, production, or manufacturing points of use or systems within the facility;

(r) Any storage tank system, liquid trap, or associated gathering lines directly related to oil or gas production and gathering operations regulated by Chapter 377, F.S.;

(s) Equipment or machinery that contains regulated substances for operational purposes, such as hydraulic lift or fluid tank systems and electrical equipment tank systems;

(t) Any pipeline facilities;

(u) Any storage tank system containing radionuclides or that is part of an emergency generator system for nuclear power generation at facilities regulated by the Nuclear Regulatory Commission under 10 C.F.R. Part 50 Appendix A;

(v) Vapor recovery holding tanks and associated vapor recovery piping systems; or

(w) Any rail or tanker truck loading or unloading operations (loading racks) specified in Chapter 5 of NFPA 30.

Specific Authority 376.303 FS. Law Implemented 376.303 FS. History–New 12-10-90, Formerly 17-761.300, Amended 7-13-98, 6-21-04.