62-762.301 Applicability.

(1) General Requirements:

(a) Aboveground storage tank systems: The requirements of this chapter, unless specified otherwise, apply to owners and operators of facilities, or owners and operators of aboveground stationary storage tank systems with individual storage tank capacities greater than 550 gallons, that contain or contained:

1. Vehicular fuel, subject to Chapter 17-61, F.A.C., after May 21, 1984;

2. Pollutants after March 12, 1991; or

3. Pollutants in unmaintained storage tank systems.

(b) Aboveground compression vessels and hazardous substance storage tank systems: Owners and operators of compression vessels and hazardous substance storage tanks with capacities of greater than 110 gallons containing hazardous substances are only required to comply with subsections 62-762.401(1)-(2), F.A.C.

(c) Aboveground mineral acid storage tank systems: Owners and operators of facilities, or owners and operators of aboveground mineral acid storage tank systems with capacities of greater than 110 gallons containing mineral acids are only required to comply with Rule 62-762.891, F.A.C.

(d) This rule is applicable to non-residential facilities.

(2) Exemptions: The following aboveground systems are exempt from the requirements of this chapter:

(a) Any storage tank system storing any hazardous waste listed or identified under Subtitle C of the Resource Conservation and Recovery Act, or a mixture of such hazardous waste and other regulated substances;

(b) Any storage tank system regulated under the Toxic Substances Control Act (15 U.S.C. 2065);

(c) Any pesticide waste degradation system regulated under Chapter 62-660, F.A.C.;

(d) Storage tank systems used solely for temporary storage of mixtures of pesticides and dilutent for reapplication as pesticides;

(e) Any storage tank system with a storage capacity of less than 30,000 gallons used for the sole purpose of storing heating oil for consumptive use on the premises where stored;

(f) Any tank that contains asphalt or asphalt products not containing other regulated substances;

(g) Any storage tank system storing regulated substances that are solid or gaseous at standard temperature and pressure;

(h) Any storage tank containing LP gas;

(i) Any storage tank system that contains small quantities (de minimus, as per 40 C.F.R. Section 280.10(b)(5)) of regulated substances;

(j) Any wastewater treatment tank system that is part of a wastewater treatment facility regulated under Section 402 or 307(b) of the Clean Water Act;

(k) Any septic tank system;

(l) Any stormwater or wastewater collection system;

(m) Any surface impoundment, pit, pond, or lagoon;

(n) Any agricultural storage tank system of 550 gallons capacity or less;

(o) Any residential storage tank system;

(p) Any emergency spill or emergency overflow containment storage tank system that is emptied as soon as possible after use, and that routinely remains empty;

(q) Any flow-through process tank system. For industrial and manufacturing facilities, integral piping is considered to terminate at the forwarding pump or valve used to transfer regulated substances to process, production, or manufacturing points of use or systems within the facility;

(r) Any storage tank system, liquid trap, or associated gathering lines directly related to oil or gas production and gathering operations regulated by Chapter 377, F.S.;

(s) Equipment or machinery that contains regulated substances for operational purposes, such as hydraulic lift or fluid tank systems and electrical equipment tank systems;

(t) Any pipeline facilities;

(u) Any storage tank system containing radionuclides or that is part of an emergency generator system for nuclear power generation at facilities regulated by the Nuclear Regulatory Commission under 10 C.F.R. Part 50 Appendix A;

(v) Vapor recovery holding tanks and associated vapor recovery piping systems; or

(w) Any rail or tanker truck loading or unloading operations (loading racks) specified in Chapter 5 of NFPA 30;

(x) Drip irrigation systems that:

1. Are not in contact with the soil;

2. Are constructed of corrosion resistant materials;

3. Are compatible with the products stored;

4. Contain less than 80% concentration of fertilizer materials by volume; and

5. Are applied on site.

(y) Systems used exclusively for the storage of aqueous solutions of sodium hypochlorite;

(z) Any mobile tank;

(aa) Any system located entirely within an industrial occupancy building;

(bb) Any storage tank system located entirely within an enclosed building or vault with an adequate roof and walls to prevent rainwater from reaching the system, and with an impervious floor containing no valves, drains, or other openings that would permit pollutants to be discharged from the system that were constructed before July 13, 1998; or

(cc) Any mobile double-wall tank, regardless of how long it is located at a facility, that is connected with a power module system that is used for the emergency or supplemental generation of electrical power by an electric utility as defined in Chapter 366, F.S. This exemption is limited to tanks that are designed and constructed to be moved to other service locations, and the relocation within a facility or from site to site is inherent in its use.

Specific Authority 376.303 FS. Law Implemented 376.303 FS. History–New 6-21-04.