- 62B-55.007 Model Standards For Existing Beachfront Lighting.

In order to provide the highest level of protection for nesting marine turtles and their hatchlings, local governments should adopt all of the following standards for existing artificial beachfront lighting sources:

(1) Existing artificial light fixtures shall be repositioned, modified, or removed so that:

(a) The point source of light or any reflective surface of the light fixture is not directly visible from the beach;

(b) Areas seaward of the frontal dune are not directly or indirectly illuminated; and,

(c) Areas seaward of the frontal dune are not cumulatively illuminated.

(2) The following measures shall be taken to reduce or eliminate the negative effects of existing exterior artificial lighting:

(a) Reposition fixtures so that the point source of light or any reflective surface of the light fixture is no longer visible from the beach;

(b) Replace fixtures having an exposed light source with fixtures containing recessed light sources or shields;

(c) Replace traditional light bulbs with yellow “bug” type bulbs not exceeding 50 watts;

(d) Replace non-directional fixtures with directional fixtures that point down and away from the beach;

(e) Replace fixtures having transparent or translucent coverings with fixtures having opaque shields covering an arc of at least 180 degrees and extending an appropriate distance below the bottom edge of the fixture on the seaward side so that the light source or any reflective surface of the light fixture is not visible from the beach;

(f) Replace pole lamps with low-profile, low-level luminaries so that the light source or any reflective surface of the light fixture is not visible from the beach;

(g) Replace incandescent, fluorescent, and high intensity lighting with the lowest wattage low pressure sodium vapor lighting possible for the specific application;

(h) Plant or improve vegetation buffers between the light source and the beach to screen light from the beach;

(i) Construct a ground level barrier to shield light sources from the beach. Ground-level barriers must not interfere with marine turtle nesting or hatchling emergence, or cause short- or long- term damage to the beach/dune system;

(j) Permanently remove or permanently disable any fixture which cannot be brought into compliance with the provisions of these standards.

(3) The following measures shall be taken to reduce or eliminate the negative effects of interior light emanating from doors and windows within line-of-sight of the beach:

(a) Apply window tint or film that meets the standards for tinted glass;

(b) Rearrange lamps and other moveable fixtures away from windows;

(c) Use window treatments (e.g., blinds, curtains) to shield interior lights from the beach; and,

(d) Turn off unnecessary lights.

Rulemaking Authority 161.63 FS. Law Implemented 161.163 FS. History–New 3-30-93, Formerly 16B-55.007.