Notice of Change/Withdrawal

DEPARTMENT OF ENVIRONMENTAL PROTECTION

RULE NOS.:		RULE TITLES:

62-610.300	General Technical Guidance, Related Rules, Technical Publications and Forms

62-610.330	Pretreatment Programs

62-610.463	Monitoring and Operating Protocol

62-610.466	Aquifer Storage and Recovery (ASR)

62-610.472	Supplemental Water Supplies

62-610.525	Projects Involving Additional Levels of Preapplication Treatment

62-610.568	Monitoring and Operating Protocol

62-610.652	Waste Treatment, Disinfection, and Monitoring

62-610.800	Permitting Requirements

62-610.865	Blending of Demineralization Concentrate with Reclaimed Water

62-610.870	Reporting and Enforcement

62-610.890	General Permit for Adding New Major Users to a Part III Reuse System

NOTICE OF CHANGE

Notice is hereby given that the following changes have been made to the proposed rule in accordance with Section 120.54(3)(d)1., F.S., published in Vol. 46 No. 242, December 15, 2020, and Vol. 47 No. 95, May 17, 2021, issues of the Florida Administrative Register. The Department is revising the Notice of Change publication to address the Joint Administrative Procedures Committee comment letter.

62-610.300 General Technical Guidance, Related Rules, Technical Publications and Forms.

(1) The technical standards and criteria contained in the following standard manuals and technical publications listed below are hereby incorporated by reference and shall be applied, if applicable, in determining whether permits allowing construction, modification or implementation of reuse and land application projects shall be issued or denied. A copy of each publication may be obtained by contacting the Department’s Wastewater Management Program, Mail Station 3545, 2600 Blair Stone Road, Tallahassee, Florida 32399-2400.

(a) U.S. Environmental Protection Agency, 1974. Design Criteria for Mechanical, Electric, and Fluid System and Component Reliability-MCD-05. Environmental Quality Instructional Resources Center, The Ohio State University, 200 Chambers Road, Room 310, Columbus, Ohio 43212, incorporated by reference herein http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX.

(b) U.S. Environmental Protection Agency, 1977. Procedures Manual for Groundwater Monitoring at Solid Waste Disposal Facilities. National Technical Information Service, 5285 Port Royal Road, Springfield, Virginia 22161, incorporated by reference herein  http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX.

(c) U.S. Department of Agriculture, Soil Conservation Service, 1973. Drainage of Agricultural Land. Water Information Center, Inc., 125 East Bethpage Road, Plainview, New York 11803, incorporated by reference herein http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX.

(d) Florida Department of Transportation, 1985. Florida Land Use, Cover and Forms Classification System. Procedure No. 550-010-001-A. Florida Department of Transportation, Maps and Publications Sales, Mail Station 12, 605 Suwannee Street, Tallahassee, Florida 32399-0450, incorporated by reference herein http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX.

(e) Florida Department of Environmental Regulation, 1991. Guidelines for Preparation of Reuse Feasibility Studies for Applicants Having Responsibility for Wastewater Management. Information Center, 2600 Blair Stone Road, Tallahassee, Florida 32399-2400, incorporated by reference herein http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX.

(2) No change.

(3) Forms.

(a) The forms and instructions used by the Department are listed in this rule. The rule numbers are the same as the form numbers. Copies of these forms and instructions may be obtained by writing to the Wastewater Management Program, Mail Station 3545, Department of Environmental Protection, Bob Martinez Center, 2600 Blair Stone Road, Tallahassee, Florida 32399-2400. In addition, these forms are available at the Department’s District Offices and from the web site for the Department’s Division of Water Resource Management at https://floridadep.gov/water. The Department adopts and incorporates by reference in this section the following forms:

1. through 4. No change.

(b) No change.

(c) Subsection 62-600.660(2), F.A.C., requires domestic wastewater treatment facilities with a permitted capacity of 100,000 gpd or greater that discharge to ground waters via reuse and land application systems to annually monitor reclaimed water or effluent for the primary and secondary drinking water standards contained in Chapter 62-550, F.A.C. Except for asbestos, total coliforms, color, odor and residual disinfectants, all parameters listed as primary and secondary drinking water standards in Chapter 62-550, F.A.C., shall be monitored and reported on the Discharge Monitoring Report (DMR), DEP Form 62-620.910(10), (adopted and incorporated by reference in Rule 62-620.910(10), F.A.C., effective November 29, 1994). The DMR forms shall be electronically submitted to the Department by June 28 using the DEP Business Portal at http://www.fldepportal.com/go/. Approved analytical methods identified in paragraph 62-620.100(3)(j), F.A.C., shall be used for analysis. If no method is included for a parameter, methods specified in Chapter 62-550, F.A.C., shall be used.

Rulemaking Authority 403.051, 403.061, 403.064 FS. Law Implemented 403.051, 403.061(7), (13), 403.064, 403.085, 403.086, 403.087, 403.088 FS. History–New 4-4-89, Amended 4-2-90, Formerly 17-610.300, Amended 1-9-96, 8-8-99, 3-9-06, 11-19-07,         .         .

- 62-610.330 Pretreatment Programs.

- (1) No change.

- (2) A pretreatment program shall not be required for Part III or V reuse projects, if the applicant indicates on Application Form 2A, Permit for Domestic Wastewater Treatment and Reuse or Disposal Facility (adopted and incorporated by reference in Rule 62-620.910(2), F.A.C., effective June 1, 2001), Department Form 62-620.910(2) that the wastewater facility has no significant industrial users, as defined in Rule 62-625.200, F.A.C., and that no significant industrial users are anticipated during the next five-year permit period. To continue the pretreatment program exemption, subsequent permit renewal applications must demonstrate that the wastewater facility continues to have no significant industrial users and none are anticipated during the next five-year permit period.

- (3) No change.

Rulemaking Authority 403.051, 403.061, 403.087 FS. Law Implemented 403.021, 403.051, 403.061, 403.062, 403.085, 403.086, 403.087, 403.088 FS. History–New 4-4-89, Amended 4-2-90, Formerly 17-610.330, Amended 1-9-96, 8-8-99,         .

- 62-610.463 Monitoring and Operating Protocol.

- (1) through (3) No change.

- (4) Monitoring for Giardia and Cryptosporidium.

- (a) For treatment plants having capacities of 1.0 mgd or larger, the permittee shall sample the reclaimed water for Cryptosporidium and Giardia as follows:

- 1. Sampling shall be conducted at one time during each two-year period. Intervals between sampling shall not be greater than two years. The sample results shall be submitted to the Department on or before November 28 of each even numbered year using Form 62-610.300(3)(a)4., Pathogen Monitoring, (adopted and incorporated by reference in Rule 62-610.300(3)(a)4., F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX).

- 2. No change.

- (b) For treatment plants having capacities less than 1.0 mgd, the permittee shall sample the reclaimed water for Cryptosporidium and Giardia as follows:

1. Sampling shall be conducted at one time during each five-year period. Intervals between sampling shall not be greater than five years. The sample results shall be submitted to the Department on or before November 28 of that year using Form 62-610.300(3)(a)4., Pathogen Monitoring, (adopted and incorporated by reference in Rule 62-610.300(3)(a)4., F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX).

- 2. No change.

- (c) through (e) No change.

- (f) EPA Method 1623.1, 1623 (adopted and incorporated by reference in Rule 62-610.100(7), F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX), or other approved methods in accordance with Rule 62-160.330, F.A.C., for reclaimed water or nonpotable waters, adjusted appropriately to accommodate the detection limit requirements, shall be used.  Methods previously allowed for EPA’s Information Collection Rule (ICR) shall not be used.  The full requirements of the approved method, including quality assurance and quality control, are to be met.  Quality assurance and sampling requirements of Chapter 62-160, F.A.C., shall apply.

- Two concentrations of Giardia and Cryptosporidium shall be recorded on Part III of Form 62-610.300(3)(a)4., Pathogen Monitoring, (adopted and incorporated by reference in Rule 62-610.300(3)(a)4., F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX).

1. Total cysts and oocysts shall be enumerated using EPA Method 1623.1,1623 (adopted and incorporated by reference in Rule 62-610.100(7), effective [date]), or other approved appoved  methods in accordance with Rule 62-160.330, F.A.C.

- 2. Potentially viable cysts and oocysts shall be enumerated using the DAPI staining technique contained in EPA Method 1623.1, 1623 (adopted and incorporated by reference in Rule 62-610.100(7), effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX), or similar enumeration techniques included in other approved methods in accordance with Rule 62-160.330, F.A.C.  Cysts and oocysts that are stained DAPI positive or show internal structure by D.I.C. shall be considered as being potentially viable.  If the laboratory reports separate values for DAPI positive and for cysts or oocysts having internal structure, the larger of the two concentrations will be reported as being potentially viable.

- (g) If the number of potentially viable cysts of Giardia reported exceeds 5 per 100 liters or oocysts of Cryptosporidium reported exceeds 22 per 100 liters, a subsequent sample shall be taken and analyzed using EPA Method 1623.1, 1623 (adopted and incorporated by reference in Rule 62-610.100(7), effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX), or other approved methods in accordance with Rule 62-160.330, F.A.C.  This subsequent sample shall be collected within 90 days of the date the initial sample was taken, analyzed for both Giardia and Cryptosporidium, and the results of the subsequent analysis shall be submitted to the Department within 60 days of sample collection using Form 62-610.300(3)(a)4., Pathogen Monitoring, (adopted and incorporated by reference in Rule 62-610.300(3)(a)4., F.A.C., effective [date]).

Rulemaking Authority 403.051, 403.061, 403.087 FS. Law Implemented 403.021, 403.051, 403.061, 403.062, 403.085, 403.086, 403.087, 403.088 FS. History–New 4-4-89, Amended 4-2-90, Formerly 17-610.463, Amended 1-9-96, 8-8-99,          .

- 62-610.466 Aquifer Storage and Recovery (ASR).

- (1) through (8) No change.

- (9) Use of Class G-II ground water containing 3000 mg/L or less of total dissolved solids.

- (a) No change.

- (b) If the applicant provides an affirmative demonstration that the receiving ground water contains between 1,000 and 3,000 mg/L (inclusive) of total dissolved solids, is not currently used as a source of public water supply, and that the receiving ground water is not reasonably expected to be used for public water supply in the future, the preapplication treatment and disinfection requirements shall be as follows:

- 1. The principal treatment and disinfection requirements in Rule 62-610.563, F.A.C., shall apply, with the following modifications:

- a. through c. No change.

- d. The extended zone of discharge shall not extend into zones having TDS concentrations less than 1000 mg/L (based on the initial TDS characterization in the initial engineering report and subsequent information submitted after the submittal of the initial engineering report).

- (10) Use of Class G-II ground water containing greater than 3000 mg/L of total dissolved solids.

- (a) Wells may be used to inject reclaimed water into Class G-II ground water containing greater than 3000 mg/L of total dissolved solids for ASR if all of the following conditions are met:

- 1. The principal treatment and disinfection requirements in Rule 62-610.563, F.A.C., shall apply, with the following modifications:

- a. through c. No change.

d. The extended zone of discharge shall not extend into zones having TDS concentrations less than 3000 mg/L (based on the initial TDS characterization in the initial engineering report and subsequent information submitted after the submittal of the initial engineering report).

- 2. No change.

- (b) No change.

- (11) through (13) No change.

- (14) Extended zone of discharge.

- (a) through (d) No change.

- (e) The extended zone of discharge shall extend vertically from the base to the top of a specifically designated aquifer, aquifers, or portion of an aquifer. The vertical and lateral limits of the extended zone of discharge shall be designated. Injection and recovery wells used in the ASR system shall be included within the extended zone of discharge. As noted in sub-subparagraphs 62-610.466(9)(b)1.d., and 62-610.466(10)(a)1.d., F.A.C., the extended zone of discharge shall not extend into zones having TDS concentrations less than the specified threshold (based on the initial TDS characterization in the initial engineering report and subsequent information submitted after the submittal of the initial engineering report).

- (f) No change.

- (15) through (17) No change.

Rulemaking Authority 403.051, 403.061, 403.087 FS. Law Implemented 403.021, 403.051, 403.061, 403.062, 403.085, 403.086, 403.087, 403.088 FS. History–New 8-8-99,         .

- 62-610.472 Supplemental Water Supplies.

- (1) through (2) No change.

- (3) Surface water and stormwater supplies.

- (a) through (c) No change.

- (d) Monitoring for Giardia and Cryptosporidium.

- 1. For treatment plants having capacities of 1.0 mgd or larger, the permittee shall sample the reclaimed water for Cryptosporidium and Giardia as follows:

- a. Sampling shall be conducted at one time during each two-year period. Intervals between sampling shall not be greater than two years. The sample results shall be submitted to the Department on or before November 28 of each even numbered year using Form 62-610.300(3)(a)4., Pathogen Monitoring, (adopted and incorporated by reference in Rule 62-610.300(3)(a)4., F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX).

- b. No change.

- 2. For treatment plants having capacities less than 1.0 mgd, the permittee shall sample the reclaimed water for Cryptosporidium and Giardia as follows:

a. Sampling shall be conducted at one time during each five-year period. Intervals between sampling shall not be greater than five years. The sample results shall be submitted to the Department on or before November 28 of that year using Form 62-610.300(3)(a)4., Pathogen Monitoring, (adopted and incorporated by reference in Rule 62-610.300(3)(a)4., F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX).

- b. No change

- 3. through 5. No change.

- 6. EPA Method 1623.1, 1623 (adopted and incorporated by reference in Rule 62-610.100(7), F.A.C., effective [date]), or other approved methods in accordance with Rule 62-160.330, F.A.C., for reclaimed water or nonpotable waters, adjusted appropriately to accommodate the detection limit requirements, shall be used. Methods previously allowed for EPA’s ICR shall not be used. The full requirements of the approved method, including quality assurance assuance and quality control, are to be met. Quality assurance and sampling requirements in Chapter 62-160, F.A.C., shall apply.

Two concentrations of Giardia and Cryptosporidium shall be recorded in Part III of Form 62-610.300(3)(a)4., Pathogen Monitoring (adopted and incorporated by reference in Rule 62-610.300(3)(a)4., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX):

1. Total cysts and oocysts shall be enumerated using EPA Method 1623.1, 1623(adopted and incorporated by reference in Rule 62-610.100(7), effective [date], or other approved methods in accordance with Rule 62-160.330, F.A.C.

- 2. No change.

- 7. If the number of potentially viable cysts of Giardia reported exceeds 5 per 100 liters or oocysts of Cryptosporidium reported exceeds 22 per 100 liters, a subsequent sample shall be taken and analyzed using EPA Method 1623.1, 1623(adopted and incorporated by reference in Rule 62-610.100(7), F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX) or other approved methods in accordance with Rule 62-160.330, F.A.C.  This subsequent sample shall be collected within 90 days of the date the initial sample was taken, analyzed for both Giardia and Cryptosporidium, and the results of the subsequent analysis shall be submitted to the Department within 60 days of sample collection using Form 62-610.300(3)(a)4., Pathogen Monitoring, (adopted and incorporated by reference in Rule 62-610.300(3)(a)4., F.A.C., effective [date]).

- (4) through (7) No change.

Rulemaking Authority 403.051, 403.061, 403.087 FS. Law Implemented 403.021, 403.051, 403.061, 403.062, 403.085, 403.086, 403.087, 403.088 FS. History–New 8-8-99,        .

- 62-610.525 Projects Involving Additional Levels of Preapplication Treatment.

- (1) through (12) No change.

- (13) The permittee shall sample the reclaimed water for Cryptosporidium and Giardia. The following requirements shall be met:

(a) Sampling shall be conducted once every two years. Intervals between sampling shall not exceed two years. The sample results shall be submitted to the Department on or before November 20 of each even numbered year using Form 62-610.300(3)(a)4., Pathogen Monitoring, (adopted and incorporated by reference in Rule 62-610.300(3)(a)4., F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX).

- (b) through (e) No change.

- (f) EPA Method 1623.1, 1623(adopted and incorporated by reference in Rule 62-610.100(7), F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX), or other approved methods in accordance with Rule 62-160.330, F.A.C., for reclaimed water or nonpotable waters, adjusted appropriately to accommodate the detection limit requirements, shall be used.  Methods previously allowed for EPA’s ICR shall not be used. The full requirements of the approved method, including quality assurance and quality control, are to be met.  Quality assurance and sampling requirements of Chapter 62-160, F.A.C., shall apply.

- Two concentrations of Giardia and Cryptosporidium shall be recorded on Part III of Form 62-610.300(3)(a)4., Pathogen Monitoring, (adopted and incorporated by reference in Rule 62-610.300(3)(a)4., F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX):

1. Total cysts and oocysts shall be enumerated using EPA Method 1623.1, 1623(adopted and incorporated by reference in Rule 62-610.100(7), F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX), or other approved appoved methods in accordance with Rule 62-160.330, F.A.C.

- 2. Potentially viable cysts and oocysts shall be enumerated using the DAPI staining technique contained in EPA Method 1623.1, 1623(adopted and incorporated by reference in Rule 62-610.100(7), F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX), or similar enumeration techniques included in other approved methods.  Cysts and oocysts that are stained DAPI positive or show internal structure by D.I.C. shall be considered as being potentially viable.  If the laboratory reports separate values for DAPI positive and for cysts or oocysts having internal structure, the larger of the two concentrations will be reported as being potentially viable.

- (g) If the number of potentially viable cysts of Giardia reported exceeds 5 per 100 liters or oocysts of Cryptosporidium reported exceeds 22 per 100 liters, a subsequent sample shall be taken and analyzed using EPA Method 1623.1, 1623 (adopted and incorporated by reference in Rule 62-610.100(7), F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX), or other approved methods in accordance with Rule 62-160.330, F.A.C.  This subsequent sample shall be collected within 90 days of the date the initial sample was taken, analyzed for both Giardia and Cryptosporidium, and the results of the subsequent analysis shall be submitted to the Department within 60 days of sample collection using Form 62-610.300(3)(a)(4)., Pathogen Monitoring, (adopted and incorporated by reference in Rule 62-610.300(3)(a)4., F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX).

- Rulemaking Authority 403.051, 403.061, 403.087 FS. Law Implemented 403.021, 403.051, 403.061, 403.062, 403.085, 403.086, 403.087, 403.088 FS. History–New 4-4-89, Amended 4-2-90, Formerly 17-610.525, Amended 1-9-96, 8-8-99,        .

- 62-610.568 Monitoring and Operating Protocol.

- (1) through (9) No change.

- (10) Except as noted in subsection 62-610.568(11), F.A.C., the permittee shall sample the reclaimed water for Cryptosporidium and Giardia. The following requirements shall be met:

- (a) Sampling shall be conducted quarterly. Intervals between sampling shall not exceed three months. The sample results shall be submitted to the Department on or before February 28, May 28, August 28, and November 28 of each year using Form 62-610.300(3)(a)4., Pathogen Monitoring, (adopted and incorporated by reference in Rule 62-610.300(3)(a)4., F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX).

- (b) through (f) No change.

(g) EPA Method 1623.1, 1623 (adopted and incorporated by reference in Rule 62-610.100(7), F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX), or other approved methods in accordance with Rule 62-160.330, F.A.C., for reclaimed water or nonpotable waters, adjusted appropriately to accommodate the detection limit requirements, shall be used.  Methods previously allowed for EPA’s ICR shall not be used. The full requirements of the approved method, including quality assurance and quality control, are to be met.  Quality assurance and sampling requirements in Chapter 62-160, F.A.C., shall apply.

Two concentrations of Giardia and Cryptosporidium shall be recorded on Part III of Form 62-610.300(3)(a)4., Pathogen Monitoring, (adopted and incorporated by reference in Rule 62-610.300(3)(a)4., F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX):

1. Total cysts and oocysts shall be enumerated using EPA Method 1623.1, 1623(adopted and incorporated by reference in Rule 62-610.100(7), effective [date]), or other approved methods in accordance with Rule 62-160.330, F.A.C.

- 2. Potentially viable cysts and oocysts shall be enumerated using the DAPI staining technique contained in EPA Method 1623.1, 1623 (adopted and incorporated by reference in Rule 62-610.100(7), F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX), or similar enumeration techniques included in other approved methods in accordance with Rule 62-160.330, F.A.C.  Cysts and oocysts that are stained DAPI positive or show internal structure by D.I.C. shall be considered as being potentially viable.  If the laboratory reports separate values for DAPI positive and for cysts or oocysts having internal structure, the larger of the two concentrations will be reported as being potentially viable.

- (h) If the number of potentially viable cysts of Giardia reported exceeds 5 per 100 liters or oocysts of Cryptosporidium reported exceeds 22 per 100 liters, a subsequent sample shall be taken and analyzed using EPA Method 1623.1, 1623 (adopted and incorporated by reference in Rule 62-610.100(7), F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX), or other approved methods in accordance with Rule 62-160.330, F.A.C.  This subsequent sample shall be collected within 90 days of the date the initial sample was taken, analyzed for both Giardia and Cryptosporidium, and the results of the subsequent sebsequent analysis shall be submitted to the Department within 60 days of sample collection using Form 62-610.300(3)(a)4., Pathogen Monitoring, (adopted and incorporated by reference in Rule 62-610.300(3)(a)4., F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX).

- (11) For projects involving discharge to waters upstream of Class I surface waters, as described in subsection 62-610.555(2), F.A.C., the permittee shall sample the reclaimed water for Cryptosporidium and Giardia. The following requirements shall be met:

- (a) Sampling shall be conducted at one time during each two-year period. Intervals between sampling shall not exceed two years. The sample results shall be submitted to the Department on or before November 28 of each even numbered year using Form 62-610.300(3)(a)4., Pathogen Monitoring, (adopted and incorporated by reference in Rule 62-610.300(3)(a)4., F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX).

- (b) through (e) No change.

- (f) EPA Method 1623.1, 1623 (adopted and incorporated by reference in Rule 62-610.100(7), F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX), or other approved methods in accordance with Rule 62-160.330, F.A.C., for reclaimed water or nonpotable waters, adjusted appropriately to accommodate the detection limit requirements, shall be used.  Methods previously allowed for EPA’s ICR shall not be used. The full requirements of the approved method, including quality assurance and quality control, are to be met.  Quality assurance and sampling requirements in Chapter 62-160, F.A.C., shall apply.

Two concentrations of Giardia and Cryptosporidium shall be recorded on Part III of Form 62-610.300(3)(a)4., Pathogen Monitoring, (adopted and incorporated by reference in Rule 62-610.300(3)(a)4., F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX):

1. Total cysts and oocysts shall be enumerated using EPA Method 1623.1, 1623 (adopted and incorporated by reference in Rule 62-610.100(7), effective [date]) or other approved methods in accordance with Rule 62-160.330, F.A.C.

- 2. Potentially viable cysts and oocysts shall be enumerated using the DAPI staining technique contained in EPA Method 1623.1, 1623 (adopted and incorporated by reference in Rule 62-610.100(7), F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX), or similar enumeration techniques included in other approved methods in accordance with Rule 62-160.330, F.A.C.  Cysts and oocysts that are stained DAPI positive or show internal structure by D.I.C. shall be considered as being potentially viable.  If the laboratory reports separate values for DAPI positive and for cysts or oocysts having internal structure, the larger of the two concentrations will be reported as being potentially viable.

(g) If the number of potentially viable cysts of Giardia reported exceeds 5 per 100 liters or oocysts of Cryptosporidium reported exceeds 22 per 100 liters, a subsequent sample shall be taken and analyzed using EPA Method 1623.1, 1623 (adopted and incorporated by reference in Rule 62-610.100(7), F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX), or other approved methods in accordance with Rule 62-160.330, F.A.C. This subsequent sample shall be collected within 90 days of the date the initial sample was taken, analyzed for both Giardia and Cryptosporidium, and the results of the subsequent analysis shall be submitted to the Department within 60 days of the sample collections using usint Form 62-610.300(3)(a)4., Pathogen Monitoring, (adopted and incorporated by reference in Rule 62-610.300(3)(a)4., F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX).

Rulemaking Authority 403.051, 403.061, 403.064, 403.087 FS. Law Implemented 403.021, 403.051, 403.061, 403.062, 403.064, 403.085, 403.086, 403.087, 403.088 FS. History–New 4-4-89, Amended 4-2-90, Formerly 17-610.568, Amended 1-9-96, 8-8-99, 11-19-07,         .

- 62-610.652 Waste Treatment, Disinfection, and Monitoring.

- (1) through (5) No change.

- (6) Monitoring.

- (a) through (b) No change.

- (c) For facilities that provide reclaimed water for use in open cooling towers, the permittee shall sample the reclaimed water for Cryptosporidium and Giardia. The following requirements shall be met:

1. Sampling shall be conducted at one time during each two-year period. Intervals between sampling shall not exceed two years. This sampling frequency is independent of the treatment plant’s capacity. The sample results shall be submitted to the Department on or before November 28 of each even numbered year using Form 62-610.300(3)(a)4., Pathogen Monitoring, (adopted and incorporated by reference in Rule 62-610.300(3)(a)4., F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX).

- 2. through 5. No change.

- 6. EPA Method 1623.1, 1623 (adopted and incorporated by reference in Rule 62-610.100(7), F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX), or other approved methods in accordance with Rule 62-160.330, F.A.C., for reclaimed water or nonpotable waters, adjusted appropriately to accommodate the detection limit requirements, shall be used.  Methods previously allowed for EPA’s ICR shall not be used. The full requirements of the approved method, including quality assurance and quality control, are to be met.  Quality assurance and sampling requirements in Chapter 62-160, F.A.C., shall apply.

Two concentrations of Giardia and Cryptosporidium shall be recorded on Part III of Form 62-610.300(3)(a)4., Pathogen Monitoring, (adopted and incorporated by reference in Rule 62-610.300(3)(a)4., F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX):

a. Total cysts and oocysts shall be enumerated using EPA Method 1623.1, 1623(adopted and incorporated by reference in Rule 62-610.100(7), F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX), or other approved methods in accordance with Rule 62-160.330, F.A.C.

- b. Potentially viable cysts and oocysts shall be enumerated using the DAPI staining technique contained in EPA Method 1623.1, 1623 (adopted and incorporated by reference in Rule 62-610.100(7), F.A.C., effective [date]), or similar enumeration techniques included in other approved methods in accordance with Rule 62-160.330, F.A.C.  Cysts and oocysts that are stained DAPI positive or show internal structure by D.I.C. shall be considered as being potentially viable.  If the laboratory reports separate values for DAPI positive and for cysts or oocysts having internal structure, the larger of the two concentrations will be reported as being potentially viable.

- 7. If the number of potentially viable cysts of Giardia reported exceeds 5 per 100 liters or oocysts of Cryptosporidium reported exceeds 22 per 100 liters, a subsequent sample shall be taken and analyzed using EPA Method 1623.1, 1623 (adopted and incorporated by reference in Rule 62-610.100(7), F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX), or other approved methods in accordance with Rule 62-160.330, F.A.C. This subsequent sample shall be collected within 90 days of the date the initial sample was taken, analyzed for both Giardia and Cryptosporidium, and the results of the subsequent analysis shall be submitted to the department within 60 days of sample collection using Form 62-610.300(3)(a)4., Pathogen Monitoring, (adopted and incorporated by reference in Rule 62-610.300(3)(a)4., F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX).

Rulemaking Authority 403.051, 403.061, 403.087 FS. Law Implemented 403.021, 403.051, 403.061, 403.062, 403.085, 403.086, 403.087, 403.088 FS. History–New 1-9-96, Amended 8-8-99,        .

- 62-610.800 Permitting Requirements.

- (1) through (6) No change.

- (7) Placing a Facility in Operation.

- (a) The permittee shall obtain written approval from the Department before placing the initial part, portion, or phase of a reuse system permitted under Part III of this chapter into operation. Written application shall be made using Form 62-610.300(3)(a)3., Application for Permission to Place a Public Access Reuse System in Operation, (adopted and incorporated by reference in Rule 62-610.300(3)(a)3., F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX). Requirements for placing the reuse system into operation are contained in Rule 62-620.630, F.A.C. Approval shall be granted if the requirements in Rule 62-620.630, F.A.C., are met.

- (b) No change.

- (8) through (13) No change.

Rulemaking Authority 403.051, 403.061, 403.064, 403.087, 403.0881 FS. Law Implemented 403.021, 403.051, 403.061, 403.064, 403.087, 403.088, 403.0881 FS. History–New 4-2-90, Formerly 17-610.800, Amended 1-9-96, 8-8-99, 11-19-07,        .

- 62-610.865 Blending of Demineralization Concentrate with Reclaimed Water.

- (1) through (7) No change.

- (8) Monitoring.

- (a) through (e) No change.

- (f) An annual scan of the parameters listed as primary and secondary drinking water standards in Chapter 62-550, F.A.C., (except for asbestos, total coliforms, color, odor, and residual disinfectants), shall be for the reclaimed water and the blend.

- 1. Results of the scan shall be reported on Discharge Monitoring Report (DMR), DEP Form 62-620.910(10), (adopted and incorporated by reference in Rule 62-620.910(10), F.A.C., effective November 29, 1994). The DMR forms shall be electronically submitted to the Department by January 28 using the DEP Business Portal at http://www-fldepportal.com/go/.

- 2. through 3. No change.

- (9) through (13) No change.

Rulemaking Authority 403.051, 403.061, 403.087 FS. Law Implemented 403.021, 403.051, 403.061, 403.062, 403.085, 403.086, 403.087, 403.088 FS. History–New 8-8-99,         .

62-610.870 Reporting and Enforcement.

(1) through (2) No change.

(3) Annual Reuse Report.

(a) Permittees having responsibility for domestic wastewater treatment facilities (new and existing) having permitted capacities of at least 0.1 mgd that discharge all or part of their reclaimed water to reuse systems permitted under this chapter shall submit an annual report to the Department and the appropriate water management district. This reporting requirement also applies to all permittees responsible for operation of reuse authorities. Domestic wastewater permits issued by the Department identify portions of domestic wastewater projects that are categorized as “reuse.” The report shall be electronically submitted annually on or before January 1 of each year using the DEP Business Portal at http://www.fldepportal.com/go/ utilizing Form 62-610.300(3)(a)2., Annual Reuse Report, (adopted and incorporated by reference in Rule 62-610.300(3)(a)2., F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX).in accordance with Form 62-610.300(3)(a)2, F.A.C.

(b) through (f) No change.

Rulemaking Authority 403.051, 403.061, 403.064, 403.087, 403.0881 FS. Law Implemented 403.021, 403.051, 403.061, 403.062, 403.064, 403.085, 403.086, 403.087, 403.088, 403.0881 FS. History–New 4-2-90, Formerly 17-610.870, Amended 1-9-96, 8-8-99, 11-19-07,       .

- 62-610.890 General Permit for Adding New Major Users to a Part III Reuse System.

- (1) A general permit is hereby granted to a permittee whose permit includes a reuse system for the addition of a new major user of reclaimed water (using 0.1 mgd or more, as an annual average), provided that:

- (a) Notice to the Department under subsection 62-4.530(1), F.A.C., is submitted on Form 62-610.300(3)(a)1., Notice of Intent to Use General Permit for Addition of a Major User of Reclaimed Water, (adopted and incorporated by reference in Rule 62-610.300(3)(a)1., F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX); and,

- (b) through (f) No change.

- (2) The notice to the Department of the permittee’s intent to use this general permit requires certification by a professional engineer registered in the State of Florida and the applicant on Form 62-610.300(3)(a)1., Notice of Intent to Use General Permit for Addition of a Major User of Reclaimed Water, (adopted and incorporated by reference in Rule 62-610.300(3)(a)1., F.A.C., effective [date], http://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX).

Rulemaking Authority 403.814(1) FS. Law Implemented 403.061, 403.087, 403.088, 403.814 FS. History–New 4-2-90, Formerly 17-610.890, Amended 1-9-96, 8-8-99,        .