- 62-6.025 Definitions.

Definitions in Chapter 62-6, Parts I and II, F.A.C., are also applicable to Chapter 62-6, Part IV, F.A.C.

(1) Advanced Secondary Treatment Standards: A wastewater system with the following operational criteria:

(a) CBOD5 and TSS.

1. The arithmetic mean of the CBOD5 or TSS values for the effluent samples collected (whether grab or composite technique is used) during an annual period shall not exceed 10 mg/l.

2. The arithmetic mean of the CBOD5 or TSS values for a minimum of four effluent samples, each collected (whether grab or composite technique is used) on a separate day during a period of 90 consecutive days (quarterly) shall not exceed 12.5 mg/l.

3. The arithmetic mean of the CBOD5 or TSS values for a minimum of four effluent samples, each collected (whether grab or composite technique is used) on a separate day of seven consecutive days shall not exceed 15 mg/l.

4. Maximum-permissible concentrations of CBOD5 or TSS values in any effluent grab sample at any time shall not exceed 20 mg/l.

(b) TN.

1. The arithmetic mean of the TN values for the effluent samples collected (whether grab or composite technique is used) during an annual period shall not exceed 20 mg/l.

2. The arithmetic mean of the TN values for a minimum of four effluent samples, each collected (whether grab or composite technique is used) on a separate day during a period of 90 consecutive days (quarterly) shall not exceed 25 mg/l.

3. The arithmetic mean of the TN values for a minimum of four effluent samples, each collected (whether grab or composite technique is used) on a separate day of seven consecutive days shall not exceed 30 mg/l.

4. Maximum-permissible concentrations of TN values in any effluent grab sample at any time shall not exceed 40 mg/l.

(c) TP.

1. The arithmetic mean of the TP values for the effluent samples collected (whether grab or composite technique is used) during an annual period shall not exceed 10 mg/l.

2. The arithmetic mean of the TP values for a minimum of four effluent samples, each collected (whether grab or composite technique is used) on a separate day during a period of 90 consecutive days (quarterly) shall not exceed 12.5 mg/l.

3. The arithmetic mean of the TP values for a minimum of four effluent samples, each collected (whether grab or composite technique is used) on a separate day of seven consecutive days shall not exceed 15 mg/l.

4. Maximum-permissible concentrations of TP values in any effluent grab sample at any time shall not exceed 20 mg/l.

(d) Fecal coliform – system operation shall result in not more than 200 fecal coliform colonies per 100 ml of effluent sample. Where chlorine is used for disinfection, the design shall include provisions for rapid and uniform mixing and a total chlorine residual of at least 0.5 mg/l shall be maintained after at least 15 minutes contact time at the peak hourly flow. To determine compliance of a system, the following operational criteria (using either MF or MPN methods) shall be applicable.

1. The arithmetic mean of the fecal coliform colonies collected during the annual period shall not exceed 200 per 100 ml of effluent.

2. The median value of the fecal coliform colonies for a minimum number of 10 samples of effluent, each collected on a separate day during a period of 30 days (monthly) shall not exceed 200 per 100 ml of sample.

3. No more than 10% of the samples collected during the period of 30 consecutive days shall exceed 400 fecal coliform colonies per 100 ml of sample.

4. Any one sample shall not exceed 800 fecal coliform colonies per 100 ml of sample.

(2) Advanced Wastewater Treatment Standards: A wastewater system with the following operational criteria:

(a) CBOD5 and TSS.

1. The arithmetic mean of the CBOD5 or TSS values for the effluent samples collected (whether grab or composite technique is used) during an annual period shall not exceed 5 mg/l.

2. The arithmetic mean of the CBOD5 or TSS values for a minimum of four effluent samples, each collected (whether grab or composite technique is used) on a separate day during a period of 90 consecutive days (quarterly) shall not exceed 6.25 mg/l.

3. The arithmetic mean of the CBOD5 or TSS values for a minimum of four effluent samples, each collected (whether grab or composite technique is used) on a separate day of seven consecutive days shall not exceed 7.5 mg/l.

4. Maximum-permissible concentrations of CBOD5 or TSS values in any effluent grab sample at any time shall not exceed 10 mg/l.

(b) TN.

1. The arithmetic mean of the TN values for the effluent samples collected (whether grab or composite technique is used) during an annual period shall not exceed 3 mg/l.

2. The arithmetic mean of the TN values for a minimum of four effluent samples, each collected (whether grab or composite technique is used) on a separate day during a period of 90 consecutive days (quarterly) shall not exceed 3.75 mg/l.

3. The arithmetic mean of the TN values for a minimum of four effluent samples, each collected (whether grab or composite technique is used) on a separate day of seven consecutive days shall not exceed 4.5 mg/l.

4. Maximum-permissible concentrations of TN values in any effluent grab sample at any time shall not exceed 6 mg/l.

(c) TP.

1. The arithmetic mean of the TP values for the effluent samples collected (whether grab or composite technique is used) during an annual period shall not exceed 1 mg/l.

2. The arithmetic mean of the TP values for a minimum of four effluent samples, each collected (whether grab or composite technique is used) on a separate day during a period of 90 consecutive days (quarterly) shall not exceed 1.25 mg/l.

3. The arithmetic mean of the TP values for a minimum of four effluent samples, each collected (whether grab or composite technique is used) on a separate day of seven consecutive days shall not exceed 1.5 mg/l.

4. Maximum-permissible concentrations of TP values in any effluent grab sample at any time shall not exceed 2.0 mg/l.

(d) Fecal coliform – system operation shall result in an effluent in which fecal coliform colonies (per 100 ml of sample) are below detectable limits. Where chlorine is used for disinfection, the design shall include provisions for rapid and uniform mixing; and the total chlorine residual of at least 1.0 mg/l shall be maintained at all times. The minimum acceptable contact time shall be 15 minutes at the peak hourly flow. To determine compliance of a system, the following operational criteria (using either MF or MPN methods) shall be applicable.

1. Fecal coliform shall be below the detection limits for 75% of the samples collected over a 30 day period.

2. Any one sample shall not exceed 25 fecal coliform colonies per 100 ml of sample.

3. Any one sample shall not exceed 5.0 mg/l of TSS at a point before application of the disinfectant.

(3) Baseline system standards – A wastewater system with the following operational criteria:

(a) Effluent concentrations from the treatment tank:

1. CBOD5 – ≤240 mg/l.

2. TSS – ≤176 mg/l.

3. TN – ≤45 mg/l.

4. TP – ≤10 mg/l.

(b) Percolate concentrations from the baseline system prior to discharge to groundwater:

1. CBOD5 – ≤5 mg/l.

2. TSS – ≤5 mg/l.

3. TN – ≤25 mg/l.

4. TP – ≤5 mg/l.

(4) Bottom infiltrative surface – the vertical projection of the bottom surface of the drainfield that is no lower in elevation than 30 inches below grade.

(5) Composite sample – means a combination of individual samples of wastewater or effluent taken at selected intervals, generally hourly or less for some specified period, to minimize the effect of the variability of the individual sample.

(6) Grab sample – a sample which is taken from a waste stream without regard to the flow in the waste stream and over a period of time not to exceed fifteen minutes.

(7) Effective drainfield depth – the vertical distance from the bottom of the drainfield to the invert of the distribution pipe.

(8) Florida Keys nutrient reduction treatment – a treatment which will provide a recovered water product that contains not more, on a permitted annual average basis, than the following concentrations from a sampling point located following the final design treatment step of the onsite sewage treatment and disposal system:

| (a) Biochemical Oxygen Demand (CBOD5) | 10 mg/l |
| --- | --- |
| (b) Suspended Solids | 10 mg/l |
| (c) Total Nitrogen, expressed as N | 10 mg/l |
| (d) Total Phosphorus, expressed as P | 1 mg/l |

(9) Innovative System – as defined by Section 381.0065(2), F.S.

(10) Performance-based treatment system – a specialized onsite sewage treatment and disposal system designed by a professional engineer with a background in wastewater engineering, licensed in the state of Florida, using appropriate application of sound engineering principles to achieve specified levels of CBOD5 (carbonaceous biochemical oxygen demand), TSS (total suspended solids), TN (total nitrogen), TP (total phosphorus), and fecal coliform found in domestic sewage waste, to a specific and measurable established performance standard. This term also includes innovative systems.

(11) Performance System Maintenance Entity – any person or business entity which has been issued a written permit by the Department and has been authorized by the design engineer or manufacturer of all treatment components used in the performance-based treatment system and provides operation and maintenance services associated with performance-based treatment system.

(12) Secondary Treatment Standards: A wastewater system with the following operational criteria:

(a) CBOD5 and TSS.

1. The arithmetic mean of the CBOD5 or TSS values for the effluent samples collected (whether grab or composite technique is used) during an annual period shall not exceed 20 mg/l.

2. The arithmetic mean of the CBOD5 or TSS values for a minimum of four effluent samples, each collected (whether grab or composite technique is used) on a separate day during a period of 30 consecutive days (monthly) shall not exceed 30 mg/l.

3. The arithmetic mean of the CBOD5 or TSS values for a minimum of four effluent samples, each collected (whether grab or composite technique is used) on a separate day of seven consecutive days shall not exceed 45 mg/l.

4. Maximum-permissible concentrations of CBOD5 or TSS values in any effluent grab sample at any time shall not exceed 60 mg/l.

(b) Fecal coliform – system operation shall result in not more than 200 fecal coliform colonies per 100 ml of effluent sample. Where chlorine is used for disinfection, the design shall include provisions for rapid and uniform mixing and a total chlorine residual of at least 0.5 mg/l shall be maintained after at least 15 minutes contact time at the peak hourly flow. To determine compliance of a system, the following operational criteria (using either MF or equivalent MPN methods) are applicable.

1. The arithmetic mean of the fecal coliform colonies collected during the annual period shall not exceed 200 per 100 ml of effluent.

2. The geometric mean of the fecal coliform colonies for a minimum of 10 samples of effluent, each collected on a separate day, shall not exceed 200 per 100 ml of sample.

3. No more than 10% of the samples collected during a period of 30 consecutive days shall exceed 400 fecal coliform colonies per 100 ml of sample.

4. Any one sample shall not exceed 800 fecal coliform colonies per 100 ml of sample.

(13) Sidewall infiltrative surfaces – the horizontal projection of the drainfield measured from the invert of the drainfield distribution pipe to the bottom infiltrative surface, or to 30 inches below finished grade, whichever is less.

(14) Total drainfield depth – the vertical distance from the bottom of the drainfield to the top of the drainfield.

(15) Wastewater strength – the sum of the CBOD5 and TSS concentrations in the effluent.

Rulemaking Authority 381.0011(4), (13), 381.0065(3)(a) FS. Law Implemented 381.0065, 381.0067, 386.041 FS. History–New 2-3-98, Amended 3-22-00, 6-18-03, 11-26-06, Formerly 64E-6.025.