- 40B-8.121 Minimum Surface Water Levels for Lakes.

(1) The following terms, as used in this rule, shall have the following meanings:

(a) “Minimum Frequent High” means a chronically high surface water level or flow with an associated frequency and duration that allows for inundation of the floodplain at a depth and duration sufficient to maintain wetland functions.

(b) “Minimum Frequent Low” means a chronically low surface water level or flow that generally occurs only during periods of reduced rainfall. This level is intended to prevent deleterious effects to the composition and structure of floodplain soils, the species composition and structure of floodplain and instream biotic communities, and the linkage of aquatic and floodplain food webs.

(c) “Seasonally Flooded” means a hydroperiod category where surface water is typically present for extended periods (30 days or more) during the growing season, resulting in a predominance of submerged or submerged and transitional wetland species. During extended periods of normal or above normal rainfall, lake levels causing inundation are expected to occur several weeks to several months every one to two years.

(d) “Semi-Permanently Flooded” means a hydroperiod category where surface water inundation persists in most years. When surface water is absent the water table is usually near the land surface. In many lakes with emergent marshes this water level is near the lower elevation that supports emergent marsh or floating vegetation and peat substrates, or other highly organic hydric substrates. This characterization may not be true for herbaceous wetlands around sandhill type lakes, which often have emergent vegetation that follows declining water levels to below the lower elevation of peat substrate. Water levels causing inundation are expected to occur approximately eighty percent of the time over a long-term period of record. Exposure of these ground elevations is expected to re-occur, on average, about every five to ten years for extended periods (several or more months) during moderate droughts.

(2) The following minimum surface water levels are established:

| Water Body Name | County (Latitude/ Longitude) | Minimum Level | Level (Feet NAVD) | Event | Hydroperiod Category | Duration (Days) | Return Interval (Interval Measurement Period) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Lake Butler | Union (30º02ꞌ00ꞌꞌN/ 082º20’12”W) | Minimum Frequent High | 129.55 | Flooding | Seasonally flooded | 30 | 2 years (June 1 to May 31) |
|  |  | Minimum Frequent Low | 127.30 | Exposure | Semi-permanently flooded | 120 | 5 years (October 1 to September 30) |

Rulemaking Authority 373.044, 373.113 FS. Law Implemented 373.042, 373.0421, 373.103 FS. History–New 8-9-21.