- 40D-8.624 Guidance and Minimum Levels for Lakes.

(1) Levels for lakes adopted during or after August 7, 2000, are set forth in Table 8-2 in subsection 40D-8.624(7), F.A.C. There are two Minimum Levels for these lakes, the High Minimum Lake Level and the Minimum Lake Level. For lakes with levels adopted during or after August 7, 2000, a reference in the District’s rules to the applicable minimum level or to established minimum water levels shall refer to the adopted High Minimum Lake Level and Minimum Lake Level described in this Rule 40D-8.624, F.A.C.

(2) Levels for lakes adopted prior to August 7, 2000, are set forth in Table 8-3 in subsection 40D-8.624(8), F.A.C. There are two Guidance Levels for these lakes, the High Level and the Low Level.

For lakes with levels adopted before August 7, 2000, a reference within the District’s rules to the applicable minimum level or to established minimum water levels shall refer to the Low Level described in this Rule 40D-8.624, F.A.C. Such a Low Level shall not be deemed a minimum level pursuant to Section 373.042, F.S.

(3) The High Minimum Lake Level (HMLL) is the elevation that a lake’s water levels are required to equal or exceed ten percent of the time on a Long-term basis. The High Minimum Lake Level is established to ensure that a lake reaches higher levels on a periodic basis.

(4) The Minimum Lake Level (MLL) is the elevation that the lake’s water levels are required to equal or exceed fifty percent of the time on a Long-term basis.

(5) Status assessments of the Minimum Levels for lakes for which Minimum Levels have been adopted will be completed annually to determine whether the respective lake’s level is below or projected to fall below the Minimum Levels. Each status assessment is independent from and not a determination of water use permit compliance or environmental resource permit compliance. Permit compliance is a regulatory function that is not within the scope of this subsection. As part of each status assessment, the District will use the following approach:

(a) The District will evaluate the Minimum Levels to determine the extent to which the water levels have been reduced due to withdrawals as of the date of each status assessment. The annual evaluation will be completed through a review of:

1. Surface Water Levels;

2. Water withdrawals;

3. Aquifer water levels;

4. Rainfall; and

5. Hydrologic modeling.

(b) The District will also evaluate the Minimum Levels every five years as part of the regional water supply planning process. This evaluation will include the use of hydrologic modeling.

(c) If the Minimum Levels are being met based on the annual evaluation or the evaluation performed as a part of the regional water supply planning process, then no further actions are required beyond continued monitoring.

(6) Levels for lakes established during or after August 7, 2000, are set forth in the following table. Guidance Levels for lakes established prior to August 7, 2000, are set forth in Table 8-3 in subsection 40D-8.624(7), F.A.C., below.

| Table 8-2 Minimum Levels Established During or After August 7, 2000. Levels are elevations, in feet above the National Geodetic Vertical Datum of 1929. |  |  |  |
| --- | --- | --- | --- |
| Location by County and Basin | Name of Lake and Section, Township and Range Information | High Minimum Lake Level | Minimum Lake Level |
| (a) In Charlotte County Within the Peace River Basin RESERVED |  |  |  |
| (b) In Citrus County Within the Coastal Rivers Basin RESERVED |  |  |  |
| (c) In Citrus County Within the Withlacoochee River Basin | Fort Cooper, Lake S-27, T-19S, R-20E | 30.1' | 28.7' |
|  | Tsala Apopka, Lake (Lakes and wetlands comprising the Floral City Pool), S-14, T-20S, R-20E | 41.2' | 39.8' |
|  | Tsala Apopka, Lake (Lakes and wetlands comprising the Hernando Pool), S-26, T-18S, R-19E | 38.7' | 37.3' |
|  | Tsala Apopka, Lake (Lakes and wetlands comprising the Inverness Pool), S-08, T-19S, R-20E | 40.1' | 38.7' |
| (d) In DeSoto County Within the Peace River Basin RESERVED |  |  |  |
| (e) In Hardee County Within the Peace River Basin RESERVED |  |  |  |
| (f) In Hernando County Within the Coastal Rivers Basin | Hunters, Lake S-32, T-23S, R-17E | 18.6' | 16.4' |
|  | Tooke Lake S-13 & 24, T-22S, R-17E | 20.4' | 16.3' |
|  | Whitehurst Pond S-13 & 24, T-22S, R-17E | 22.4' | 17.5' |
|  | Weekiwachee Prairie, Lake (Lake Theresa) S-23, T-23S, R-17E | 21.4' | 18.3' |
| (g) In Hernando County Within the Hillsborough River Basin RESERVED |  |  |  |
| (h) In Hernando County Within the Withlacoochee River Basin | Lindsey, Lake S-25, T-21S, R-19E | 67.6' | 65.3' |
|  | Mountain, Lake S-16, T-23S, R-20E | 102.0' (CAT 3) | 98.7' (CAT 3) |
|  | Neff, Lake S-20, T-23S, R-20E | 100.7' | 93.7' |
|  | Spring, Lake S-15, T-23S, R-20E | 180.1' | 179.0' |
| (i) In Highlands County Within the Peace River Basin | Angelo, Lake S-25, T-33S, R-28E | 101.3' | 100.0' |
|  | Anoka, Lake S-27, T-33S, R-28E | 122.8' | 121.7' |
|  | Damon, Lake S-3, T-33S, R-28E | 97.4' | 96.3' |
|  | Denton, Lake S-02, T-34S, R-28E | 114.1' | 112.8' |
|  | Jackson, Lake S-30, T-34S, R-29E | 102.3' | 101.2' |
|  | June-In-Winter, Lake S-34, T-36S, R-29E | 74.5' | 74.0' |
|  | Letta, Lake S-31, T-33S, R-29E | 98.8' | 97.7' |
|  | Little Jackson, Lake S-06, T-35S, R-29E | 102.3' | 101.2' |
|  | Lotela, Lake S-26, T-33S, R-28E | 106.1' | 105.0' |
|  | Placid, Lake S-24, T-37S, R-29E | 92.6' | 91.4' |
|  | Tulane, Lake S-27, T-33S, R-28E | 117.9' | 116.6' |
|  | Verona, Lake S-23, T-33S, R-28E | 117.4' | 115.8' |
| (j) In Hillsborough County Within the Alafia River Basin | Wimauma, Lake S-9 and 16, T-32S, R-20E | 81.0' | 78.4' |
| (k) In Hillsborough County Within the Hillsborough River Basin | Hanna, Lake S-18, T-27S, R-19E | 61.2' | 59.2' |
|  | Hooker, Lake S-1, 12, T-29S, R-20E | 43.7' | 42.1' |
|  | Keene, Lake S-7, T 27S, R-19E | 61.5' | 60.1' |
|  | Kell, Lake S-13, 24, T-22S, R-17E | 65.6' | 64.2' |
|  | Stemper, Lake S-13, T-27S, R-18E | 60.8' | 59.4' |
| (l) In Hillsborough County Within the Northwest Hillsborough Basin | Alice, Lake S-16, T-27S, R-17E | 40.7' | 38.9' |
|  | Allen, Lake S-10, T-27S, R-18E | 61.6' | 60.7' |
|  | Barbara, Lake S-19, T-27S, R-18E | 53.1' | 51.4' |
|  | Bird, Lake S-26, T-27S, R-18E | 49.3' | 48.1' |
|  | Brant, Lake S-14, T-27S, R-18E | 57.6' | 55.9' |
|  | Calm, Lake S-14, T-27S, R-17E | 49.6' | 47.7' |
|  | Carroll, Lake S-10, 11, 14, 15, T-28S, R-18E | 36.8' | 35.4' |
|  | Charles, Lake S-23, T-27S, R-18E | 53.3' | 51.9' |
|  | Church, Lake S-28, T-27S, R-17E | 35.2' | 34.4' |
|  | Crenshaw, Lake S-22, T-27S, R-18E | 54.45' | 53.45' |
|  | Crescent, Lake S-10, T-27S, R-17E | 41.3' | 40.3' |
|  | Crystal, Lake S-14, T-27S, R-18E | 59.8' | 58.2' |
|  | Cypress, Lake S-24, T-27S, R-17E | 48.9' | 47.4' |
|  | Dan, Lake S-6, T-27S, R-17E | 32.3' | 30.9' |
|  | Deer, Lake S-1, T-27S, R-18E | 66.5' | 65.1' |
|  | Dosson Lake S-20, T-27S, R-18E | 53.2' | 52.1' |
|  | Echo, Lake S-28, T-27S, R-17E | 35.2' | 34.4' |
|  | Ellen, Lake S-19, T-27S, R-18E | 53.1' | 51.4' |
|  | Fairy (Maurine), Lake S-34, T-27S, R-17E | 32.5' | 32.1' |
|  | Garden, Lake S-17, T-27S, R-17E | 30.5' | 29.5' |
|  | Halfmoon, Lake S-31, T-27S, R-18E | 42.5' | 41.1' |
|  | Harvey, Lake S-3, T-27S, R-18E | 61.6' | 60.7' |
|  | Helen, Lake S-19, T-27S, R-18E | 53.1' | 51.4' |
|  | Hobbs, Lake S-1, T-27S, R-18E | 64.9' | 63.2' |
|  | Horse, Lake S-26, T-27S, R-17E | 44.9' | 43.9' |
|  | Jackson, Lake S-17, T-27S, R-17E | 32.2' | 31.2' |
|  | Juanita, Lake S-22, T-27S, R-17E | 41.0' | 39.5' |
|  | Little Moon, Lake S-28, T-27S, R-17E | 39.6' | 38.2' |
|  | Merrywater, Lake S-22, T-27S, R-18E | 56.6' | 55.2' |
|  | Mound, Lake S-11, T-27S, R-17E | 50.7' | 49.3' |
|  | Platt, Lake S-35, T-27S, R-18E | 49.5' | 48.1' |
|  | Pretty, Lake S-26, T-27S, R-17E | 43.9' | 42.5' |
|  | Rainbow, Lake S-22, T-27S, R-17E | 39.6' | 38.2' |
|  | Raleigh, Lake S-27, T-27, R-17 | 41.1' | 37.9' |
|  | Reinheimer, Lake S-15, T-27S, R-18E | 58.9' | 57.5' |
|  | Rogers, Lake S-27, T-27, R-17 | 38.7' | 35.6' |
|  | Round, Lake S-22, T-27S, R-18E | 54.1' | 53.1' |
|  | Saddleback, Lake S-22, T-27S, R-18S | 53.6' | 52.7' |
|  | Sapphire, Lake S-14, T-27S, R-18E | 62.7' | 61.0' |
|  | Starvation, Lake S-21, T-27S, R-18E | 52.7' | 50.4' |
|  | Strawberry (North Crystal), Lake S-14, T-27S, R-18E | 59.3' | 58.3' |
|  | Sunset, Lake S-17, T-27S, R-17E | 32.8' | 31.5' |
|  | Sunshine Lake S-20, T-27S, R-18E | 53.2' | 52.1' |
|  | Taylor, Lake S-16, T-27S, R-17E | 37.6' | 36.6' |
|  | Virginia, Lake S-3, T-27S, R-18E | 61.6' | 60.7' |
| (m) In Lake County Within the Green Swamp Basin RESERVED |  |  |  |
| (n) In Levy County Within the Withlacoochee River Basin | Marion, Lake S-02, T-14S, R-17E | 52.6' | 49.4' |
| (o) In Manatee County Within the Manasota River Basin RESERVED |  |  |  |
| (p) In Marion County Within the Withlacoochee River Basin | Bonable, Lake S-31, T-15, R-18 | 62.8' | 58.3' |
|  | Little Bonable, Lake S-30, T-15, R-18 | 57.8' | 52.2' |
|  | Tiger, Lake S32, T15, R-18 | 62.8' | 58.3' |
| (q) In Pasco County Within the Coastal Rivers Basin | Big Fish, Lake S-21, T-24, R-19E | 75.4' | 72.8' |
|  | Crews, Lake S-20, T-24S, R18E | 52.4' | 51.0' |
|  | Green, Lake S-16, T-26, R-18E | 74.2' | 71.9' |
|  | Moon, Lake S-28, T-25, R-17E | 39.6' | 38.2' |
|  | Pierce Lake S-9, T-25S, R-18E | 71.9' | 69.8' |
|  | Unnamed Lake No. 22 S-27, T-24S, R-18E | 59.3' | 55.8' |
| (r) In Pasco County Within the Green Swamp Basin RESERVED |  |  |  |
| (s) In Pasco County Within the Hillsborough River Basin | Bell, Lake S-13, T-26, R-18E | 70.8' | 69.4' |
|  | Bird, Lake S-36, T-26, R-18E | 66.6' | 65.2' |
|  | Buddy, Lake S-17, T-25S, R-21E | 92.5' | 87.6' |
|  | Hancock, Lake S-5, T-24S, R-20E | 101.7' | 99.4' |
|  | Iola, Lake S-15, T-24S, R-20E | 146.5' | 141.9' |
|  | Jessamine Lake S-11, T-24S, R-20E | 140.3' | 133.3' |
|  | King Lake S-7, T-26S, R-19E | 72.4' | 70.8' |
|  | King Lake (East) S-22, T-25S, R-20E | 105.1' | 102.7' |
|  | Middle, Lake S-4, T-24S, R-20E | 102.5' | 100.2' |
|  | Padgett, Lake S-24, T-26S, R-18E | 70.0' | 68.6' |
|  | Pasadena, Lake S-16, T-25S, R-21E | 92.5' | 87.6' |
| (t) In Pasco County Within the Pinellas-Anclote River Basin | Camp, Lake S-34, T-26, R-18E | 63.4' | 62.0' |
|  | Linda, Lake S-26, T-26S, R-18E | 66.2' | 64.7' |
|  | Parker (Ann), Lake S-35, T-26, R-17E | 48.1' | 46.7' |
| (u) In Pasco County Within the Withlacoochee River Basin | Clear, Lake S-1, T-25S, R-20E | 126.2' | 124.9' |
| (v) In Pinellas County Within the Pinellas-Anclote River Basin RESERVED |  |  |  |
| (w) In Polk County Within the Alafia River Basin RESERVED |  |  |  |
| (x) In Polk County Within the Green Swamp Basin RESERVED |  |  |  |
| (y) In Polk County Within the Hillsborough River Basin RESERVED |  |  |  |
| (z) In Polk County Within the Ocklawaha River Basin |  |  |  |
|  | Lowery, Lake S-14, T-27S, R-26E | 129.7' | 127.9' |
| (aa) In Polk County Within the Peace River Basin | Annie, Lake S-3, T-29S, R-27E | 115.2' | 112.8' |
|  | Aurora, Lake S13, T30S, R28E | 110.3' | 97.3' |
|  | Easy, Lake S19, T30S, R28E | 109.8' | 106.5' |
|  | Eva, Lake S-32, T-27S, R-27E | 119.2' | 118.1' |
|  | Bonnie, Lake S-31, T-29S, R-28E | 105.8' | 102.1' |
|  | Clinch, Lake S-31, T-31S, R-28E | 105.7' | 103.2' |
|  | Crooked, Lake S-01, T-31S, R-27E | 120.7' | 117.7' |
|  | Crystal, Lake S-02, T-30S, R-27E | 117.5' | 114.2' |
|  | Dinner, Lake S-15, T-29S, R-27E | 113.6' | 110.6' |
|  | Eagle Lake S-01, T-29S, R-25E | 131.2' | 129.1' |
|  | Hancock, Lake S-08, T-29S, R-25E | 98.8' | 97.6' |
|  | Lee, Lake S-10, T-29S, R-27E | 116.0' | 113.9' |
|  | Mabel, Lake S-11, T-29S, R-27E | 106.7' | 103.9' |
|  | McLeod, Lake S-07, T-29S, R-26E | 130.3' | 128.3' |
|  | North Lake Wales S-01, T-30S, R-27E | 109.2' | 105.7' |
|  | Parker, Lake S-8, T-28S, R-24E | 130.3' | 129.3' |
|  | Starr, Lake S-14, T-29S, R-27E | 106.4' | 103.2' |
|  | Venus Lake S-9, T-29S, R-27E | 120.4' | 118.2' |
|  | Wailes Lake S-01, T-30S, R-27E | 107.7' | 104.8' |
| (bb) In Sarasota County Within the Manasota Basin RESERVED |  |  |  |
| (cc) In Sumter County Within the Green Swamp Basin RESERVED |  |  |  |
| (dd) In Sumter County Within the Withlacoochee River Basin | Big Gant, Lake S-14, T-22S, R-22E | 76.3' | 74.9' |
|  | Black, Lake S-23, T-18S, R-23E | 53.9' | 51.3' |
|  | Deaton, Lake S-14, T-19S, R-23E | 64.8' | 63.2' |
|  | Miona, Lake S-27, T-18S, R-23E | 53.9' | 51.3' |
|  | Okahumpka, Lake S-21, T-19S, R-23E | 58.1' | 56.7' |
|  | Panasoffkee, Lake S-29, T-19S, R-22E | 40.8' | 39.4' |

(7) Guidance Levels established for lakes prior to August 7, 2000, are set forth in the following table:

| Table 8-3 Guidance Levels established prior to August 7, 2000 |  |  |
| --- | --- | --- |
| Location of Impoundment by County and Basin | High Level in Feet Above Mean Sea Level (msl) | Low Level in Feet Above Mean Sea Level (msl) |
| (a) In Charlotte County Within the Peace River Basin |  |  |
| (b) In Citrus County Within the Coastal Rivers Basin |  |  |
| (c) In Citrus County Within the Withlacoochee River Basin |  |  |
| Bradley, Lake S23, T20S, R20E | 42.50' | 40.25' |
| Cato, Lake S5, T19S, R20E | 38.00' | 35.50' |
| Connell, Lake S6, T19S, R20E | 35.50' | 32.00' |
| Cooter, Lake S17, T19S, R20E | 40.50' | 38.25' |
| Hog Pond, Lake (Nina) S2, T19S, R19E | 34.50' | 32.00' |
| Holden, Lake (Inverness) S32, T19S, R20E | 32.00' | 29.50' |
| Magnolia, Lake S3, T20S, R20E | 31.00' | 28.50' |
| Rush, Lake (Williams) S03, T17S, R18E | 35.75' | 33.25' |
| Little, Lake (Consuella) S15, T20S, R20E | 41.50' | 39.00' |
| Twin, Lake S6, T19S, R20E | 37.00' | 35.00' |
| (d) In DeSoto County Within the Peace River Basin |  |  |
| (e) In Hardee County Within the Peace River Basin |  |  |
| (f) In Hernando County Within the Coastal Rivers Basin |  |  |
| (g) In Hernando County Within the Hillsborough River Basin LAKES |  |  |
| Nicks, Lake S29, T23S, R20E | 113.00' | 110.50' |
| St. Clair, Lake S33, T23S, R20E | 113.00' | 110.50' |
| (h) In Hernando County Within the Withlacoochee River Basin LAKES |  |  |
| Elizabeth, Lake S11, T23S, R21E | 60.25' | 57.00' |
| Francis, Lake S11, T23S, R21E | 60.25' | 57.00' |
| Geneva, Lake S11, T23S, R21E | 60.25' | 57.00' |
| Sparkman, Lake S24, T23S, R19E | 91.50' | 89.00' |
| (i) In Highlands County Within the Peace River Basin LAKES Sec. Twsp. Rng. |  |  |
| Adelaide, Lake S5, T33S, R28E | 106.50' | 104.00' |
| Apthhorpe, Lake S18, T36S, R30E | 71.50' | 68.00' |
| Blue, Lake S30, T36S, R30E | 77.50' | 75.00' |
| Bonnet, Lake S8, T34S, R29E | 90.75' | 88.00' |
| Brentwood, Lake S10, T33S, R28E | 102.75' | 99.50' |
| Buck, Lake S29, T37S, R30E | 94.00' | 91.50' |
| Byrd, Lake S9, T33S, R28E | 108.25' | 105.50' |
| Carrie, Lake S21, T36S, R29E | 75.50' | 73.00' |
| Charlotte, Lake S17, T35S, R29E | 93.75' | 91.25' |
| Chilton, Lake S7, T33S, R28E | 114.00' | 111.00' |
| Clay, Lake S29, T36S, R30E | 78.75' | 76.00' |
| Crews, Lake S32, T36S, R29E | 119.50' | 117.00' |
| Dinner, Lake S17, T34S, R29E | 102.50' | 98.50' |
| Francis, Lake S22, T36S, R29E | 70.50' | 67.50' |
| Glenada, Lake S34, T33S, R28E | 120.00' | 117.00' |
| Grassy, Lake S17, T37S, R30E | 91.50' | 88.50' |
| Harry, Lake S1, T36S, R29E | 67.50' | 63.00' |
| Henry, Lake S25, T36S, R29E | 75.50' | 73.00' |
| Hill, Lake S17, T36S, R29E | 99.25' | 96.00' |
| Huckleberry, Lake S7, T35S, R29E | 104.50' | 102.00' |
| Huntley, Lake S5, T37S, R30E | 83.75' | 81.00' |
| Josephine, Lake S32, T35S, R29E | 72.50' | 69.00' |
| Lake Lachard S36, T36S, R29E | 78.50' | 76.00' |
| Lelia, Lake S34, T33S, R29E | 114.50' | 112.50' |
| Little Bonnet, Lake S36, T33S, R28E | 100.00' | 97.00' |
| Little Red Water, Lake S14, T36S, R29E | 103.25' | 100.50' |
| Lost, Lake S12, T37S, R29E | 88.00' | 84.00' |
| McCoy, Lake S6, T37S, R30E | 87.00' | 84.00' |
| Mirror, Lake S7, T37S, R30E | 93.50' | 90.00' |
| Center Nellie S13, T36S, R29E | 71.50' | 67.00' |
| Nellie N.W., Lake S13, T36S, R29E | 71.50' | 67.00' |
| Nellie S.E., Lake S13, T36S, R29E | 71.50' | 67.00' |
| Olivia, Lake S6, T33S, R28E | 117.50' | 114.50' |
| Pearl, Lake S6, T37S, R30E | 87.00' | 84.00' |
| Persimmon, Lake S10, T36S, 29E | 68.25' | 65.00' |
| Pioneer, Lake S11, T33S, R28E | 108.00' | 104.50' |
| Pythias, Lake S2, T33S, R28E | 101.00' | 98.00' |
| Red Beach Lake S15, T35S, R29E | 76.50' | 73.75' |
| Red Water Lake S14, T36S, R29E | 70.50' | 67.50' |
| Ruth, Lake S18, T35S, R29E | 94.00' | 91.50' |
| Saddlebags, Lake S6, T37S, R30E | 84.00' | 81.00' |
| Sebring, Lake S14, T34S, R28E | 107.25' | 104.50' |
| Simmons, Lake S24, T36S, R29E | 72.50' | 68.00' |
| Sirena, Lake S1, T37S, R29E | 87.00' | 84.00' |
| Trout, Lake S34, T32S, R28E | 101.00' | 98.00' |
| Unnamed Lake (B) S20, T37S, R30E | 91.50' | 88.50' |
| Unnamed Lake (F) S24, T36S, R29E | 78.00' | 74.00' |
| Viola, Lake S14, T33S, R28E | 109.50' | 105.75' |
| Wolf Lake S24, T35S, R28E | 92.50' | 90.00' |
| (j) In Hillsborough County Within the Alafia River Basin LAKES |  |  |
| Carlton, Lake S7, T29, R21 | 93.50' | 90.50' |
| Edward Medard Reservoir S36, T32, R21 | 62.75' | 57.50' |
| Grady, Lake S26, T29, R20 | 39.00' | 36.00' |
| Hickory Hammock, Lake S34, T32, R21 | 32.25' | 30.50' |
| Unnamed Lake #2 S22, T32, R21 | 56.00' | NO RECOMMENDATION |
| (k) In Hillsborough County Within the Hillsborough River Basin LAKES |  |  |
| Bellows, Lake (East Lake) S2, T29, R19 | 23.75' | 21.50' |
| Burrell, Lake S31, T27, R19 | 50.00' | 47.50' |
| Commiston, Lake S12, T27, R18 | 63.00' | 60.50' |
| Eckles, Lake S11, T28, R18 | 32.50' | 30.00' |
| Egypt, Lake S27, T28, R18 | 37.50' | 35.00' |
| Gornto, Lake S21, T29, R20 | 38.50' | 36.00' |
| Hart, Lake S6, T27, R19 | 66.00' | 64.00' |
| Hog Island, Lake S1, T27, R18 | 66.00' | 64.00' |
| Kathy, Lake S20, T29, R20 | 43.50' | 42.50' |
| Long, Lake S36, T27, R18 | 50.25' | 48.00' |
| Long Pond S13, T29, R20 | 46.50' | 44.00' |
| Mud, Lake (Lake Walden) S6, T29, R22 | 115.00' | 112.50' |
| Thonotosassa, Lake S14, T28, R20 | 37.00' | 34.50' |
| Unnamed Lake S18, T27, R19 | 63.00' | 60.50' |
| Unnamed Lake S7, T27, R19 | 61.00' | 58.50' |
| Valrico, Lake S24, T29, R20 | 45.00' | 42.50' |
| Weeks, Lake S2, T29, R20 | 43.25' | 41.00' |
| (l) In Hillsborough County Within the Northwest Hillsborough Basin LAKES Sec. Twsp. Rng. |  |  |
| SWEETWATER CREEK WATERSHED |  |  |
| Avis, Lake S15, T28S, R18E | 37.00' | 34.50' |
| Bay, Lake S3, T28, R18 | 46.75' | 44.00' |
| Boat, Lake S15, T28, R18 | 35.50' | 33.75' |
| Brooker, Lake S2, T27, R18 | 64.25' | 61.00' |
| Chapman, Lake S25, T27, R18 | 52.25' | 49.50' |
| Cooper, Lake S11, T27, R18 | 61.75' | 59.75' |
| Elaine, Lake S15, T28S, R18E | 37.00' | 34.50' |
| Ellen, Lake S15, T28S, R18E | 41.50' | 39.00' |
| Gass, Lake S36, T27, R18 | 49.50' | 46.25' |
| George, Lake S10, T28, R18 | 48.00' | 45.00' |
| Geraci, Lake S15, T27, R18 | 63.50' | 61.50' |
| Halls, Lake S3, T28S, R18E | 50.00' | 47.50' |
| Lipsey, Lake S10, T28, R18 | 41.50' | 39.00' |
| Magdalene, Lake S35, T27, R18 | 50.00' | 47.50' |
| Thomas, Lake S11, T27, R18 | 63.50' | 61.25' |
| Twin, Lake S22, T28, R18 | 32.00' | 30.00' |
| White Trout, Lake S15, T28, R18 | 36.50' | 34.00' |
| ROCKY CREEK WATERSHED |  |  |
| Armistead, Lake S25, T27, R17 | 44.00' | 40.50' |
| Browns, Lake S2, T27, R18 | 63.50' | 60.75' |
| Josephine, Lake S25, T27, R17 | 46.00' | 42.75' |
| LeClare, Lake S30, T27, R18 | 52.00' | 49.50' |
| Rock, Lake S25, T27, R17 | 46.00' | 42.75' |
| Turkey Ford, Lake S18, T27, R18 | 54.00' | 51.50' |
| DOUBLEBRANCH CREEK WATERSHED |  |  |
| Hixon, Lake S3, T28, R17 | 36.50' | 33.25' |
| Unnamed Lake #1 S2, T28S, R17E | 37.50' | 35.00' |
| Unnamed Lake #2 S2, T28S, R17E | 37.50' | 35.00' |
| BROOKER CREEK WATERSHED |  |  |
| Artillery, Lake S3, T27S, R17E | 44.00' | 40.50' |
| Buck, Lake S28, T27, R17 | 35.00' | 32.00' |
| Elizabeth, Lake S11, T27, R17 | 53.00' | 51.00' |
| Fern, Lake S10, T27, R17 | 46.00' | 43.00' |
| Frances, Lake S4, T27, R17 | 40.50' | 38.00' |
| Island Ford, Lake S10, T27, R17 | 41.50' | 39.00' |
| James, Lake S23, T27S, R17E | 46.50' | 43.50' |
| Keystone, Lake S15, T27, R17 | 42.00' | 39.75' |
| Little, Lake S23, T27S, R17E | 46.50' | 43.50' |
| Velburton, Lake S21, T27, R17 | 40.00' | 37.25' |
| ANCLOTE RIVER WATERSHED |  |  |
| Hiawatha, Lake S2, T27, R17 | 50.50' | 48.00' |
| Osceola, Lake S3, T27, R17 | 46.50' | 44.50' |
| (m) In Lake County Within the Green Swamp Basin |  |  |
| (n) In Levy County Within the Withlacoochee River Basin |  |  |
| (o) In Manatee County Within the Manasota Basin |  |  |
| (p) In Marion County Within the Withlacoochee River Basin |  |  |
| (q) In Pasco County Within the Coastal Rivers Basin LAKES |  |  |
| Garden, Lake S16, T25S, R16E | 19.00' | 16.25' |
| Richey, Lake S3, T26S, R16E | 13.00' | 10.00' |
| Worrell, Lake (Bass Lake) S26, T25S, R16E | 19.00' | 16.25' |
| (r) In Pasco County Within the Green Swamp Basin |  |  |
| (s) In Pasco County Within the Hillsborough River Basin LAKES |  |  |
| Catfish, Lake S30, T25S, R19E | 68.00' | 65.50' |
| Cow (East), Lake S19, T26S, R19E | 78.50' | 76.00' |
| Floyd, Lake S36, T26S, R19E | 68.50' | 66.00' |
| Gooseneck, Lake S29, T26S, R19E | 73.50' | 71.00' |
| Hog (Joyce), Lake S19, T26S, R19E | 76.50' | 73.50' |
| JoAnn, Lake S30, T26S, R19E | 68.00' | 65.50' |
| Moody, Lake S10, T24S, R20E | 110.00' | 107.50' |
| Myrtle, Lake S30, T26S, R19E | 68.00' | 65.50' |
| Saxon, Lake S30, T26S, R19E | 71.25' | 69.00' |
| Tampa (Turtle), Lake S32, T26S, R19E | 65.50' | 63.00' |
| Toni, Lake S30, T26S, R19E | 68.00' | 65.50' |
| Twin, Lake S28, T26S, R19E | 67.50' | 65.00' |
| Unnamed Lake #26 S25, T26S, R18E | 68.00' | 65.50' |
| (t) In Pasco County Within the Pinellas-Anclote River Basin LAKES |  |  |
| Bass, Lake (Holiday) S34, T26S, R17E | 48.75' | 45.75' |
| Big Lake Vienna S23, T26S, R18E | 70.25' | 67.00' |
| Fishing, Lake S34, T26S, R17E | 48.75' | 45.75' |
| Geneva, Lake (Mud) S26, T26S, R17E | 50.00' | 48.00' |
| Little Moss (Como), Lake S35, T26S, R18E | 66.00' | 63.00' |
| Minniola, Lake S34, T26S, R17E | 50.00' | 48.00' |
| Moss, Lake S35, T26S, R18E | 64.00' | 61.50' |
| Seminole, Lake S35, T26S, R17E | 48.75' | 46.00' |
| Thomas, Lake S11, T26S, R18E | 75.00' | 72.50' |
| Wistaria, Lake S2, T26S, R18E | 74.00' | 71.00' |
| (u) In Pasco County Within the Withlacoochee River Basin LAKES |  |  |
| (v) In Pinellas County Within the Pinellas-Anclote River Basin LAKES |  |  |
| Lake Tarpon Sec.Twsp.Rng. S7, T27S, R16E S8, T27S, R16E S16, T27S, R16E S17, T27S, R16E S18, T27S, R16E S19, T27S, R16E S20, T27S, R16E S21, T27S, R16E S28, T27S, R16E S29, T27S, R16E S30, T27S, R16E S32, T27S, R16E S33, T27S, R16E S4, T28S, R16E | 3.80' | 2.20' |
| (w) In Polk County Within the Alafia River Basin LAKES |  |  |
| Scott, Lake S17, T29, R24 | 168.00' | 165.00' |
| (x) In Polk County Within the Green Swamp Basin LAKES |  |  |
| Agnes, Lake S4, T27, R25 | 135.75' | 134.75' |
| Alfred, Lake S30, T27S, R26E | 130.75' | 128.25' |
| Arietta, Lake S27, T27, R25 | 144.00' | 141.00' |
| Camp, Lake S20, T27, R26 | 134.50' | 132.00' |
| Clearwater Lake S5, T27, R25 | 143.50' | 141.00' |
| Cummings, Lake S31, T27S, R26E | 131.00' | 127.50' |
| Eva, Lake S29, T27S, R26E | 131.50' | 129.00' |
| Grassy, Lake (Big Glades) S19, T27, R26 | 132.00' | 129.50' |
| Griffin, Lake S30, T27S, R26E | 131.50' | 129.00' |
| Gum, Lake S17, T27, R26 | 131.00' | 128.50' |
| Haines, Lake S33, T27, R26 | 128.75' | 126.50' |
| Helene, Lake S34, T26, R25 | 144.00' | 141.00' |
| Juliana, Lake S15, T27, R25 | 132.50' | 130.00' |
| Little Lake Agnes S3, T27, R25 | 136.00' | 133.00' |
| Little Van, Lake S26, T27, R25 | 139.00' | 136.50' |
| Mattie, Lake S14, T27, R25 | 132.50' | 130.00' |
| Mud, Lake S6, T27, R25 | 141.50' | 137.75' |
| Myrtle, Lake S32, T27, R25 | 141.00' | 138.50' |
| Swoope, Lake S29, T27, R26 | 132.50' | 130.00' |
| Tennessee, Lake S9, T27, R25 | 134.00' | 130.00' |
| Van, Lake S26, T27, R25 | 132.75' | 130.00' |
| Whistle, Lake S33, T27, R25 | 137.50' | 135.00' |
| (y) In Polk County Within the Hillsborough River Basin LAKES |  |  |
| Bonnet, Lake S14, T28, R23 | 148.00' | 145.00' |
| Hunter, Lake S24, T28, R23 | 162.75' | 160.25' |
| (z) In Polk County Within the Peace River Basin |  |  |
| Ada, Lake S33, T28S, R27E | 123.00' | 120.00' |
| Altamaha, Lake S11, T30S, R27E | 122.50' | 120.00' |
| Amoret, Lake S24, T30S, R27E | 115.25' | 113.00' |
| Ariana, Lake S3, T28S, R25E | 137.00' | 134.50' |
| Banana, Lake S10, T29S, R24E | 106.50' | 103.50' |
| Belle, Lake S11, T30S, R27E | 120.00' | 117.00' |
| Bess, Lake S18, T29S, R27E | 125.25' | 123.00' |
| Big Gum, Lake S26, T29S, R28E | 95.00' | 92.00' |
| Blue, Lake S13, T28S, R25E | 149.00' | 146.50' |
| Blue Lake S24, T30S, R27E | 117.00' | 114.00' |
| Bonny, Lake S20, T28S, R24E | 130.50' | 128.00' |
| Buckeye, Lake S22, T28S, R26E | 129.00' | 126.00' |
| Buffum, Lake S12, T31S, R26E | 132.25' | 129.25' |
| Cannon, Lake S19, T28S, R26E | 132.00' | 129.50' |
| Connie, Lake S9, T28S, R26E | 128.75' | 126.50' |
| Cooper (Worth) S02, T30S, R27E | 123.50' | 121.00' |
| Crystal, Lake S21, T28S, R27E | 122.00' | 119.00' |
| Crystal, Lake S23, T29S, R26E | 129.50' | 127.00' |
| Cypress, Lake S36, T29S, R28E | 98.50' | 95.00' |
| Lake Daisy S6, T29S, R27E | 130.00' | 127.00' |
| Lake Deer S25, T28S, R25E | 140.75' | 138.50' |
| Dell, Lake S28, T28S, R27E | 123.75' | 121.50' |
| Lake Dexter S2, T29S, R26E | 132.00' | 129.00' |
| Easy, Lake S19, T30S, R28E | 115.25' | 113.00' |
| Echo, Lake S05, T28S, R26E | 131.00' | 128.00' |
| Effie, Lake S3, T30S, R27E | 118.00' | 115.00' |
| Elbert, Lake S22, T28S, R26E | 135.50' | 133.00' |
| Eloise, Lake S3, T29S, R26E | 132.00' | 129.50' |
| Fannie, Lake S11, T28S, R26E | 125.75' | 123.50' |
| Lake Florence S35, T28S, R26E | 128.75' | 127.00' |
| Lake Fox S6, T29S, R27E | 135.00' | 132.00' |
| Garfield, Lake S5, T30S, R26E | 104.75' | 101.00' |
| Gator, Lake S26, T30S, R26E | 133.00' | 130.75' |
| George, Lake S06, T28S, R26E | 130.00' | 127.50' |
| Gibson, Lake S25, T27S, R23E | 143.50' | 141.50' |
| Gordon, Lake S16, T28S, R27E | 119.00' | 116.00' |
| Lake Grassy S2, T29S, R25E | 129.00' | 126.50' |
| Lake Gross (Grassy) S14, T29S, R26E | 136.00' | 133.50' |
| Hamilton, Lake S18, T28S, R27E | 121.50' | 119.00' |
| Hart, Lake S24, T29S, R26E | 124.50' | 122.00' |
| Hartridge, Lake S8, T28S, R26E | 132.00' | 129.50' |
| Henry, Lake S16, T31S, R26E | 159.00' | 156.00' |
| Henry, Lake S36, T27S, R26E | 126.50' | 124.50' |
| Hickory, Lake S17, T32S, R28E | 98.50' | 96.00' |
| Howard, Lake S30, T28S, R26E | 132.00' | 129.50' |
| Ida, Lake S28, T31S, R28E | 79.00' | 76.50' |
| Ida, Lake S17, T28S, R26E | 135.25' | 132.00' |
| Idyl, Lake S16, T28S, R26E | 134.00' | 131.50' |
| Idylwild, Lake S18, T28S, R26E | 132.00' | 129.50' |
| Jessie, Lake S12, T28S, R25E | 132.00' | 129.50' |
| Josephine, Lake S13, T30S, R27E | 120.00' | 116.50' |
| Josephine, Lake S27, T28S, R27E | 121.50' | 118.00' |
| Lee, Lake S16, T28S, R27E | 123.50' | 121.50' |
| Lena, Lake S9, T28S, R25E | 137.00' | 134.50' |
| Leonore, Lake S10, T31S, R28E | 87.00' | 84.50' |
| Link, Lake S27, T28S, R26E | 128.00' | 125.00' |
| Little Aurora S13, T30, R28 | 100.50' | 98.00' |
| Little Gum, Lake S35, T29S, R28E | 96.50' | 94.00' |
| Little Lake Hamilton S5, T28S, R27E | 121.50' | 119.00' |
| LuLu, Lake S4, T29S, R26E | 132.00' | 129.50' |
| Mariam, Lake S27, T28S, R26E | 124.75' | 122.75' |
| Marie, Lake S27, T28S, R27E | 121.00' | 118.00' |
| Martha, Lake S21, T28S, R26E | 142.00' | 139.00' |
| Maude, Lake S21, T28S, R26E | 140.50' | 137.50' |
| May, Lake S29, T28S, R26E | 132.00' | 129.50' |
| Medora, Lake S36, T27S, R25E | 138.00' | 134.50' |
| Menzie, Lake S28, T28S, R27E | 122.00' | 120.00' |
| Middle Lake Hamilton S7, T28S, R27E | 121.50' | 119.00' |
| Lake Millsite S11, T29S, R25E | 123.50' | 121.00' |
| Mirror, Lake S20, T28S, R27E | 132.00' | 129.50' |
| Moody, Lake S17, T31S, R28E | 93.50' | 91.00' |
| Myrtle, Lake S19, T29S, R27E | 118.50' | 116.50' |
| Lake Ned S1, T29S, R26E | 128.50' | 126.00' |
| Otis, Lake S28, T28S, R25E | 128.00' | 125.00' |
| Pansy, Lake S08, T28S, R26E | 129.00' | 126.50' |
| Parker, Lake S32, T29S, R27E | 122.00' | 119.50' |
| Parks, Lake S36, T29S, R28E | 102.50' | 100.00' |
| Polecat, Lake S27, T30S, R26E | 142.00' | 139.50' |
| Reedy, Lake S35, T31S, R28E | 79.75' | 77.25' |
| Reeves, Lake S13, T29S, R26E | 124.50' | 122.00' |
| Lake River S1, T29S, R26E | 139.50' | 136.00' |
| Rochelle, Lake S4, T28S, R26E | 128.75' | 126.50' |
| Round, Lake S13, T29S, R26E | 129.25' | 126.50' |
| Roy, Lake S34, T28S, R26E | 132.00' | 129.50' |
| Ruby, Lake S12, T29S, R26E | 125.25' | 123.00' |
| Ruth, Lake S28, T28S, R27E | 121.50' | 117.50' |
| Saddlebag, Lake S6, T30S, R29E | 105.00' | 102.00' |
| Saint Anne Lake S14, T30S, R28E | 96.00' | 93.00' |
| Sanitary (Mariana), Lake S01, T28S, R25E | 137.50' | 135.00' |
| Sara, Lake S17, T28S, R27E | 121.50' | 119.00' |
| Scott, Lake S18, T29S, R24E | 168.00' | 165.00' |
| Lake Sears S36, T28S, R25E | 141.00' | 138.00' |
| Serena, Lake S12, T30S, R27E | 118.00' | 115.00' |
| Shipp, Lake 32, T28S, R26E | 132.00' | 129.50' |
| Silver, Lake S5, T32S, R28E | 103.00' | 100.50' |
| Silver, Lake S20, T28S, R26E | 146.50' | 144.00' |
| Smart, Lake S9, T28S, R26E | 128.75' | 126.50' |
| Lake Spirit S35, T28S, R25E | 131.50' | 129.00' |
| Spring, Lake S20, T28S, R27E | 132.00' | 129.50' |
| Streety, Lake S24, T32S, R27E | 105.50' | 102.50' |
| Summit, Lake S34, T28S, R26E | 132.00' | 129.50' |
| Sunset, Lake S10, T30S, R28 | 98.00' | 95.50' |
| Surveyors, Lake S26, T30S, R26E | 133.00' | 130.75' |
| Thomas, Lake S1, T30S, R28E | 99.50' | 97.00' |
| Lake Thomas S35, T28S, R25E | 132.00' | 128.00' |
| Tractor, Lake S14, T30S, R27E | 123.25' | 121.00' |
| Trask, Lake S22, T28S, R27E | 113.00' | 108.00' |
| Trout, Lake S34, T32S, R28E | 101.00' | 98.00' |
| Twin, Lakes S11, T30S, R27E | 123.75' | 120.00' |
| Walker, Lake S21, T30S, R26E | 141.00' | 137.00' |
| Warren, Lake S11, T30S, R27E | 123.50' | 121.00' |
| Weader (Weaver), Lake S03, T30S, R27E | 121.75' | 119.00' |
| Winterset, Lake S11, T29S, R26E | 132.00' | 129.50' |
| (aa) In Sarasota County Within the Manasota Basin |  |  |
| (bb) In Sumter County Within the Green Swamp Basin |  |  |
| (cc) In Sumter County Within the Withlacoochee River Basin LAKES |  |  |
| Cherry, Lake S24, T18S, R23E | 55.00' | 53.00' |

Rulemaking Authority 373.044, 373.113, 373.171 FS. Law Implemented 373.036, 373.042, 373.0421, 373.086, 373.709 FS. History–New 6-7-78, Amended 1-22-79, 4-27-80, 10-21-80, 12-22-80, 3-23-81, 4-14-81, 6-4-81, 10-15-81, 11-23-81, 1-5-82, 3-11-82, 5-10-82, 7-4-82, 9-2-82, 11-8-82, 1-10-83, 4-3-83, 7-5-83, 9-5-83, 10-16-83, 12-12-83, 5-8-84, 7-8-84, 12-16-84, 2-7-85, 5-13-85, 6-26-85, 11-3-85, 3-5-86, 6-16-86, Formerly 16J-8.678, Amended 9-7-86, 2-12-87, 9-2-87, 2-18-88, 6-27-88, 2-22-89, 3-23-89, 9-26-89, 7-26-90, 10-30-90, 3-3-91, 9-30-91, 10-7-91, 7-26-92, 3-1-93, 5-11-94, 6-6-96, 2-23-97, 8-7-00, 1-8-04, 12-21-04 (13), 12-21-04 (13), 6-5-05, 5-2-06, 1-1-07, 2-12-07, 1-10-08, 2-18-08, 4-7-08, 5-20-08, 5-10-09, 4-13-11, 3-12-12, 11-25-12, 2-21-13 (12)(f), 2-21-13 (12)-(13), 9-3-13, 1-7-15, 7-1-15, 9-21-15, 11-30-16, 12-28-16, 2-12-17 (12)(s), 2-12-17 (12)(z), 2-12-17 (12)(z), 2-19-17 (12)(l), 2-19-17 (12)(q), 3-2-17 (12)(l), 3-2-17 (12)(z), 3-22-17, 4-2-17 (12)(q), 4-2-17 (12)(z), 4-20-17 (12)(i), 4-20-17 (12)(i), 2-5-18, 3-20-18, 5-30-18, 1-23-19, 2-11-19, 3-20-19, 4-28-19, 7-17-19, 5-27-20, 8-9-20, 4-12-21, 6-1-21, 9-27-21.