- 64E-4.009 Surveys.

Each registrant shall make or cause to be made such radiation protection surveys as may be necessary to comply with Rule 64E-4.009, F.A.C. At intervals not to exceed six months, surveys shall be performed which include but are not limited to:

(1) A determination that all laser protective devices are labeled correctly and functioning within the design specifications and are properly chosen for lasers in use.

(2) A determination that all warning devices are functioning within their design specifications.

(3) A determination that the laser controlled area is properly controlled and posted with accurate warning signs in accordance with Rule 64E-4.008, F.A.C.

(4) A re-evaluation of potential hazards from surfaces which may be associated with Class IIIb and Class IV beam paths.

(5) Additional surveys required to evaluate the laser and collateral radiation hazard incident to the use of lasers.

Rulemaking Authority 501.122(2) FS. Law Implemented 501.122(2)(c), (d) FS. History–New 9-6-84, Formerly 10D-89.122.