- 64E-4.014 Scanning Devices.

Scanning devices shall incorporate a means to turn off the beam or to prevent laser emission in case the beam stops scanning or slows down significantly. In cases where a mirror ball is used with a scanning beam, the limits of subsection 64E-4.013(2), F.A.C., shall be met with the mirror ball stationary; or the mirror ball shall incorporate a means to turn off the beam or to prevent laser emission if the mirror ball stops rotating or slows down significantly such that the limits of subsection 64E-4.013(2) or (3), F.A.C., are exceeded. Any such scan failure safeguard system must have a reaction time fast enough to preclude audience access to levels in excess of Class I.

Rulemaking Authority 501.122(2) FS. Law Implemented 501.122(2)(a), (d) FS. History–New 9-6-84, Formerly 10D-89.131.