64E-5.331 Disposal of Specific Wastes.

(1) A licensee can dispose of the following licensed material without regard to its radioactivity:

(a) 0.05 microcurie (1.85 kBq) or less of hydrogen 3 or carbon 14 per gram of medium used for liquid scintillation counting;

(b) 0.05 microcurie (1.85 kBq) or less of hydrogen 3 or carbon 14 per gram of animal tissue, averaged over the weight of the entire animal.

(c) Any radioactive material which is not a sealed source with a physical half-life of less than 90 days if all of the following are met:

1. Radioactive material to be disposed is held for decay in storage a minimum of 10 half-lives;

2. The radioactive material is monitored at the container surface before disposal as ordinary trash and its radioactivity cannot be distinguished from the background radiation level in a low background radiation area with an appropriate radiation survey instrument set on its most sensitive scale and with no interposed shielding;

3. All radiation labels are removed or obliterated, unless specifically authorized in writing or license condition by the department;

4. Each generator column is separated and monitored individually with all radiation shielding removed to ensure that its contents have decayed to background levels before disposal; and

5. The licensee shall retain a record of each disposal for 3 years. The record shall include:

a. The date of the disposal;

b. The date on which the radioactive material was placed in storage;

c. The radionuclides disposed;

d. The model and serial number of the radiation survey instrument used;

e. The background dose rate;

f. The radiation dose rate measured at the surface of each container; and

g. The name of the individual who performed the disposal.

(2) A licensee shall not dispose of tissue as specified in subsection 64E-5.331(1), F.A.C., in a manner that would permit its use either as food for humans or as animal feed.

(3) The licensee shall maintain records as specified in Rule 64E-5.340, F.A.C.

Specific Authority 404.051, 404.081 FS. Law Implemented 404.022, 404.051(1), (4), 404.081 FS. History–New 1-1-94, Formerly 10D-91.465.