- 64E-5.1412 Source Rack Protection.

If the product to be irradiated moves on a product conveyor system, the source rack and the mechanism which moves the rack must be protected by a barrier or guides to prevent products and product carriers from hitting or touching the rack or mechanism. A collision alarm system on the protective barrier must cause an alarm at the control console notifying the operator that a collision between the barrier and product conveyor has occurred.

Rulemaking Authority 404.051(4) FS. Law Implemented 404.051(1), (5), (6), 404.061, 404.081, 404.141 FS. History–New 8-14-96, Formerly 10D-91.1512.