- 64E-5.1413 Power Failures.

(1) If electrical power at a panoramic irradiator is lost for longer than 10 seconds, the sources must return automatically to the shielded position.

(2) The lock on the door of the radiation room of a panoramic irradiator must not be deactivated by a power failure.

(3) During a power failure, the area around the pool of an irradiator shall not be entered without using an operable and calibrated radiation survey meter.

Rulemaking Authority 404.051(4) FS. Law Implemented 404.051(1), (5), (6), 404.061, 404.081, 404.141 FS. History–New 8-14-96, Formerly 10D-91.1513.