64E-6.001 General.

(1) The provisions of Part I of this rule shall apply to all areas of the state except where specific provisions found in Part II which specifically addresses the Florida Keys, or specific provisions found in Part IV which specifically address performance-based treatment systems, exempt or modify compliance with Part I or Part II requirements. Performance-based treatment systems are intended as an alternative to the systems conforming to the prescriptive standards detailed in Parts I and II of this rule and shall be used only for a single family residence. Designs for performance-based treatment systems allow for the use of alternative and innovative methods, materials, processes, and techniques that reduce the total biological, chemical, hydraulic, organic, nutrient, bacterial and viral discharge to the environment. Where used, the performance-based treatment systems shall be designed, operated, constructed, maintained and used in conformance with Section 381.0065(4)(j), F.S. Part III addresses the registration of septic tank contractors and certification of partnerships and corporations. Part V addresses fees for Parts I, II, III, and IV of this rule.

(2) Except for places of employment meeting the provisions of Chapter 64E-10, F.A.C., buildings used or intended for human occupancy, employment or service to the public and locations where people congregate shall connect toilets and other wastewater generating fixtures to an approved wastewater treatment and disposal system. Also, property or locations where people congregate, are employed, or where property is used by the public for temporary and short periods, such as construction sites, fairs, carnivals, revivals, field locations for agricultural workers, encampments or other use shall be provided with an approved wastewater treatment and disposal system. Establishments with permanent structures shall not rely upon systems designed for temporary use as the primary means of wastewater treatment and disposal.

(3) Use of holding tanks and portable toilets shall be in accordance with the provisions of Rule 64E-6.010, F.A.C.

(4) Except as provided for in Section 381.00655, F.S., any existing and prior approved system which has been placed into use and which remains in satisfactory operating condition shall remain valid for use under the terms of the rule and permit under which it was approved. Alterations that change the conditions under which the system was permitted and approved, sewage characteristics or increase sewage flow will require that the owner, or their authorized representative, apply for and receive reapproval of the system by the DOH county health department, prior to any alteration of the structure, or system. If an applicant requests that the department consider the previous structure’s or establishment’s most recent approved occupancy, the applicant must provide written documentation that the onsite sewage treatment and disposal system was approved by the department for that previous occupancy. An applicant will be required to complete Form DH 4015, 10/97, Application for Onsite Sewage Treatment and Disposal System Construction Permit, herein incorporated by reference, and provide a site plan in accordance with paragraph 64E-6.004(3)(a), F.A.C., to provide information of the site conditions under which the system is currently in use and conditions under which it will be used. The applicant shall have all system tanks pumped by a permitted septage disposal service to determine tank volume based on the actual measurements of the tank. The service pumping the tank shall perform a visual inspection of the tank when the tank is empty to detect any observable defects or leaks in the tank, and shall submit the results to the DOH county health department as part of the application. If a prior approved existing system has been approved by the DOH county health department within the preceding three years, and the system was determined to be in satisfactory operating condition at that time, a new inspection is not required unless there is a record of failure of the system. If it is determined that a new inspection is not required, only the application fee shall be charged for this application and approval. A commercial system out of service for more than one year shall be brought into full compliance with current requirements of this chapter prior to the system being placed into service. If the use of a building is changed or if additions or alterations to a building are made which will increase domestic sewage flow, change sewage characteristics, or compromise the integrity or function of the system, the onsite sewage treatment and disposal system serving such building shall be brought into full compliance with the provisions and requirements of these rules. Proper well setbacks shall be maintained. Prior to any modification of the system, the owner shall apply for and obtain a permit for modification of the system from the county health department in accordance with Rule 64E-6.004, F.A.C. The permit shall be valid for 18 months from the date of issue. Where building construction has commenced, it shall be valid for an additional 90 days. Necessary site investigations and tests shall be performed at the expense of the owner by either an engineer with soils training who is registered in the state of Florida pursuant to Section 471, F.S., registered septic tank contractors, master septic tank contractors, or persons certified under Section 381.0101, F.S., or department personnel for the appropriate fee specified in Section 381.0066, F.S.

(a) For residences, flows shall be calculated using new system criteria for bedrooms and building area, including existing structures and any proposed additions. Table I and footnotes shall apply. For example, a current three bedroom, 1300 square foot home would be able to add building area to have a total of 2250 square feet of building area with no change in their approved system, provided no additional bedrooms are added. No part of the existing structure, or the addition to the structure shall be allowed to cover any part of the system. Non-load bearing structures, such as a concrete patio floor, are allowed to cover the septic tank, provided that access to the tank is provided for maintenance. The structure above the septic tank shall have a minimum opening of 225 square inches at each end of the septic tank for access into the tank. The structure shall not be in direct contact with the tank. A barrier of soil or plastic shall be used between the tank and non-load bearing structure. For those residences that add sewage flow, the system shall be required to be altered to meet the following criteria:

1. The septic tank need not be replaced if it is structurally sound and is within one tank size of the required specifications found in Table II, for the proposed structure. An approved outlet filter shall be installed if one is currently not in place.

2. The county health department shall require the existing drainfield to be increased to current rule drainfield size requirements for the proposed estimated sewage flow using the appropriate soil loading rate and sizing criteria for new systems. Where the existing elevation of the bottom surface of the drainfield is less than 24 inches above the wet season high water table, the bottom of the drainfield shall be maintained at the existing separation or a minimum of 12 inches above the wet season high water table, whichever is greater.

3. Where the bottom of the drainfield is less than 12 inches above the wet season high water table, the drainfield shall be brought into full compliance with all new system standards, as long as it is the intent of the applicant to proceed with the addition to the residence.

4. Any system where the tank needs to be replaced or is replaced as part of a system upgrade shall be brought into full compliance with all new system specifications.

(b) For commercial establishments, the system shall not be required to be altered if domestic sewage flow is not expected to increase by more than 20% of original design flow or require more than one tank size adjustment. A department approved outlet filter device shall be installed. Any commercial system where the tank needs to be replaced shall be brought into full compliance with all new system specifications.

(c) These requirements do not authorize a residence or establishment to exceed the lot flow allowances authorized under paragraph 64E-6.005(7)(c), F.A.C. Establishments that currently exceed lot flow allowances shall not be allowed to increase sewage flow.

(d) Any system which is used to treat and dispose of commercial wastewater shall be brought into full compliance with the provisions and requirements of current rules when any change in sewage flow or characteristics is made.

(e) Repair of the system to repair system standards shall not alter the standards found in this subsection for existing system use or modification.

(f) The installation of a laundry system, a gray water system, a grease interceptor, or additional drainfield as a precautionary measure to prolong system functioning of an existing system is considered a modification to the system. Such installation is not a modification if it is associated with an increase in estimated sewage flow or change in sewage characteristics, if the system is in failure or if the existing system is in non-compliance with the terms of the original permit, in which case it will be considered a new system.

(g) Where the current structure exceeds the design capacity of the existing system, the system shall not be allowed for use with any addition.

(5) The department Procedure for Voluntary Inspection and Assessment of Existing Systems, May, 2000, herein incorporated by reference, shall be applied except in situations pertaining to an increase in sewage flow or change in sewage characteristics, or failure of the system. The inspection is designed to assess the condition of a system at a particular moment in time. The inspection will identify obviously substandard systems, for example systems without drainfields. The inspection is not designed to determine precise code compliance, nor provide information to demonstrate that the system will adequately serve the use to be placed upon it by this or any subsequent owner. Nothing in this section shall be construed to limit the amount of detail an inspector may provide at their professional discretion. Persons allowed to perform work under this section shall be master septic tank contractors, registered septic tank contractors, state-licensed plumbers, and persons certified under Section 381.0101, F.S. Department employees are excluded from performing these evaluations. Aerobic treatment units and performance-based treatment systems shall not be evaluated using this criteria, but shall be evaluated by the approved maintenance entity which maintains the unit or system. Nothing in this section restricts the person having ownership of, control of, or use of an onsite sewage treatment and disposal system from requesting a partial inspection. The inspector shall provide the person requesting the inspection a copy of the department Procedure for Voluntary Inspection and Assessment of Existing Systems and written notice of their right to request an inspection based on part or all of the standards.

(6) Citations issued by the department shall be on Form DH 3146, 11/02, Citation for Violation, Onsite Sewage Programs/ Sanitary Nuisance, hereby incorporated by reference.

(7) All forms incorporated herein may be obtained by contacting the department.

Specific Authority 381.0011(13), 381.006, 381.0065(3)(a), 489.553(3), 489.557(1) FS. Law Implemented 154.01, 381.001(2), 381.0011(4), 381.0012, 381.0025, 381.006(7), 381.0061, 381.0065, 381.0067, 386.041, 489.553 FS. History–New 12-22-82, Amended 2-5-85, Formerly 10D-6.41, Amended 3-17-92, 1-3-95, 5-14-96, 2-13-97, Formerly 10D-6.041, Amended 11-19-97, 2-3-98, 3-22-00, 9-5-00, 5-24-04.