64E-8.003 New Private and Multi-family Water System Construction.

(1) All potable wells serving private water systems and multi-family water systems shall be separated from major contaminant sources per subsection 64E-8.001(4) and subparagraphs 64E-8.002(2)(a), (b)1. and 3., F.A.C., and shall be constructed at least 75 feet from any OSTDS. Paragraph 64E-8.002(2)(c), F.A.C., shall also apply to wells serving private and multi-family systems.

(2) No person shall construct or alter a multi-family water system without approval on Form DH 4093, 8/97; an applicant must complete and submit Form DH 4092B, 9/97, to the county health department with:

(a) Two sets of a site plans drawn to scale and two sets of a construction plan, each drawing must be being a minimum 8.5 × 11 inches and of sufficient clarity for reproduction; and

(b) A $40 processing fee.

(3) Construction approval will be granted if the system meets the criteria in subsections 64E-8.002(4), (5), (6) and 64E-8.003(1), F.A.C.

(4) The applicant shall analyze the water per paragraphs 64E-8.006(2)(d) and (e), F.A.C., and provide a copy of the results to the department. These results shall not exceed the MCL’s in Rule 64E-8.006, F.A.C.

(5) The department shall inspect to determine that the system has been constructed in compliance with the approved plans. Re-inspection requests must be accompanied by a $25 fee.

(6) If the inspection and laboratory results are satisfactory, the department shall authorize operation using Form DH 4093.

Specific Authority 403.862(1)(f), 381.0011(4), (13), 381.006, 381.0062(1), (3)(a) FS. Law Implemented 381.0012, 381.006(1), (2), 381.0061, 381.0062(1), (3), 381.0067, 403.862(1)(f) FS. History–New 1-1-93, Amended 8-20-96, Formerly 10D-4.026, Amended 1-26-98, 1-24-00, 11-13-00, 10-7-02.