64E-8.006 Water Quality Standards and Monitoring.

(1) Samples collected for compliance with this rule, except chlorine residuals, shall be analyzed by a laboratory certified for the contaminant by the department using a department-certified method for analyzing drinking water samples, in accordance with the referenced methods required in Chapter 64E-1, F.A.C., Clinical and Water Testing Laboratories.

(a) Chemical MCL (Maximum Contaminant Level)

1. Lead – 0.015 milligrams per liter (mg/L). One liter first flush sample collected from an indoor tap after water has been in plumbing for at least six hours.

2. Nitrate as N - 10.0 mg/L. Sample shall be representative of the water being consumed.

(b) Microbiological MCL –

1. A total coliform MCL violation occurs when a total coliform test reveals the presence of total coliform bacteria in any compliance and repeat sample.

2. A fecal coliform MCL violation occurs when a fecal coliform or Escherichia coli test reveals the presence of fecal coliform bacteria or E. coli in any compliance or repeat sample, providing the repeat test also reveals the presence of coliform bacteria.

3. The supplier shall collect a repeat sample on the first business day following notification that coliform bacteria were present in the previous sample. Failure to provide a repeat sample to the laboratory by the deadline specified by the department will result in use of the previous sample’s results as if they were the repeat sample’s results.

4. Unless coliform bacteria are present, a result of too numerous to count (TNTC), confluent, or turbid with the absence of gas or acid, is invalid and the supplier shall collect a replacement sample on the next business day after notification of the invalid results.

(2) Monitoring requirements – Initial survey and routine analyses shall be conducted by the supplier for all contaminants per Rule 64E-8.006 and paragraph 64E-8.004(2)(c), F.A.C.

(a) Microbiological survey for newly constructed Limited Use Systems:

1. The supplier shall collect and have analyzed one source water sample per day for five days within 23 consecutive days.

a. Collection of two samples per day is permitted if collected six hours apart and the pump is run at rated capacity fifteen minutes before each collection.

b. No more than one of these five samples and neither of the last two of these samples shall reveal the presence of coliform bacteria.

2. Upon completing system construction and disinfection, the supplier shall clear all residual disinfectant from the system and collect, and have analyzed one satisfactory, remote distribution microbiological sample per day for two consecutive days.

(b) The following monitoring schedule applies to Limited Use Systems:

1. Analysis for the chemicals listed in this section every 36 months for Limited Use Community Systems, or every 60 months for Limited Use Commercial Systems,

a. The department shall require continued annual or more frequent chemical monitoring if analysis results indicate the presence of contaminants which may pose a threat to public health as determined by the department, and

2. Microbiological analysis of a minimum of one remote sample of finished water every calendar quarter.

a. When treatment includes disinfection, a source water microbiological sample must also be collected on the same day as the finished water sample.

b. The department shall increase microbiological monitoring frequency to monthly for one year if the supply or system has a recent history of intermittent, unsatisfactory microbiological analyses.

(c) All system owners shall report analysis results, except for daily chlorine residuals, to the department within five days of receipt.

(d) Additional analyses for contaminants not listed in this rule shall be required for all systems if the well is:

1. Located within a delineated area regulated under the Department of Environmental Protection’s Rule Chapter 62-524, F.A.C., New Potable Water Well Permitting in Delineated Areas, or

2. Within 1000 feet of a contaminated well or a known contaminant source.

(e) Multi-family Water Systems shall be sampled for one distribution microbiological test, a two-consecutive-day source water microbiological test, and tested for the chemicals in this section one time upon completion of construction.

(f) Any chemicals or pathogens detected in concentrations which exceed a published or calculated health advisory level shall be reported to the department and removed by corrective actions taken by the supplier per Rule 64E-8.007, F.A.C.

(3) Laboratory Methods –

(a) Microbiological analyses shall be in accordance with those methods specified in the Code of Federal Regulations (CFR) Parts 141.21(f)(3), (4), and (5), hereby incorporated by reference.

(b) Fees charged to the supplier when samples are submitted by or through the department shall include:

1. Laboratory analysis costs of $10 for each lead and nitrate sample analyzed in the initial survey, for routine monitoring, or for non-public health concerns; and

2. Laboratory analysis costs of $10 for each microbiological sample analyzed for routine monitoring, for a new distribution line clearance, for an initial well survey, or for non-public health concerns.

(4) Department Monitoring – Department staff may collect monitoring compliance samples upon a supplier’s request for a fee of $40 per microbiological sampling site visit, $50 per chemical sampling site visit, or $55 per combined chemical and microbiological sampling site visit, plus any locally mandated fees, laboratory analysis fees, and shipping fees.

(5) Monitoring by Supplier – Rule Chapter 62-524, F.A.C., precludes all persons, except department staff, from sampling water systems for initial compliance with delineated area listed chemical MCL’s within DEP delineated areas. For initial clearance, the department shall charge the supplier $50 for each delineated area chemical sampling site visit, $50 for delineated area clearance processing, and each laboratory analysis fee required for the particular contaminant(s).

Specific Authority 403.862(1)(f), 381.0011(4), (13), 373.309, 381.006, 381.0062(1), (3)(a), 381.0202(3) FS. Law Implemented 381.0012, 381.006(1), (2), 381.0061, 381.0062, 381.0067, 373.309(1)(e)6., 403.862(1)(f), 381.0202(3) FS. History–New 1-1-93, Amended 8-20-96, Formerly 10D-4.029, Amended 1-26-98, 1-24-00, 11-13-00.