- 64E-14.011 Heating.

(1) All private living quarters, and bathrooms of migrant labor camps and residential migrant housing shall be equipped with heat producing devices to maintain a room temperature of 68 degrees Fahrenheit.

(2) Heating appliances, other than electrical, shall be provided with a flue pipe or vent connected to the appliance and discharging to the outside air. Cooking facilities shall not be considered heating equipment.

(3) Automatically operated heat producing equipment shall be provided with controls to cut off the fuel supply upon the failure or interruption of flame or ignition or whenever a predetermined safe temperature or pressure is exceeded. All steam and hot water systems shall be provided with safety devices designed to prevent hazardous pressures and excessive temperatures.

Rulemaking Authority 381.006(5), 381.0086 FS. Law Implemented 381.0086 FS. History–New 1-1-77, Amended 10-7-84, Formerly 10D-25.62, Amended 8-6-92, 11-17-94, Formerly 10D-25.062.