- 69A-2.018 Transportation; Blasting Agents.

(1) When blasting agents are transported in the same vehicle with other explosives, all of the requirements of rules 69A-2.013, 69A-2.014, 69A-2.015, and 69A-2.016, F.A.C., shall be complied with.

(2) Vehicles transporting blasting agents shall only be driven by and be in charge of a driver who is capable, careful, reliable and in possession of a valid motor vehicle operator’s license. Such a person shall also be familiar with the State vehicle and traffic laws.

(3) No sparking metal, sparking metal tools, oils, matches, firearms, acids or other corrosive liquids shall be carried in the bed or body of any vehicle containing blasting agents.

(4) No person shall be permitted to ride upon, drive, load or unload a vehicle containing blasting agents while smoking or under the influence of intoxicants or narcotics.

(5) It is prohibited for any person to transport or carry any blasting agents upon any public vehicle carrying passengers for hire.

(6) Vehicles transporting blasting agents shall be in safe operating condition at all times.

(7) When offering blasting agents for transportation on public highways the packaging, marking and labeling of containers of blasting agents shall comply with the requirements of the Interstate Commerce Commission regulations.

(8) Vehicles used for transporting blasting agents on public highways shall be marked and placarded in accordance with subsection 69A-2.014(2), F.A.C.

Rulemaking Authority 552.13 FS. Law Implemented 552.094, 552.12, 552.13 FS. History–New 6-25-66, Repromulgated 12-24-74, Formerly 4A-2.18, Amended 11-14-99, Formerly 4A-2.018.