69A-58.005 Serious Life Safety Hazards.

(1) Serious life safety hazards as set forth in Section 1013.12, F.S., and in paragraph (b), below, require prompt corrective action by the board or withdrawal of the educational or ancillary plants from use until corrected.

(2)(a) Serious life safety hazards include:

1. Non-functional fire alarm systems;

2. Non-functional fire sprinkler system;

3. Doors with padlocks or other locks or devices which preclude egress at any time;

4. Inadequate exits;

5. Hazardous electrical system conditions;

6. Potential structural failure;

7. Storage conditions that create a fire hazard.

(b) Other conditions may be identified to the division by the authority having jurisdiction for designation as a serious life safety hazard, including:

1. Placement of functional smoke and heat detectors in a manner not consistent with NFPA 72, the edition as adopted in Rule 69A-3.012, F.A.C.;

2. Inaccessible or expired fire extinguishers; and

3. Fire doors with doorstops or wedges holding them open.

(c) The criteria to be used by the division to determine whether such other condition shall be designated as a serious life safety hazard shall be either:

1. Those conditions located in Section 6.2, NFPA 101, the edition as adopted in Rule 69A-3.012, F.A.C., to wit:

a. The relative danger to the start and spread of fire,

b. The danger of smoke or gases generated, and

c. The danger of explosion or other occurrence potentially endangering the lives and safety of the occupants of the building or structure. Hazard of contents shall be determined by the authority having jurisdiction on the basis of the character of the contents and the processes or operations conducted in the building or structure. For the purposes of these rules, where different degrees or hazard of contents exist in different parts of a building or structure, the most hazardous shall govern the classification unless hazardous areas are separated or protected as specified in Section 8.4 and the applicable sections of Chapters 11 through 42 of NFPA 101, the edition as adopted in Rule 69A-3.012, F.A.C.; or

2. The criteria located in NFPA 1, the Fire Prevention Code, Section 2-28.1, the edition as adopted in Rule 69A-3.012, F.A.C., for hazardous occupancies, to wit, the total amount of Class A combustibles and Class B flammables present, in storage, production, use, finished product, or combination thereof, is over and above those expected in occupancies classed as ordinary (moderate) hazard. Those occupancies could consist of woodworking, vehicle repair, cooking areas, product displays, and storage and manufacturing processes such as painting and coating, including flammable liquid handling. Also included is warehousing of or in-process storage of other than Class I and Class II commodities as defined by NFPA 13, Standard for the Installation of Sprinkler Systems, Section 10:1-5.3, the edition as adopted in Rule 69A-3.012, F.A.C.

Specific Authority 1013.12 FS. Law Implemented 1013.12 FS. History–New 2-18-03, Formerly 4A-58.005.