- 62-730.160 Standards Applicable to Generators of Hazardous Waste.

(1) The Department adopts by reference 40 CFR Part 262 revised as of July 1, 2021 http://www.flrules.org/Gateway/reference.asp?No=Ref-15288, and as amended in the 86 Federal Register dated October 1, 2021 (54381-54386) http://www.flrules.org/Gateway/reference.asp?No=Ref-15292, including the Appendix, with the exception of 40 CFR 262.16(c) and 262.10(k): and the optional addition of “267” to 262.10(a)(2), 262.10(f), 262.10(g)(2), 262.11(e), 262.15(a), 262.16, 262.17, and 262.41(b).

(2) Large Quantity Generators, as defined in 40 C.F.R Part 260.10 shall submit biennial reports required by 40 C.F.R. 262.41 [as adopted in subsection 62-730.160(1), F.A.C.] to the Department on Form 62-730.900(8) (“RCRA Subtitle C Site Identification and Hazardous Waste Report”) as adopted in subsection 62-730.150(3), F.A.C. Rule 62-730.900, F.A.C., contains information on obtaining a copy of this form.

(3) Generators of hazardous waste who accumulate hazardous waste on-site under 40 CFR 262.16, and 262.17 [as adopted in subsection 62-730.160(1), F.A.C.], shall maintain written documentation of the inspections required under 40 C.F.R 262.16 and 262.17 [as adopted in subsection 62-730.160(1), F.A.C.]. The generator shall keep the written documentation of the inspections under this section for at least three years from the date of the inspection. At a minimum, this documentation shall include the date and time of the inspection, the legibly printed name of the inspector, the number of containers, the condition of the containers, a notation of the observations made, and the date and nature of any repairs or other remedial actions.

(4) Generators required to inspect containers under 40 C.F.R. 262.16(b)(2)(iv) and 262.17(a)(1)(v), [as adopted in subsection 62-730.160(1), F.A.C.] shall maintain adequate aisle space between containers of hazardous waste to allow for inspection of the condition and labels of the individual containers.

(5) 40 C.F.R. 262.14(a)(5)(iii) [as adopted in subsection 62-730.160(1), F.A.C.] shall refer to hazardous waste management programs approved by EPA.

Rulemaking Authority 403.704, 403.721, 403.8055 FS. Law Implemented 403.704, 403.72, 403.721 FS. History–New 5-19-82, Amended 5-20-82, 3-31-83, 1-5-84, 2-2-84, 8-24-84, 7-5-85, 10-3-85, Formerly 17-30.16, Amended 9-19-86, 10-31-86, 3-31-87, 5-26-87, 6-28-88, Formerly 17-30.160, Amended 1-25-89, 8-13-90, 9-10-91, 10-14-92, 10-7-93, Formerly 17-730.160, Amended 1-5-95, 9-7-95, 4-30-97, 8-19-98, 2-4-00, 12-20-00, 8-1-02, 10-1-04, 1-29-06, 4-6-06, 5-1-07, 4-25-08, 5-8-09, 6-8-10, 10-12-11, 6-29-12, 4-23-13, 6-17-15, 4-5-16, 6-18-18, 6-26-19, 4-21-23.