Notice of Bid/Request for Proposal

DEPARTMENT OF MILITARY AFFAIRS

Project 224121 Invitation to Bid

STATE OF FLORIDA, DEPARTMENT OF MILITARY AFFAIRS

PUBLIC ANNOUNCEMENT

INVITATION TO BID

Issuing Agency:  Department of Military Affairs

Project Number and Name:  224121 Camp Blanding Street Paving

Project Location:  Camp Blanding Joint Training Center, 5629 FL-16 W, Starke, FL

General Statement of Work: Licensed contractor shall furnish all labor, supervision, materials, equipment, bonding and insurance, and permits (if needed) for the following:

Repave St. Augustine Street, Lakeland Street, Daytona Beach Street, Gainesville Street, Ruston Street including parking, Cherry Street, Palmetto Street, North Miami Street.  All roads will be milled at 1.5” minimum, swept and tacked prior to installing a leveling course, and final paving to consist of 1.5” of 12.5 mix per FDOT Standards.  There shall be a 2% fall from the centerline to edges of road.  Clean all roadside ditches from culvert to culvert to maintain a natural flow.  Repair any concrete lined ditches damaged in the cleaning process.  Remove all grass from roadway edges and driveways.  Replace all culverts like for like and add concrete mitered ends. Sweep new paving.  Ensure minimum 30’ radius at intersections.  Sod areas where grass is stripped or damaged.  Re-stripe with center line and edge markings (no reflectors).  Haul all removed dirt to Camp Blanding stockpile on post.  Dispose of removed culvers through Camp Blanding recycle program. Remove all other construction debris off site. For all streets with existing utility cuts, Contractor to ensure applicable testing and monitoring samples are obtained utilizing a vendor of their choice, with minimum 98% compaction required prior to paving. Seal coat and restripe Avenue B Parking Lots, approximately 237,437 sf.  Site visit attendance will be mandatory prior to bidding.

Estimated Construction Cost (range):  Level 5 $1,000,000-$1,100,000

Type of Contractor:  Asphalt

Date of VIP Posting: May  https://vendor.myfloridamarketplace.com/

Site Visit Date:  As stated on the Vendor Information Portal

Bid Opening Date:  As stated on the Vendor Information Portal (late bids will not be accepted)

Point Of Contact:  Department of Military Affairs, CFMO Contract Management Office at (904)823-0285 or e-mail ng.fl.flarng.list.cfmo-contracting@army.mil.

FOR COMPLETE INFORMATION AND SUBMISSION REQUIREMENTS PLEASE REFER TO THE MYFLORIDAMARKETPLACE VENDOR INFORMATION PORTAL AT:

https://vendor.myfloridamarketplace.com/