Notice of Proposed Rule

DEPARTMENT OF ENVIRONMENTAL PROTECTION

RULE NOS.:	RULE TITLES:

62-6.001	General

62-6.002	Definitions

62-6.003	Permits

62-6.024	Private Provider Inspectors

62-6.027	Permits and Inspections

62-6.030	Fees

PURPOSE AND EFFECT: The proposed revisions to Chapter 62-6, Florida Administrative Code, (F.A.C.),will update compliance standards, and definitions in rules 62-6.001, 62-6.002, 62-6.003, 62-6.027, and 62-6.030, F.A.C., and create a new rule 62-6.024, F.A.C., relating to enforcement and auditing for private provider inspections of the onsite sewage treatment and disposal systems, as directed by Chapter 2022-105, Laws of Florida.

SUMMARY: The Department is proposing amendments to Chapter 62-6, F.A.C., Standards for Onsite Sewage Treatment and Disposal Systems, to update regulatory requirements due to these statutory changes. The Department proposed new rule 62-6.024, F.A.C., to adopt rules for the enforcement and auditing requirements pursuant to the private provider inspections of an OSTDS and proposed amendments to rules 62-6.001, 62-6.002, 62-6.003, 62-6.027, and 62-6.030, F.A.C., to update definitions, incorporate private provider inspections, clarify engineer certification requirements, update the fee structure for the review and processing of construction inspection documentation from the private provider inspectors, and clarify examination and continuing education related fees.

SUMMARY OF STATEMENT OF ESTIMATED REGULATORY COSTS AND LEGISLATIVE RATIFICATION:

The Agency has determined that this will not have an adverse impact on small business or likely increase directly or indirectly regulatory costs in excess of $200,000 in the aggregate within one year after the implementation of the rule. A SERC has not been prepared by the Agency.

The Agency has determined that the proposed rule is not expected to require legislative ratification based on the statement of estimated regulatory costs or if no SERC is required, the information expressly relied upon and described herein: Based on the Department’s economic review, neither a SERC nor legislative ratification is required because the adoption of the proposed rule does not increase regulatory costs directly or indirectly to the public.

Any person who wishes to provide information regarding a statement of estimated regulatory costs, or provide a proposal for a lower cost regulatory alternative must do so in writing within 21 days of this notice.

RULEMAKING AUTHORITY: 381.0065(3)(a), 381.0065(3)(c), 381.0065(8)(h), 381.0066, 403.061, 489.553(3) and 489.557(1) F.S.

LAW IMPLEMENTED: 381.0065, 381.0066, 381.0067, Part I 386, 403.121, Part III 489, F.S.

IF REQUESTED WITHIN 21 DAYS OF THE DATE OF THIS NOTICE, A HEARING WILL BE SCHEDULED AND ANNOUNCED IN THE FAR.

THE PERSON TO BE CONTACTED REGARDING THE PROPOSED RULE IS: Eb Roeder, Division of Water Resource Management, MS 3596, 2600 Blair Stone Road, Tallahassee, Florida 32399-2400, (850)245-8402 or by email at Eb.Roeder@FloridaDEP.gov.

Public participation is solicited without regard to race, color, religion, sex, pregnancy, national origin, age, handicap, or marital status. Persons who require special accommodations under the American with Disabilities Act (ADA) or persons who require translation services (free of charge) are asked to contact DEP’s Limited English Proficiency Coordinator at (850)245-2118 or LEP@FloridaDEP.gov at least ten (10) days before the hearing. If you have a hearing or speech impairment, please contact the agency using the Florida Relay Service, (800)955-8771 (TDD) or (800)955-8770 (voice).

THE FULL TEXT OF THE PROPOSED RULE IS:

62-6.001 General.

(1) The provisions of Part I (Rules 62-6.001 through 62-6.016, F.A.C.) of this chapter apply to all areas of the state except where specific provisions in Part II (Rules 62-6.017 through 62-6.0182, F.A.C.), addressing the Florida Keys, or specific provisions in Part IV (Rules 62-6.025 through 62-6.0295, F.A.C.), addressing performance-based treatment systems, exempt or modify compliance with Part I. Part III (Rules 62-6.019 through 62-6.024 62-6.023, F.A.C.) addresses the qualifications and registration of septic tank contractors and private provider inspectors, and the authorization of partnerships and corporations. Part V (Rule 62-6.030, F.A.C.) addresses fees for services throughout the chapter. The provisions of this chapter must be used in conjunction with Chapter 381, Chapter 403, and Part III of Chapter 489, F.S.

(2) through (5) No change.

(6) In accordance with subsection 381.0065(4), F.S., a municipality or political subdivision of the state may not issue a building or plumbing permit for any building that requires the use of an onsite sewage treatment and disposal system unless the owner or builder has received a construction permit for such system from the Department. Citations issued by the Department shall be on Form DEP 3146, 11/02, Citation for Violation, Onsite Sewage Program/Sanitary Nuisance, hereby incorporated by reference.

(7) and (8) No change.

Rulemaking Authority 381.0065(3)(a), 381.0065(4)(e), 381.0065(8)(h), 403.061, 489.553(3), 489.557(1) FS. Law Implemented 381.0065, 381.0067, 386.041, 489.553 FS. History–New 12-22-82, Amended 2-5-85, Formerly 10D-6.41, Amended 3-17-92, 1-3-95, 5-14-96, 2-13-97, Formerly 10D-6.041, Amended 11-19-97, 2-3-98, 3-22-00, 9-5-00, 5-24-04, 11-26-06, 6-25-09, 4-28-10, 7-16-13, Formerly 64E-6.001, Amended 6-28-23, x-xx-xx.

Editorial Note: Ratified by Ch. 2023-308, LOF.

62-6.002 Definitions.

For the purposes of this chapter, the following words and phrases shall have the following meanings indicated:

(1) No change.

(2) Accelerated certification training – a training program established by the Department regarding general onsite sewage treatment and disposal system regulations, including permitting, and inspection topics such as site evaluation, system design, system material and construction standards, regulatory requirements, and soil morphology. The training program requires a minimum of 24 hours of pre-certification coursework which is presented in several parts. Satisfactory completion of the accelerated certification training requires an individual examination and a passing score for each part of the training program.

(2) through (5) renumbered (3) through (6) No change.

(7)(6) Approved or approval – where a permitted an onsite sewage treatment and disposal system is constructed and installed subject to in compliance with the standards and requirements of this chapter, and the following types of approval: which has received final installation approval. “Approved” installation does not imply that a system will perform satisfactorily for a specific period of time.

(a) Construction approval – the Department or a private provider inspector will grant construction approval following a satisfactory construction inspection of the construction and installation of the system. The Department or a private provider inspector, or authorized representative of the private provider inspector, must conduct the construction inspection. The construction approval of the system or a component of the system is only a partial or preliminary approval based on the compliant status of the system at the time of inspection.

(b) Final installation approval – the Department will grant final installation approval after the complete construction and installation of the system and after confirmation that the system complies with the statutory requirements and this chapter’s applicable rules, standards, and requirements. “Approved” or “approval” of a final installation does not imply that a system will perform satisfactorily for a specific period of time.

(8)(7) Approved maintenance entity – any person or business entity which has been issued a written permit by the Department to provide maintenance services associated with approved onsite aerobic treatment units or performance-based treatment systems.

(8) through (13) renumbered (9) through (14) No change.

(15) Construction inspection – critical assessment and evaluation of the construction, installation, modification, abandonment, or repair of an onsite sewage treatment and disposal system performed by an individual qualified under Florida statute or rule to perform the inspection. Each construction inspection does not extend beyond one calendar day.

(14) through (30) renumbered (16) through (32) No change.

(33) Installer – a registered septic tank contractor, master septic tank contractor, contractor as defined by paragraph 489.105(3)(m), F.S., or a person who is under the supervision and control of such contractors during the construction, installation, repair, modification, or abandonment of an onsite sewage treatment and disposal system.

(31) through (44) renumbered (34) through (47) No change.

(48) Private provider inspector – a person who meets the requirements of subsection 381.0065(8), F.S., who is hired by the owner of the system, or by a contractor with the owner’s written authorization, to perform construction inspections pursuant to the applicable regulatory requirements for an onsite sewage treatment and disposal system inspection.

(45) through (59) renumbered (49) through (63) No change.

Rulemaking Authority 381.0064, 381.0065(3)(a), 381.0065(3)(c), 381.0065(8)(h), FS. Law Implemented 381.0064, 381.0065 FS. History–New 12-22-82, Amended 2-5-85, Formerly 10D-6.42, Amended 3-17-92, 1-3-95, Formerly 10D-6.042, Amended 11-19-97, 3-22-00, 11-26-06, 7-16-13, Formerly 64E-6.002, Amended .

62-6.003 Permits and Inspections.

(1) System Construction Permit. – A person may not construct, repair, modify, or abandon an onsite sewage treatment and disposal system without first obtaining a system construction permit approved in accordance with this chapter. The Department issues the system No portion of an onsite sewage treatment and disposal system shall be installed, repaired, altered, modified, abandoned or replaced until a construction permit has been issued on Form DEP 4016, effective date xx-xx-xxxx 08/09, Construction Permit and Inspection, herein adopted and incorporated by reference at https://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX. Copies of this document are available as provided in subsection (7) below. If building construction has commenced, the system construction permit shall be valid for an additional 90 days beyond the eighteen month expiration date. A fee shall not be charged for a repair permit issued within 12 months from the date of final authorization of the onsite sewage treatment and disposal system. If a construction or repair permit for an onsite sewage treatment and disposal system is transferred to another person the date of the construction or repair permit shall not be amended, but shall run from the date of original issuance prior to the transfer. Servicing or replacing with like kind mechanical or electrical parts of an approved onsite sewage treatment and disposal system; pumping of septage from a system; or making minor structural corrections to a tank, or distribution box, does not constitute a repair.

(a) Amended System Construction Permit. The Department shall only require the permittee to conform to new or additional permit conditions for good cause and after notice and, if requested, an administrative hearing pursuant to section 120.57, F.S. The Department shall allow the permittee time to conform to the new or additional conditions by extending the expiration date of any amended system construction permit by 180 days and any amended repair system construction permit by 90 days from the prior expiration date.

(b) For the purpose of this paragraph, good cause shall include any of the following:

1. A showing of any change or difference in the environment, including site conditions, or surrounding conditions that requires a revision to conform to applicable requirements pursuant to this chapter.

2. Adoption or revision of statutes, rules, or standards, which require the revision of a permit condition for compliance.

3. A showing that there are material and substantial alterations or additions to the permitted onsite sewage treatment and disposal system, which occurred after permit issuance which justify the application of permit conditions that are different in or absent from the existing permit.

4. The Department’s receipt of new information which was not available at the time of permit issuance and would have justified the application of different permit conditions at the time of issuance.

5. A showing that there is a technical mistake in a permit condition which needs to be corrected.

(c) If amendments to a system construction permit are made at an applicant’s request, the applicant must pay a permit application amendment fee.

(d) If building construction has commenced, the system construction permit shall be valid for an additional 90 days beyond the eighteen month expiration date. A fee shall not be charged for a repair system construction permit issued within 12 months from the date of final installation approval of the onsite sewage treatment and disposal system.

(e) If a system construction permit for an onsite sewage treatment and disposal system is transferred to another person the date of the system construction permit shall not be amended, but shall run from the date of original issuance prior to the transfer. Rule 62-6.002, F.A.C., defines what is and what is not considered a repair. All repairs require a system construction permit.

(2) Construction System iInspection. Before sealing manhole covers or covering with fill material, earth and before placing a system into service, a person installing, repairing, modifying, replacing, abandoning, or constructing any portion of an onsite sewage treatment and disposal system must shall notify the Department or an authorized private provider inspector to schedule and complete a construction inspection. The construction inspection must meet the following requirements: of the completion of the construction activities and shall have the system inspected by the Department for compliance with the requirements of this chapter, except as noted in subsection 62-6.003(3), F.A.C., for repair installations.

(a) The construction inspection must be performed onsite in person by an individual qualified to perform an inspection of the system in accordance with statutory and rule requirements.

(b) The construction inspection results must be recorded on Part 2 of Form DEP 4016, herein adopted and incorporated by reference in subsection 62-6.003(1), F.A.C. Copies of this document are available as provided in subsection (7) below. A separate inspection form is required for each site visit to assess and record conditions of the system construction at the time of inspection.

(c) At each construction inspection, the inspector must assess and record all items listed on the inspection form and must document the observations and status on the day and time of the inspection. A construction inspection is not complete unless, during the inspection, each item on the inspection form is observed and the condition of the system is recorded.

(d) The construction inspection must confirm compliance with the appropriate statutory and regulatory requirements, this chapter and the information on the system construction permit.

(e) The inspector must document the inspection with photographs of the system being inspected, which shall include information about time and location of the photograph through electronic metadata or included in the notes of the report. A minimum of one photograph should clearly depict the overall layout of the system in relation to the associated structure, including the system's placement and orientation. Such photographs must show an overview of the system installation relative to the structure, the number of drainlines, and extent of drainfield. Any components or aspects of the installation that do not meet regulatory standards must be documented with detailed photographs highlighting the specific areas of concern. Photographs must be in an acceptable digital format (JPEG or PNG) and submitted electronically along with the inspection report.

(f) All construction inspections for a system construction permit must be performed before the system construction permit expires, including any authorized extension. If the construction inspections are not completed prior to the system construction permit expiration, a construction permit application must be resubmitted for review and issuance from the Department on Form DEP 4016, herein adopted and incorporated by reference in subsection 62-6.003(1), F.A.C., which is available as provided in subsection (7) below.

(g)(a) If the observed portion of the system construction is approved after an inspection by the Department or private provider inspector, the Department or private provider inspector must notify shall issue a “Construction Approval” notice to the installer of the inspection results recorded on the inspection form, Part 2 of Form DEP 4016, herein adopted and incorporated by reference in subsection 62-6.003(1), F.A.C., by providing a copy of all the inspection forms involved in inspecting the system. Part 2 of Form DEP 4016 is available as provided in subsection (7) below. The inspection form, Part 2 of Form DEP 4016, must record the items that are in compliance, the items that are not applicable for the particular system construction permit involved, the items that were unobserved and need to be addressed before a final installation approval, and specify which inspection items were out of compliance and must be addressed before a final installation approval will be granted by the Department.

(h) When the Department or private provider inspector disapproves the inspected portion of the system construction due to noncompliance, the inspector must record the disapproval and notify the installer of the inspection results by providing a copy of the disapproved inspection report, documented on the inspection form (Part 2 of Form DEP 4016), which is adopted and incorporated by reference in subsection 62-6.003(1), F.A.C., and is available as outlined in subsection (7) below. The disapproved inspection report on Part 2 of Form DEP 4016 must list the items in compliance, the items not applicable to the specific system construction permit, the unobserved items requiring resolution before final installation approval, and the inspection items found which are out of compliance. The report must also specify the corrective actions needed for final installation approval by the Department. Upon disapproval, the following actions are required to address the noncompliant items:

1. (b) If the system installation does not pass the construction inspection on any type of system installation, Tthe

installer shall make all required corrections and notify the Department or private provider inspector conducting the inspection of the completion of the work prior to the required reinspection of the system. A reinspection fee shall be charged to the installer for each additional Department inspection conducted until the final installation approval is issued by the Department leading up to construction approval.

2. If the inspection records identify a need for a permit amendment per subsection 62-6.003(1), F.A.C., the system construction inspection must be disapproved. The Department will notify the applicant or authorized agent listed on Form DEP 4016, herein adopted and incorporated by reference in subsection 62-6.003(1), F.A.C., when the Department re-issues or denies the amended system construction permit.

(i)(c) The Department will not grant fFinal installation approval shall not be granted until the Department has confirmed that all requirements in statute, requirements of this chapter, and the system construction permit are met, including confirmation that the building construction and lot grading comply with the plans and specifications in the permit as issued are in compliance with plans and specifications submitted with the permit application. The Department will issue a final installation approval notice in writing to the applicant or authorized agent listed on Form DEP 4016, herein adopted and incorporated by reference in subsection 62-6.003(1), F.A.C., by providing a copy of the Department’s final installation approval recorded on Part 2 of Form DEP 4016. The following are required prior to final installation approval:

1. If subsection 62-6.004(4), F.A.C., and paragraph 62-6.009(7)(a), F.A.C., require the system to be designed by a Florida-licensed engineer, the Department requires the design engineer or their designee, who must also be a Florida-licensed engineer, to inspect the installed system and certify its compliance with the approved design and installation requirements. The certification must state: “I certify that the engineering features of this onsite sewage treatment and disposal system as installed have been examined by me and found to comply with all specifications contained in the engineering design that was the basis for issuance of the construction permit. I certify that the required components are installed for the system to function as permitted and designed. I certify that there is reasonable assurance, in my professional judgment, that the system, when properly operated and maintained, will comply with all applicable statutory requirements and rules of the Department.” Single-family residences are excluded from this requirement; however, performance-based treatment systems must comply with subsection 62-6.027(5), F.A.C., for all installations. The design engineer must approve all changes to the engineering specifications. In addition, if the system was designed by an engineer, who shall be licensed in the State of Florida, the Department shall require the design engineer or the design engineer’s designee, who shall be a licensed engineer, to certify that the installed system complies with the approved design and installation requirements. Single family residences are excluded from this requirement, however, all changes to the engineering specifications shall be approved by the design engineer.

2. A reinspection fee is required for each system construction inspection performed by the Department prior to the Department granting final installation approval. If additional site visits after the construction approval inspection are necessary to establish the compliance of the building construction and lot grading, or to establish the compliance with any provision of this chapter, a reinspection fee shall be charged to the permit applicant for each inspection of the building and site leading to the final installation approval.

3. When If an operating permit is required for the onsite sewage treatment and disposal system, the final installation approval shall not be granted until the operating permit application, a copy of any maintenance agreement required pursuant to this chapter, and fee have been received by the Department.

4. The written notice by the property owner that confirms the execution and recording of the notice in the public property records at the county courthouse which informs subsequent property owners as required by paragraphs 62-6.009(7)(g) and 62-6.027(5)(d), F.A.C., or a recorded easement complying with these requirements.

5. Pumpout receipts for abandonments of tanks as required by paragraph 62-6.011(2)(b), F.A.C.

6. Other documentation, bills of lading or invoices required by rule, permit or product approval.

(j)(d) Where an establishment is serviced by an onsite sewage treatment and disposal system, subsection Section 381.0065(4), F.S., governs shall govern when occupancy of a building can be allowed. The Department must first authorize and issue a final installation approval notice before a system can be placed into service or a building occupied. “Approved” or “approval” of installation does not imply that a system will perform satisfactorily for a specific period of time.

(e) Systems which are required to have an annual or biennial operating permit and the structures which they serve shall be inspected by the Department at least once per year during the term of the permit to determine compliance with the terms of the operating permit.

(3) Construction Inspection by a Private Provider Inspector. Repair Inspections – A private provider inspector must physically inspect and evaluate the installation of an onsite sewage treatment and disposal system repair shall be inspected by the Department or a master septic tank contractor to determine compliance with statutory requirements, this chapter, and the system construction permit standards prior to final covering of the system. This responsibility begins when the inspector receives authorization from the owner or an authorized contractor under paragraph 381.0065(8)(d), F.S., and notice from the installer that the system is ready for inspection. The private provider inspector must also obtain a complete copy of the approved system construction permit and applicable application documents. All construction iInspections must shall comply with the requirements of subsection 62-6.003(2), F.A.C., and the following:

(a) Authorization requirement. The owner’s written acknowledgement electing to use a private provider inspector must be completed using Form DEP 4015A, effective date xx-xx-xxxx, Property Owner’s Notice Authorizing Private Provider Inspector, herein adopted and incorporated by reference at https://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX. Copies of this document are available as provided in subsection (7) below. An owner or an authorized contractor’s use of a private provider inspector is subject to the following requirements: A master septic tank contractor may, at their option, cover a system repair when the following conditions are met:

1. The owner or an authorized contractor or their agent must submit Form DEP 4015A to master septic tank contractor has requested an inspection from the Department at the time of the permit application or by 2 p.m. local time, two business days before the first scheduled inspection by the Department. Form DEP 4015A, including the owner’s written acknowledgment, must be signed, dated, and submitted to the Department prior to the first inspection performed by the private provider inspector. The private provider inspector must confirm that Form DEP 4015A is effective and timely submitted to during the normal duty day before the date and time the repair will be ready for inspection. Inspections must be scheduled during normal inspection hours and in conjunction with the inspection schedule of the Department.

2. If an owner or authorized contractor makes any changes to the listed private provider inspector or the service to be performed by the private provider inspector, or requests that the Department perform the system construction inspection, the owner or authorized contractor must update and submit to the Department an amended Form DEP 4015A within one business day after the change. At the date and time specified for inspection, the Department is not on site to conduct an inspection within 30 minutes of the scheduled time. If the Department is on site to conduct the inspection and the system is not ready for inspection within 30 minutes after the scheduled time, a reinspection shall be requested. A reinspection fee shall be charged. Contractors shall cancel or reschedule inspections not later than two hours prior to the scheduled time. In such cases, no reinspection fee shall be charged.

3. The master septic tank contractor is physically on site and conducts the inspection.

(b) Submittal of inspection reports and fees. The master septic tank contractor shall document the inspection of Form DEP 4016, and fax or hand deliver the form to the Department by the next normal duty day following the inspection.

1. The private provider inspector must complete an inspection report on Part 2 of Form DEP 4016, herein adopted and incorporated by reference in subsection 62-6.003(1), F.A.C., for each construction inspection and provide all completed inspection reports and records to the owner or authorized contractor and the Department by end of the seventh calendar day after completion of each construction inspection.

2. Private provider inspectors must submit their inspection reports electronically through the Department's specified portal. The Department cannot accept private provider inspections performed by individuals who do not meet the requirements of subsection 381.0065(8), F.S.

3. Private provider inspections must be conducted by a private provider inspector meeting the requirements of subsection 381.0065(8), F.S.

4. All reports and records must bear the written or electronic signature of the private provider inspector who performed the inspection. If subsequent inspections are performed by a different private provider inspector, each private provider inspector must obtain all previous construction inspection reports and records from the owner or authorized contractor for auditing purposes by the Department and for use in completion of the system construction inspection. All inspection reports and records submitted by a private provider inspector qualified pursuant to subparagraph 381.0065(8)(c)4., F.S., must also be signed, dated, and sealed by the engineer supervising that private provider inspector.

5. Payment of an inspection report review fee is required for the review and processing of each private provider inspector report that results in a construction approval.

(c) Documents required for final installation approval from the Department. The owner, or authorized contractor, or their agent, and the private provider inspector must submit the documentation required by paragraph 62-6.003(2)(i), F.A.C., as applicable, for Department review. The Department shall not grant final installation approval until the Department reviews the documentation submitted and confirms compliance with the requirements of this chapter and the system construction permit. A master septic tank contractor shall not cover a system repair when the Department has performed an inspection and has notified the contractor of violations. Any system that has been inspected by the Department and found to be in violation of construction standards of this rule, must receive a reinspection from the Department before the system may be covered. A reinspection fee shall be charged for each reinspection leading to final approval.

(d) The private provider inspector must retain all records for each inspection and all supporting documentation for a minimum of three years from the date of the inspection. These documents must be maintained in hardcopy or electronic format and provided to the Department or property owner upon request. The Department shall issue a “final approval” of the system repair based on the master septic tank contractor’s inspection.

(e) Nothing herein prevents the Department from inspecting a system inspected by a master septic tank contractor. No inspection is final until approved by the Department.

(4) No change.

(5) Operating Ppermits. – No business or facility shall occupy a building served by an onsite sewage treatment and disposal system if the building is located in an area zoned or used for industrial or manufacturing purposes or its equivalent; or where a business will generate commercial sewage waste; and no structure shall be occupied where an aerobic treatment unit or performance-based treatment system is used, until an “Application for Onsite Sewage Treatment and Disposal System Operating Permit” and appropriate fee has been received and approved by the Department. Applications shall be submitted using Form DEP 4081, effective date xx-xx-xxxx, 10/96 “Application for Onsite Sewage Treatment and Disposal System Operating Permit,” is herein adopted and incorporated by reference at https://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX.

(a) Property owners or their authorized agents are required to obtain an annual operating permit for systems located in an area zoned or used for industrial or manufacturing purposes or its equivalent or where a business will generate commercial sewage waste, subject to all of the following: The permit shall designate the person or entity responsible for the operation and maintenance of the system; the type of activity proposed on the site; persons or businesses which will use the system; equipment and types and quantities of chemical compounds which will be used by the building occupants which are likely to be discharged into the onsite sewage treatment and disposal system.

1. The annual operating permit must designate:

a. The person or entity responsible for the operation and maintenance of the system.

b. The type of activity proposed on the site.

c. Persons or businesses which will use the system.

d. Equipment and types and quantities of chemical compounds which will be used by the building occupants which are likely to be discharged into the onsite sewage treatment and disposal system.

2. At a minimum, the owner or person responsible for maintenance of the system must shall test, or cause to be tested, the onsite sewage treatment and disposal system effluent in a qualitative and quantitative manner for any chemical compounds associated with the particular industrial or manufacturing operations conducted in that establishment, as directed by the Department. The frequency of testing shall be specified on the annual operating permit.

(b) Operating permits are not transferable. If the owner of the system remains the same but the tenancy of the building changes, Form DEP 4081A, effective date xx-xx-xxxx, a Business Survey, Form DEP 4081A, 10/96, herein adopted and incorporated by reference at https://www.flrules.org/Gateway/reference.asp?No=Ref-XXXXX must be completed and submitted to the Department for review. Changes in building occupancy shall be reviewed per subsection Section 381.0065(4), F.S.

(c) Property owners or their authorized agents are required to Maintenance entities contracting to service aerobic treatment systems and performance-based treatment systems shall obtain a biennial operating permit from the Department for aerobic treatment systems and performance-based treatment systems the system. Persons operating an aerobic treatment unit or performance-based treatment system shall permit Department personnel right of entry to the property during normal working hours to allow for effluent sampling or evaluating the general state of repair or function of the system. Persons required to obtain an annual operating permit for an onsite sewage treatment and disposal system in an industrial or manufacturing zone or its equivalent, or where the system receives commercial sewage, shall not be required to obtain another operating permit for an aerobic treatment unit or performance-based treatment system at that site. Performance-based treatment systems that also include an aerobic treatment unit require only one biennial operating permit for the system.

(d) The Department shall inspect onsite sewage treatment and disposal systems which are required to have an annual or biennial operating permit and the structures which they serve at least once per year during the term of the permit to determine compliance with the terms of the operating permit.

(6) Expired Permits. – Any new construction, repair, or modification permit issued by the Department, that has received construction approval within the previous five years but has not received final installation approval may be approved provided all of the following conditions are met:

(a) through (c) No change.

(d) One or more construction inspections are A final system inspection is performed showing compliance with all rules under which the original permit construction approval was issued granted. If applicable, a system re-inspection fee is paid.

(7) No change.

Rulemaking Authority 381.0065(3)(a), 381.0065(3)(c), 381.0065(8)(h), 403.061, 489.553(3), 489.557(1) FS. Law Implemented 381.0065, 381.0067, 386.041 FS. History–New 12-22-82, Amended 2-5-85, Formerly 10D-6.43, Amended 3-17-92, 1-3-95, 5-14-96, 2-13-97, Formerly 10D-6.043, Amended 3-22-00, 4-21-02, 5-24-04, 11-26-06, 6-25-09, 4-1-10, 4-28-10, 7-16-13, Formerly 64E-6.003, Amended .

62-6.024 Private Provider Inspectors.

(1) Accelerated certification training for private provider inspectors. In order to pass the accelerated certification training a person must successfully complete the pre-certification coursework and achieve a passing score on each of the examinations.

(a) Prior to participating in the pre-certification coursework and taking the associated examinations, a person requesting authorization to take the examinations as part of their qualifications must submit the following to the Department:

1. Account creation and examination fees, for other than Department employees.

2. Contact information:

a. Name (first name, middle, last name, generational suffix if applicable).

b. Firm or business name, if applicable.

c. Mailing address (street address, city, state, zip).

d. Email address.

e. Date of birth.

f. Telephone number.

3. Confirmation of at least one of the following prequalifications, as applicable:

a. Applicants qualifying under subparagraph 381.0065(8)(c)1., F.S., and section 381.0101, F.S., as an environmental health professional certified in the onsite sewage program must submit proof of their determination of eligibility to take part in the accelerated certification training in accordance with subsection 64E-18.003(6), F.A.C.

b. Applicants qualifying under subparagraph 381.0065(8)(c)3., F.S., must submit proof of their active license issued under Chapter 471, F.S., as an engineer in Florida.

c. Applicants qualifying under subparagraph 381.0065(8)(c)4., F.S., or paragraph 381.0101(2)(b), F.S., must submit a signed, sealed, and dated statement from the engineer under whose supervision they work confirming supervisory oversight.

(b) After submitting a complete application, applicants must register for participation in and complete the pre-certification coursework.

(c) After completing the pre-certification coursework eligible applicants must successfully complete each part of the accelerated certification training examination administered by the Department. The minimum passing score for each part of the examination must be a 70 percent correct response to all questions.

1. Examination results for applicants achieving a passing score on each part of their examination will be reported to the appropriate licensing or registration entity and to the private provider inspector registry.

2. Applicants not achieving a passing score on any part of the examination for any part or failing to take a scheduled examination are required to submit an additional examination fee before being authorized to take that part of the examination again.

3. If the applicant is unable to pass any part of the examination within two attempts, the applicant must reregister for the respective pre-certification coursework and examination pursuant to paragraphs 62-6.024(1)(b) and (c), F.A.C.

(d) Examination results are valid for a period of 2 years from the date of examination.

(2) Prohibited inspections. Private provider inspectors are prohibited from conducting an inspection in any of the following conditions:

(a) An onsite sewage treatment and disposal system must not be inspected by the private provider inspector or authorized representative of the private provider inspector who was the installer for that system.

(b) Engineers licensed in Florida and qualified pursuant to subparagraph 381.0065(8)(c)3., F.S., or persons working under the supervison of the engineer and qualified pursuant to subparagraph 381.0065(8)(c)4., F.S., are prohibited from acting as the private provider inspector on a system when the engineer also serves as the engineer certifying completion for the installation of the onsite sewage treatment and disposal system according to subsection 62-6.003(2), F.A.C., or paragraph 62-6.027(5)(a), F.A.C.

(3) Complaints and Enforcement. A private provider inspector must perform the inspection in compliance with the requirements of this chapter and section 381.0065, F.S.

(a) A private provider inspector who violates the requirements of this chapter, section 381.0065, F.S., Part III of Chapter 489, F.S., or other Department requirements is subject to enforcement pursuant to section 381.0065, F.S. and Part I of Chapter 403, F.S.

(b) A private provider inspector must comply with the requirements of the applicable authority with jurisdiction over their license or certification under Chapter 381, F.S., Chapter 471, F.S., or Chapter 489, F.S. For private provider inspectors who are qualified pursuant to subparagraph 381.0065(8)(c)4., F.S., and who may not have an established professional licensing board regulating their profession, enforcement action will include the licensed professional engineer licensed under Chapter 471, F.S., who supervised the private provider inspector. The Department may refer cases for further investigation to the applicable authority with jurisdication over the license or certification.

(4) A private provider inspector hired to perform a construction inspection is authorized to perform one or more construction inspections of an onsite sewage treatment and disposal system during the construction and installation of the system prior to final installation approval by the Department, and shall be subject to the following registration and auditing requirements:

(a) Prior to submitting a construction inspection report for a construction inspection conducted on or after {effective date + 90 days}, a private provider inspector must submit the following registration information to the Department:

1. Private provider inspector name (first name, middle, last name, generational suffix if applicable).

2. Private provider inspector firm or business name, if applicable.

3. Business address (street address, city, state, zip).

4. Business email address unique to private provider inspector.

5. Business telephone number.

6. Qualification statement or resume pursuant to sub-subparagraph 381.0065(8)(d)1.c., F.S.

7. Qualifying professional license or certification number pursuant to paragraph 381.0065(8)(c), F.S.

8. If the private provider inspector is working under the supervision of an engineer pursuant to subparagraph 381.0065(8)(c)4., F.S., a dated, signed, and sealed statement from the engineer which attests to their supervisory oversight.

(b) Upon receiving the registry information required by paragraph 62-6.024(4)(a), F.A.C., the Department will review and verify compliance with subsection 381.0065(8), F.S. If the registry information submitted does not meet the requirements of subsection 381.0065(8), F.S., the Department will notify the private provider inspector within 30 days of such determination. Once the Department confirms the registry information, the Department will assign the private provider inspector an active registry number for identification purposes. To remain active in the registry the private provider inspector must confirm or update their information in writing to the Department by October 31 of each year. However, any change to license, registration, or certification required by paragraph 381.0065(8)(c), F.S., to qualify as a private provider inspector, shall be reported to the Department within seven days of the effective date of the change.

(c) The Department may audit up to 25 percent of the private provider inspectors each year in accordance with the provisions of paragraph 381.0065(8)(e), F.S. The private provider inspector shall provide the required inspection records and documentation during the Department’s normal business hours within 21 days of an audit request. An audit performed pursuant to paragraph 381.0065(8)(e), F.S. will:

1. Consist of an assessment of the tools, methods, and reporting used by a private provider inspector. The private provider inspectors will be audited to confirm compliance with requirements of this chapter, Chapter 381, F.S., Form DEP 4015A, herein adopted and incorporated by reference in subsection 62-6.003(3), F.A.C., and Part 2 of Form DEP 4016, herein adopted and incorporated by reference in subsection 62-6.003(1), F.A.C.

2. Include a field compliance component, during which a Department inspector observes the private provider inspector performing a construction inspection. If the Department inspector identifies deficiencies in the private provider’s inspection, the private provider inspector must not issue a construction inspection approval until the applicable requirements under this chapter have been met.

3. Include a component to assess the completeness, timeliness, and accuracy of submitted inspection records and registry information.

4. Generate an audit report that will include an assessment of the performance of the private provider inspector, and requirements for training or other measures to address deficiencies found. The audit report will be given to the private provider inspector within 30 days of completion of the audit. Subject to sections 120.57 and 120.569, F.S., the private provider inspector, after receiving the results of the audit report, shall have 30 days to respond to the audit report and provide a corrective action plan to address any deficiencies that the Department may have identified.

(d) The Department will create and maintain a publicly accessible portal for the public to locate information on a registered private provider inspector.

(5) Private provider inspectors conducting inspections under subsection 381.0065(8), F.S., are responsible for ensuring that their inspections are conducted in accordance with the requirements of this chapter and the system construction permit. Private provider inspectors must determine whether the onsite sewage treatment and disposal system complies with all applicable standards. Failure to properly conduct inspections or determine compliance may result in enforcement action upon the private provider inspector by the Department.

Rulemaking Authority 381.0065(3)(a), 381.0065(3)(c), 381.0065(8)(h), 489.553, FS. Law Implemented 381.0065, Part I 386, 489.552, FS. History–New .

62-6.027 Permits and Inspections.

(1) No change.

(2) System Construction Permit. – No portion of a performance-based treatment system shall be installed, repaired, altered, modified, abandoned or replaced until a construction permit has been issued on Form DEP 4016, herein adopted and incorporated by reference in subsection 62-6.003(1), F.A.C. If building construction has commenced, the system construction permit shall be valid for an additional 90 days beyond the eighteen month expiration date. A fee shall be charged for a repair permit issued within 12 months from the date of final authorization of the performance-based treatment system. If a construction or repair permit for a performance-based treatment system is transferred to another person, the date of the construction or repair permit shall not be amended, but shall run from the date of original issuance prior to the transfer. Servicing or replacing with like kind mechanical or electrical parts of a performance-based treatment system; pumping of septage from a system; or making minor structural corrections to a tank, or distribution box, does not constitute a repair, however, all services must be performed by the performance system maintenance entity. Any proposed change from the original design, including increasing or decreasing changes in flow rate, shall require that the system be re-engineered to achieve the desired performance standard under the altered conditions.

(3) and (4) No change.

(5) Construction System inspection. – Construction inspections of a performance-based treatment system must comply with the requirements of subsections 62-6.003(2) and (3), F.A.C., with the following additional requirements: Before covering with earth and before placing the performance-based treatment system into service, a person installing or constructing any portion of the performance-based treatment system shall notify the Department of the completion of the construction activities and shall have the system inspected by the Department.

(a) Prior to or concurrent with the last construction a final installation inspection by the Department or a private provider inspector, the professional engineer who designed the system or the design engineer’s designee, shall inspect the entire installation and the engineer shall certify in writing that the installed system complies with the approved design and installation requirements. This certification shall read as follows: “I certify that the engineering features of this performance-based treatment system as installed have been examined by me and found to comply with all specifications contained in the engineering design that was the basis for issuance of the construction permit. I certify that the required components are installed for the system to function as permitted and designed. I certify that the operation and maintenance manual for this performance-based treatment system has been prepared or examined by me or by an individual(s) under my direct supervision and that there is reasonable assurance, in my professional judgment, that the system, when properly operated and maintained in accordance with this manual, will achieve the established performance standard and comply with all applicable statutory requirements and rules of the Department.”

(b) No change.

(c) In addition to the requirements of paragraph 62-6.003(2)(b), F.A.C., the Department or private provider inspector performing a construction inspection must also notify the design engineer of inspection results. If the system is found to not comply with the construction permit during the construction inspection on any type of system installation, the Department shall notify the engineer. The installer shall make all required corrections and notify the Department of the completion of the work prior to reinspection of the system. A reinspection fee shall be charged for each additional inspection leading up to construction approval.

(d) Final installation approval will not be issued until the Department has confirmed:

1. No change.

2. Other system construction requirements, as reviewed during a system construction inspection, are in compliance with the permit, and the plans and specifications provided with the permit application, including building construction and lot grading,

3. through 4. No change.

(e) No change.

(6) and (7) No change.

Rulemaking Authority 381.0065(3)(a), 381.0065(3)(c), 381.0065(8)(h), FS. Law Implemented 381.0065, Part I 386 FS. History–New 2-3-98, Amended 4-21-02, 6-18-03, 6-25-09, 4-28-10, Formerly 64E-6.027, Amended 8-27-24, Amended .

62-6.030 Fees.

(1) The following fees are required for services provided by the Department.

| (a) and (b) No change. |  |
| --- | --- |
| (c) Application and Existing System and System Repair Evaluation. | $50.00 |
| (d) through (f) No change. |  |
| (g) Permit or permit application amendment for new system, modification, or repair to system. | $55.00 |
| (h) through (j) No change. |  |
| (k) Application for system abandonment permit, includes permit issuance and inspection, or review of the first inspection report by a private provider inspector. | $50.00 |
| (l) through (x) No change. |  |
| (y) Review of each private provider inspector’s construction approval inspection report. | $35.00 |

(2) The following fees are required to accompany applications for registration of individuals for septic tank contractor or master septic tank contractor, or for a certificate of authorization for partnerships and corporations, or for private provider inspectors and site evaluators, as specified.

| (a) Account creation Application for registration including examination. | $25.00 75.00 |
| --- | --- |
| (b) Contractor examination or reexamination Initial registration. | $75.00 100.00 |
| (c) Application review for contractor registration and renewal Renewal of registration. | $100.00 |
| (d) No change. |  |
| (e) Accelerated certification training course examination or reexamination, Part I. | $50.00 |
| (f) Accelerated certification training course examination or reexamination, Part II. | $50.00 |
| (g) Accelerated certification training course examination or reexamination, Part III. | $50.00 |

Rulemaking Authority 154.06(1), 381.0065(3)(c), 381.0065(8)(h), 381.0066, 489.557(1) FS. Law Implemented 381.0064, 381.0065, 381.0066, 489.557 FS. History–New 2-3-98, Amended 3-22-00, 4-21-02, 5-24-04, 11-26-06, 9-24-07, Formerly 64E-6.030, Amended.

NAME OF PERSON ORIGINATING PROPOSED RULE: Eb Roeder

NAME OF AGENCY HEAD WHO APPROVED THE PROPOSED RULE: Alexis Lambert, Secretary

DATE PROPOSED RULE APPROVED BY AGENCY HEAD: April 28, 2025

DATE NOTICE OF PROPOSED RULE DEVELOPMENT PUBLISHED IN FAR: July 1, 2022, November 18, 2022