Notice of Declaratory Statement

DEPARTMENT OF BUSINESS AND PROFESSIONAL REGULATION

Florida Building Commission

RULE NO.:	RULE TITLE:

61G20-1.001	Florida Building Code Adopted

NOTICE IS HEREBY GIVEN that the Florida Building Commission has received the petition for declaratory statement from Larry Burchett. The petition seeks the agency's opinion as to the applicability of 61G20-1.001, F.A.C., as it applies to the petitioner.

Petitioner states that he is the technical advisor for a homeowners association, and asserts that some number of homes in the community constructed between 2019 and 2023 experienced "premature failures of swimming pool auto-fill valves," and that "toilet fill valves were failing and leaking." Petitioner further asserts that "[t]he cause of the fluctuations and of high pressures on the closed loop plumbing systems within the one hundred homes was in fact no thermal expansion control," and that "[t]he industry standard of maximum 80 psi regulation was overlooked by Collier County Inspections, the Builder, and the contracted Plumber." Petitioner then presents the following questions: 1. Since the builder and plumber allowed the substitution of a thermal expansion tank required by the manufacturer to be substituted, using a relief valve, is this Code applicable in this situation? 2. Since the builder and plumber allowed the use of a pressure relief valve rated at 125 psi to be used as a substitution of a thermal expansion tank, is this Code section applicable in this situation? 3. Does the wording "a means for controlling increased pressure" [in Code Section P2903.4] allow for any means? As an example, could you use a hot water bottle plumbed into the system? Could you use a pressure cooker blow off valve to control pressure? Could you use a non-rated ASME Device to control increased pressure? 4. Can a Shutoff Valve be installed between any relief valve and the hot water heating device? [O]r worded differently, Can a relief valve be installed in front (upstream) of a shutoff valve and the hot water heater to control pressure created by the thermal expansion?

A copy of the Petition for Declaratory Statement may be obtained by contacting: the Agency Clerk’s Office, Department of Business and Professional Regulation, 2601 Blair Stone Road, Tallahassee, Florida 32399, (850)921-0342, AGC.Filing@myfloridalicense.com.

Please refer all comments to: Mo Madani, Building Codes and Standards Office, Department of Business and Professional Regulation, 2601 Blair Stone Road, Tallahassee, Florida 32399, (850)487-1824, mo.madani@myfloridalicense.com or W. Justin Vogel, Office of the General Counsel, Department of Business and Professional Regulation, 2601 Blair Stone Road, Tallahassee, Florida 32399, (850)717-1795, wjustin.vogel@myfloridalicense.com.

RESPONSES, MOTIONS TO INTERVENE, OR REQUESTS FOR A HEARING MUST BE FILED WITHIN 21 DAYS OF THIS NOTICE.