68D-23.104 Placement of Markers.

(1)(a) Except as provided below, no person, municipality, county or other governmental entity shall place, cause to be placed, or maintain in place any marker in, on or over the waters of the state or the shores thereof without a permit from the division.

(b) Counties, municipalities and other government entities are exempt from permitting under this rule when placing:

1. Swimming Area Markers or Special Event Markers on inland lakes and their associated canals.

2. Informational markers other than Swimming Area Markers or Special Event Markers, when placed on the shores of inland lakes and their associated canals or on such waters within fifty feet (50') of the ordinary high water line.

(c) Nothing herein shall prevent counties, municipalities or other governmental entities from choosing to voluntarily apply for waterway marker permits.

(2) Any person, municipality, county, or other governmental entity desiring to place a marker shall make application to the division on the Florida Uniform Waterway Marker Application form, FWC/DLE 153 (01/2006), which is adopted and incorporated herein by reference. Application forms may be obtained by submitting a request to: Florida Fish and Wildlife Conservation Commission, Division of Law Enforcement, Boating and Waterways Section, 620 South Meridian Street, Tallahassee, Florida 32399-1600 or download the application from the Commission website at: http://www.myfwc.com/boating/waterways/index.htm. Each application must include:

(a) A scale drawing no larger than 8½ inches by 11 inches, reproducible on standard office photocopying equipment, showing the approximate placement of the proposed markers with each proposed marker labeled to correspond to the list required below. If the application is for regulatory markers, the drawing must also depict the exact boundaries of the proposed boating restricted area.

(b) A list of the markers proposed, labeled to correspond to the drawing required above.

(c) A statement of the specifications for the markers proposed, including:

1. A description giving the type, size, shape, color, material, height above mean high water for each marker sign or buoy, and the number, letter or message displayed thereon;

2. A description of the type, size, and material used for:

a. Any structure which will support a marker sign;

b. Any anchor, anchoring system, chain, tether, rode, or other ground tackle which will secure a marker buoy, including a mooring buoy, to the bottom;

3. A statement of the color, characteristic, height above mean high water, intensity, and nominal range of any light which will be placed on the markers;

4. A statement of the type signal (whistle, horn, bell, etc.) and characteristic for any audible fog signal;

5. The latitude and longitude expressed in degrees and decimal minutes of the location where each marker will be placed, and the datum in which the coordinates are expressed (WSG-84, NAD-83, etc.).

(d) A statement of the purpose for placing the proposed markers. If the application is for regulatory markers, this statement must include the purpose for regulating vessel operation and a statement of the facts and circumstances justifying the establishment of the restriction on speed or operation.

(e) A statement listing the names or titles of the individuals responsible for the placement and maintenance of the markers along with an address and a contact telephone number for each individual.

(f) If the application is for regulatory markers, the applicant must enclose therewith proof of the lawful imposition of restrictions on the speed or operation of vessels for which the regulatory markers are requested, as follows:

1. A copy of an ordinance adopted pursuant to Section 327.22, F.S., which imposes the restriction only upon vessels resident within the county or municipality imposing the restriction; or

2. A copy of an ordinance adopted pursuant to Section 327.60, F.S., which imposes the restriction for reasons of vessel traffic safety or public safety; or

3. A copy of an ordinance adopted by a county or municipality and approved by the commission pursuant to paragraph 370.12(2)(p), F.S., which imposes the restriction for reasons of manatee protection; or

4. A copy of the statute, special act, rule, regulation, order, or other instrument which imposes the restriction for reasons other than public safety or manatee protection, and a statement of the specific authority under which the restriction is imposed.

(3) Upon receipt of all statements and other documents specified above, the division will:

(a) Determine whether:

1. The proposed markers conform to the United States Aids to Navigation System and this chapter; and

2. The proposed markers and any support structures or moorings conform to the United States Coast Guard Aids to Navigation – Technical Manual (Comdtinst M16500.3A).

(b) Determine whether or not the placement of the proposed markers in the proposed locations would create an unreasonable hazard to navigation.

(c) For danger markers only, determine whether or not the markers as proposed would clearly mark the hazard or danger and adequately notice mariners of the hazard or danger.

(d) For regulatory markers only:

1. Determine whether or not the markers as proposed would clearly mark the area as a boating restricted area and adequately notice mariners of the restriction imposed on vessel speed or operation.

2. Based on the authority under which the restriction is imposed:

a. Determine whether or not the restriction applies only to vessels resident in the ordaining municipality or county, for ordinance adopted pursuant to Section 327.22, F.S.; or

b. Determine whether or not there exists a conflict with the provisions of Chapter 327, F.S., or any amendments thereto or regulations thereunder, for ordinances adopted pursuant to Section 327.60, F.S.; or

c. Forward the request to the Imperiled Species Management Section for review and approval pursuant to paragraph 370.12(2)(o), F.S., for ordinances adopted thereunder; or

d. For all other regulatory markers, determine whether or not the restriction to be implemented by the proposed markers is supported by statute, special act, rule, ordinance, or other enactment or order.

(4) The division may consult, coordinate, or cooperate with any other governmental entity having concurrent jurisdiction over the waters for which the permit is requested.

(5) After obtaining the requested permit, the applicant must install, inspect, maintain, and remove the permitted marker at its own expense and as directed by the division.

(6) Discontinuance and removal. Any permitted waterway marker may be discontinued and removed by the owner after 30 days notice to the Boating and Waterways Section. Upon completion of the removal of the marker, the applicant must notify the Boating and Waterways Section in writing within 30 working days.

Specific Authority 327.04, 327.40, 327.41 FS. Law Implemented 327.40, 327.41 FS. History–New 12-23-01, Amended 10-5-06.