69A-58.0031 New Construction.

(1) New construction and new buildings are subject to and controlled by NFPA 1, the  edition as adopted in Rule 69A-3.012, F.A.C., in Chapter 20, relating to “Educational occupancies” and NFPA 101, the edition as adopted in Rule 69A-3.012, F.A.C., Chapter 14, “New educational occupancies,” except where specifically otherwise provided in this rule chapter.

(2) Notwithstanding any rule or adopted code or standard in conflict herewith, the following procedures apply with respect to new construction and new buildings.

(a) Prior to commencement of any new construction or remodeling:

1. The board shall approve or cause to be approved the plans, drawings, designs, proposals, blueprints, and other construction or remodeling documents and evaluate the same for complete compliance with the Florida Fire Prevention Code in accordance with Section 1013.38(1), F.S., or

2. The board must show compliance with all applicable firesafety codes and standards by at least one of the other means provided in Section 1013.38(2)(a) through (d), F.S.

(b) The method of compliance must be documented and maintained as part of the construction records file.

(c) Upon request by the local fire official, the board shall provide reasonable access to all construction documents and provide in writing to the local fire official the method(s) employed to achieve compliance with the Florida Fire Prevention Code.

(3) A certificate of occupancy shall not be issued until the board has determined that the building or structure and its site conditions comply with all applicable statutes, these rules, and all applicable firesafety codes and standards.

(4) Horizontal exits referenced in NFPA 101, subdivision 14.2.2.5 and exit passageways referenced in NFPA 101, subdivision 14.2.2.7 are prohibited.

Specific Authority 1013.12 FS. Law Implemented 1013.12 FS. History–New 11-26-06.