62-303.430 Biological Impairment.

(1) All bioassessments used to list a water on the verified list shall be conducted in accordance with Chapter 62-160, F.A.C., including Department-approved Standard Operating Procedures. To be used for placing waters on the verified list, any bioassessments conducted before the adoption of applicable SOPs for such bioassessments as part of Chapter 62-160, F.A.C., shall substantially comply with the subsequent SOPs.

(2) If the water was listed on the planning list based on bioassessment results, the water shall be determined to be biologically impaired if there were two or more failed bioassessments within the five years preceding the planning list assessment. If there were less than two failed bioassessments during the last five years preceding the planning list assessment, the Department will conduct an additional bioassessment. If the previous failed bioassessment was a BioRecon, then an SCI will be conducted. Failure of this additional bioassessment shall constitute verification that the water is biologically impaired.

(3) If the water was listed on the planning list based on other information specified in subsection 62-303.330(4), F.A.C., indicating biological impairment, the Department will conduct a bioassessment in the water segment, conducted in accordance with the methodology in Rule 62-303.330, F.A.C., to verify whether the water is impaired. For streams, the bioassessment shall be an SCI. Failure of this bioassessment shall constitute verification that the water is biologically impaired.

(4) Following verification that a waterbody is biologically impaired, a water shall be included on the verified list for biological impairment if:

(a) There are water quality data reasonably demonstrating the particular pollutant(s) causing the impairment and the concentration of the pollutant(s); and

(b) One of the following demonstrations is made:

1. If there is a numeric criterion for the specified pollutant(s) in Chapter 62-302, F.A.C., but the criterion is met, an identification of the specific factors that reasonably demonstrate why the numeric criterion is not adequate to protect water quality and how the specific pollutant is causing the impairment, or

2. If there is not a numeric criterion for the specified pollutant(s) in Chapter 62-302, F.A.C., an identification of the specific factors that reasonably demonstrate how the particular pollutant(s) are associated with the observed biological effect.

Specific Authority 403.061, 403.067 FS. Law Implemented 403.062, 403.067 FS. History–New 6-10-02, Amended 12-11-06.