62-303.480 Drinking Water Use Support and Protection of Human Health.

If the water segment was listed on the planning list due to exceedances of a human health-based water quality criterion and there were insufficient data from the last five years preceding the planning list assessment to meet the data sufficiency requirements of subsection 62-303.320(4), F.A.C., additional data will be collected as needed to meet the requirements. Once these additional data are collected, the Department shall re-evaluate the data using the methodology in subsection 62-303.380(2), F.A.C., and limit the analysis to data collected during the five years preceding the planning list assessment, the additional data collected pursuant to this paragraph (not to include data older than 7.5 years), and data older than 7.5 years if it is demonstrated to be representative of current conditions. Any determinations to use older data shall be documented by the Department, and the documentation shall provide the basis for the decision that the data are representative of current conditions. For this analysis, the Department shall exclude any data meeting the requirements of subsection 62-303.420(5), F.A.C. The following water segments shall be listed on the verified list:

(1) For human health-based criteria expressed as maximums, water segments that meet the requirements in subsection 62-303.420(7), F.A.C., or

(2) For human health-based criteria expressed as annual averages, water segments that have an annual average that exceeds the applicable criterion.

(3) For bacteriological water quality criteria, water segments shall be included on the verified list if:

(a) The number of samples that do not meet the applicable single-sample bacteriological water quality criteria meet the requirements in subsection 62-303.420(6), F.A.C.

(b) There are two or more exceedances of a bacteriological water quality criterion expressed as a monthly geometric mean during a calendar year or more than four exceedances of the monthly criterion over the verified period, or

(c) There are two or more months during a calendar year or more than four months during the verified period in which greater than 10 percent of the samples at a given station exceed 400 counts per 100 ml.

(d) As to paragraphs (3)(b) and (c) above, to assess the monthly data for a sampling location there shall be at least five samples collected within that month, with at least one sample from each full week of the month.

Specific Authority 403.061, 403.067 FS. Law Implemented 403.062, 403.067 FS. History–New 6-10-02, Amended 12-11-06.