58A-5.023 Physical Plant Standards.

(1) GENERAL REQUIREMENTS.

(a) The ALF shall be located, designed, equipped, and maintained to promote a residential, non-medical environment, and provide for the safe care and supervision of all residents.

(b) The facility’s physical structure, including the interior and exterior walls, floors, roof and ceilings shall be structurally sound and in good repair. Peeling paint or wallpaper, missing ceiling or floor tiles, or torn carpeting shall be repaired or replaced. Windows, doors, plumbing, and appliances shall be functional and in good working order. All furniture and furnishings shall be clean, functional, free-of-odors, and in good repair. Appliances may be disabled for safety reasons provided they are functionally available when needed.

(c) In order to ensure a safe and sanitary environment, the ALF shall be subject to annual inspection by the county health department pursuant to Chapter 64E-12, F.A.C.

(d) Indoor radon testing as mandated by Section 404.056(5), F.S., shall be completed by all facilities.

(2) HEATING AND COOLING.

(a) When outside temperatures are 65 degrees Fahrenheit or below, an indoor temperature of at least 72 degrees Fahrenheit shall be maintained in all areas used by residents during hours when residents are normally awake. During night hours when residents are asleep, an indoor temperature of at least 68 degrees Fahrenheit shall be maintained.

(b) During hours when residents are normally awake, mechanical cooling devices, such as electric fans, must be used in those areas of buildings used by residents when inside temperatures exceed 85 degrees Fahrenheit provided outside temperatures remain below 90 degrees Fahrenheit. No residents shall be in any inside area that exceeds 90 degrees Fahrenheit. However, during daytime hours when outside temperatures exceed 90 degrees, and at night, an indoor temperature of no more than 81 degrees Fahrenheit must be maintained in all areas used by residents.

(c) Residents who have individually controlled thermostats in their bedrooms or apartments shall be permitted to control temperatures in those areas.

(3) COMMON AREAS.

(a) A minimum of 35 square feet of living and dining space per resident, live-in staff, and live-in family member shall be provided except in facilities comprised of apartments. This space shall include living, dining, recreational, or other space designated accessible to all residents, and shall not include bathrooms, corridors, storage space, or screened porches which cannot be adapted for year round use. Facilities with apartments may count the apartment’s living space square footage as part of the 35 square footage living and dining space requirement.

1. Those facilities which were licensed as of May 14, 1981, which demonstrate compliance with all other applicable rules shall be granted a 10 percent waiver in the square footage requirement upon request.

2. Those facilities also serving as adult day care centers must provide an additional 35 square feet of living and dining space per adult day care client. Excess floor space in residents’ bedrooms or apartments cannot be counted toward meeting the requirement of 35 square feet of living and dining space requirements for adult day care participants. Day care participants may not use residents’ bedrooms for resting unless the room is currently vacant.

(b) A room, separate from resident bedrooms, shall be provided where residents may read, engage in socialization or other leisure time activities. Comfortable chairs or sofas shall be provided in this communal area.

(c) The dining area shall be furnished to accommodate communal dining.

(4) BEDROOMS. Residents shall be given the option of choosing their own roommate or roommates if possible.

(a) Resident bedrooms designated for single occupancy shall provide a minimum inside measurement of 80 square feet of usable floor space. Usable floor space does not include closet space or bathrooms.

(b) Resident bedrooms designated for multiple occupancy shall provide a minimum inside measurement of 60 square feet of usable floor space per room occupant.

(c) Resident bedrooms designated for multiple occupancy in facilities newly licensed or renovated 6 months after 10-17-99, shall have a maximum occupancy of two persons. Resident bedrooms designated for multiple occupancy in facilities licensed prior to 10-17-99, shall have a maximum occupancy of four persons.

(d) All resident bedrooms shall open directly into a corridor, common use area or to the outside. A resident must be able to exit his bedroom without having to pass through another bedroom unless the 2 rooms have been licensed as one bedroom.

(e) Pursuant to Section 429.27, F.S., residents shall be given the option of using his/her own belongings as space permits. Each resident bedroom or sleeping area, where furnishings are supplied by the facility shall, at a minimum, be furnished with the following:

1. A clean, comfortable bed with a mattress no less than 36 inches in width and 72 inches in length with the top surface of the mattress a comfortable height to assure easy access by the resident;

2. A closet or wardrobe space for the hanging of clothes;

3. A dresser, chest, or other furniture designed for the storage of personal effects; and

4. A table, bedside lamp or floor lamp, waste basket, and comfortable chair shall be provided if requested.

(f) All resident bedrooms shall be for the exclusive use of residents. Live-in staff and their family members shall be provided with sleeping space separate from the sleeping and congregate space required for residents.

(g) The facility shall maintain master or duplicate keys to resident bedrooms to be used in the event of an emergency.

(5) BATHROOMS.

(a) There shall be at least one bathroom with one toilet and sink per six persons, and one bathtub or shower per eight persons. All residents, all live-in staff and family members, and respite care participants must be included when calculating the required number of toilets, sinks, bathtubs and showers. All adult day care participants shall be included when calculating the required number of toilets and sinks.

(b) Each bathroom shall have a door in working order to ensure privacy. The entry door to bathrooms with a single toilet shall have a lock that the resident can operate from the inside with no key needed. A non-locking door shall be permitted if the resident’s safety would otherwise be jeopardized. The facility shall maintain master or duplicate keys to resident bathrooms to be used in the event of an emergency.

(c) There shall be non-slip safety devices such as bath mats or peel off stickers in the showers and bathtubs of all facilities. Showers and bathtubs with a non-skid surface require a separate non-skid device only if the surface is worn. Grab bars shall be required in showers and bathtubs. Grab bars, whether portable or permanent, must be securely affixed to the floor or adjoining walls. Facilities newly licensed or renovated 6 months after (10-17-99) must have grab bars next to the commode.

(d) Sole access to a toilet or bathtub or shower shall not be through another resident’s bedroom, except in apartments within a facility.

(e) Residents who use portable bedside commodes shall be provided with privacy in their use.

(6) LINENS AND LAUNDRY. Facilities shall make available linens and personal laundry services for residents who require such services. Linens provided by a facility shall be free of tears, stains, and not threadbare.

(7) SECURITY. External boundaries of a facility or a distinct part of a facility, including outside areas, may be secured using egress control or perimeter control devices if the following conditions are met.

(a) The use of the device complies with all life-safety requirements.

(b) Residents residing within a secured area are able to move freely throughout the area, including the resident’s bedroom or apartment, bathrooms and all common areas, and have access to outdoor areas on a regular basis and as requested by each resident.

(c) Residents capable of entering and exiting without supervision have keys, codes, or other mechanisms to exit the secured area without requiring staff assistance.

(d) Staff who provide direct care or who have regular contact with residents residing in secured areas complete Level 1 Alzheimer’s training as described in Rule 58A-5.0191, F.A.C.

(8) Pursuant to Section 429.41, F.S., facilities with 16 or fewer residents shall not be required to maintain an accessible telephone in each building where residents reside, maintain written staff job descriptions, have awake night staff, or maintain standardized recipes as provided in paragraphs 58A-5.0182(6)(g), 58A-5.019(2)(e), 58A-5- 019(4)(a), and 58A-5.020(2)(b), F.A.C., respectively.

Specific Authority 429.41 FS. Law Implemented 404.056,  429.27, 429.41 FS. History–New 5-14-81, Amended 1-6-82, 5-19-83, 9-17-84, Formerly 10A-5.23, Amended 10-20-86, 6-21-88, 8-15-90, 9-30-92, Formerly 10A-5.023, Amended 10-30-95, 6-2-96, 10- 17-99, 7-30-06.