62-18.050 Location of Microphone.

For vehicles measured on the highway, the microphone shall be located at a height of not less than 2 feet nor more than 6 feet above the plane of the roadway surface and not less than 3 1/2 feet above the surface upon which the microphone stands. The standard distance from the microphone to the center of the lane of travel of vehicles measured on the highway shall be 50 feet. The actual distance may range from 26 to 118 feet, based on correction factors which are to be applied to meter readings to correct them to the standard distance, as shown in Table 1.

|  | Distance from | Sound level |
| --- | --- | --- |
|  | microphone to | correction factor, dB |
|  | center of lane of travel |  |
|  | 26-29 ft. | – 7 |
|  | 30-32 ft. | – 6 |
|  | 33-35 ft. | – 5 |
|  | 36-39 ft. | – 3 |
|  | 40-43 ft. | – 2 |
|  | 44-48 ft. | – 1 |
|  | 49-58 ft. | 0 |
|  | 59-70 ft. | + 1 |
|  | 71-83 ft. | + 2 |
|  | 84-99 ft. | + 3 |
|  | 100-118 ft. | + 4 |
| Table 1 |  |  |

Rulemaking Authority 403.061 FS. Law Implemented 403.061, 403.415(7), 316.293(3) FS., Chapter 74-110, Laws of Florida. History–New 11-27-74, Formerly 17-18.05, 17-18.050.